| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0180 |
| --- | --- |

| Title: | AhG 9: On ALF, LMCS and Scaling List Parameters Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Fabrice Le Léannec Tangi Poirier Philippe De Lagrange | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In the VVC draft 8 specification, ALF, LMCS and Scaling List parameters are signaled using the adaptation parameters set. This is to have the maximum flexibility where the encoder can signal any of these tools’ parameters at any moment. However, for certain profiles, APS is not signaled. For these profiles, ALF, LMCS and Scaling List shall not be employed. This contribution proposes additional coding mechanism for these parameters such that the tools can still be used with restricted decoders.

# Introduction

In VVC draft 8, there exist many constraint flags that are used to define profiles with limited coding tools. Among these flags, there is a flag to disable coding any NAL unit with APS. Specifically, in VVC draft 8, the general constraint information flags are coded in this way:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| no_aps_constraint_flag | u(1) |
| … |  |

The highlighted flag (no_aps_constraint_flag) is meant for disabling the coding of any APS information:

no_aps_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to PREFIX_APS_NUT or SUFFIX_APS_NUT present in OlsInScope. no_aps_constraint_flag equal to 0 does not impose such a constraint.

This flag was adopted in the 16th JVET meeting in Geneva meeting in 2019 (JVET-P0366). Its main motivation was to limit the types of NAL units for the decoder such that the decoder does not need specific implementation for all NAL unit types. However, at that point, it was not discussed on how low-level tools (ALF, LMCS and Scaling Lists) can be used in such a constraint.

In this contribution, we propose allowing alternative signaling of these parameters. Namely, we propose signaling these parameters at slice level, as well as APS. By doing so, decoders that do not implement APS NAL unit decoding can still employ ALF, LMCS and/or scaling lists.

# Proposal

Instead of only signaling the parameters at APS level, the parameters are signaled at SH level.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag ) |  |
| sh_alf_parameters_in_sh_flag | u(1) |
| if(sh_alf_parameters_in_sh_flag) |  |
| alf_data() |  |
| if( sps_alf_enabled_flag && !alf_info_in_ph_flag && ! sh_alf_parameters_in_sh_flag) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| if( sps_ccalf_enabled_flag ) { |  |
| slice_cc_alf_cb_enabled_flag | u(1) |
| if( slice_cc_alf_cb_enabled_flag ) |  |
| slice_cc_alf_cb_aps_id | u(3) |
| slice_cc_alf_cr_enabled_flag | u(1) |
| if( slice_cc_alf_cr_enabled_flag ) |  |
| slice_cc_alf_cr_aps_id | u(3) |
| } |  |
| } |  |
| } |  |
| … |  |
| if( sps_lmcs_enabled_flag ) |  |
| sh_lmcs_parameters_in_sh_flag | u(1) |
| if(sh_lmcs_parameters_in_sh_flag) |  |
| lmcs_data() |  |
| if( sps_scaling_list_enabled_flag ) |  |
| sh_scaling_list_parameters_in_sh_flag | u(1) |
| if(sh_scaling_list_parameters_in_sh_flag) |  |
| scaling_list_data() |  |
| } |  |

The semantics of these flags are:

sh_alf_parameters_in_sh_flag equal to 0 specifies that the adaptive loop filter parameters are not signaled at the SH level. sh_alf_parameters_in_sh_flag equal to 1 specifies that the adaptive loop filter parameters are signaled at the SH level. When not present the value of sh_alf_parameters_in_sh_flag is inferred to be zero.

sh_lmcs_parameters_in_sh_flag equal to 0 specifies that the lmcs parameters are not signaled at the SH level. sh_lmcs_parameters_in_sh_flag equal to 1 specifies that the lmcs parameters are signaled at the SH level. When not present the value of sh_lmcs_parameters_in_sh_flag is inferred to be zero.

sh_scaling_list_parameters_in_sh_flag equal to 0 specifies that the scaling list parameters are not signaled at the SH level. sh_scaling_list_parameters_in_sh_flag equal to 1 specifies that the scaling list are signaled at the SH level. When not present the value of sh_scaling_list_parameters_in_sh_flag is inferred to be zero.

Additionally, the following conformance constraints must be added for no_aps_constraint_flag:

It is a requirement of bitstream conformance that all sh_alf_parameters_in_sh_flag, sh_lmcs_parameters_in_sh_flag and sh_scaling_list_parameters_in_sh_flag shall be 1 if no_aps_constraint_flag is equal to 1

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).