| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0182-r2 |
| --- | --- |

| Title: | AhG 9: Removed Redundant Slice Level TSRC Flag Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Fabrice Le Léannec Tangi Poirier Michel Kerdranvat | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In the 17th JVET meeting, a slice level flag for disabling transform skip residual coding (TSRC) in JVET-Q0082. Whether transform skip itself is activated or not, this flag is independently coded. This contribution proposes coding this flag conditionally to transform skip, which leads to removed redundant coding at slice level.

# Introduction

The residuals of transform skip stage can either be coded in regular residual coding (RRC) or transform skip residual coding (TSRC) using the slice level flag slice_ts_residual_coding_disabled_flag. This is mainly used for lossless coding where RRC provides better coding performance for camera captured contents.

At SPS level, transform skip can be disabled by the flag sps_transform_skip_enabled_flag. That is, all residuals pass through the transform stage and only RRC is used if sps_transform_skip_enabled_flag is set to zero. In this case, it becomes redundant and does not need to be coded. Therefore, this contribution proposes coding slice_ts_residual_coding_disabled_flag only if transform skip is enabled.

# Proposal 1

slice_ts_residual_coding_disabled_flag is conditionally coded if sps_transform_skip_enabled_flag equals to one. That is, switching between RRC and TSRC is allowed if transform skip is allowed. The corresponding specification changes are shown below (added part is in blue):

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if (sps_transform_skip_enabled_flag) { |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

# Proposal 2

In order to further reduce the signaling overhead, we propose to add an SPS level to condition the presence of slice_ts_residual_coding_disabled_flag. The new flag, named sps_tsrc_disabled_flag, is used to disable TSRC. When this SPS flag is zero, the slice level flag is signaled to indicate whether RRC or TSRC is used. Otherwise, slice_ts_residual_coding_disabled_flag is not signaled but inferred to be one. The corresponding specification changes are:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_tsrc_disabled_flag | u(1) |
| … |  |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if (!sps_tsrc_disabled_flag) { |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

sps_tsrc_disabled_flag equal to 1 specifies that the residual_coding_ts( ) syntax structure shall not be used to parse the residual samples of a transform skip block. sps_tsrc_disabled_flag equal to 0 specifies that either the residual_ts_coding( ) or residual_ts_coding_ts( ) syntax structures is used to parse the residual samples of a transform skip block.

slice_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 01.

# Proposal 3: Combination with JVET-R0271-Variant 2

During the meeting, it is identified that DQ and SDH shall not be activated when RRC is enabled. JVET-R0271 has several variants that deactivate DQ and SDH when RRC is enabled (or vice versa). In this proposal, we combine proposal 2 with variant 2 of R0271.

The corresponding specification changes are:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_tsrc_disabled_flag | u(1) |
| … |  |
| } |  |
| } |  |

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !ph_dep_quant_enabled_flag ) |  |
| pic_sign_data_hiding_enabled_flag | u(1) |
| … |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if (!sps_tsrc_disabled_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( sps_dep_quant_enabled_flag && !slice_ts_residual_coding_disabled_flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !slice_dep_quant_enabled_flag && !slice_ts_residual_coding_disabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| … |  |
| } |  |

sps_tsrc_disabled_flag equal to 1 specifies that the residual_coding_ts( ) syntax structure shall not be used to parse the residual samples of a transform skip block. sps_tsrc_disabled_flag equal to 0 specifies that either the residual_ts_coding( ) or residual_ts_coding_ts( ) syntax structures is used to parse the residual samples of a transform skip block.

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

pic_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. pic_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When pic_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

slice_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 01.

slice_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. slice_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When slice_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

slice_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. slice_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When slice_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

# Proposal 4: Combination with JVET-R0271-Variant 3

In this proposal, we combine proposal 2 with variant 2 of JVET-R0271.

The corresponding specification changes are:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_tsrc_disabled_flag | u(1) |
| … |  |
| } |  |
| … |  |
| if (! sps_tsrc_disabled_flag) |  |
| sps_dep_quant_enabled_flag | u(1) |
| if( !sps_dep_quant_enabled_flag ) |  |
| sps_sign_data_hiding_enabled_flag | u(1) |
| … |  |
| } |  |

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !ph_dep_quant_enabled_flag ) |  |
| pic_sign_data_hiding_enabled_flag | u(1) |
| … |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !slice_dep_quant_enabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| if( !sps_tsrc_disabled_flag && !slice_dep_quant_enabled_flag && ! slice_sign_data_hiding_enabled_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( NumEntryPoints > 0 ) { |  |
| … |  |
| } |  |

sps_tsrc_disabled_flag equal to 1 specifies that the residual_coding_ts( ) syntax structure shall not be used to parse the residual samples of a transform skip block. sps_tsrc_disabled_flag equal to 0 specifies that either the residual_ts_coding( ) or residual_ts_coding_ts( ) syntax structures is used to parse the residual samples of a transform skip block.

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

pic_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. pic_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When pic_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

slice_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0! (slice_dep_quant_enabled_flag || slice_sign_data_hiding_enabled_flag).

slice_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. slice_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When slice_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

slice_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. slice_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When slice_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

# Discussion

In JVET-R0483, an analysis is made to distinguish the different approaches by computing the number of bits spent in SH. We use the same analysis here to distinguish Proposal 3 and Proposal 4 from other proposed methods in JVET-R0483. Specifically, in JVET-R0483, the SPS flag “sps_ts_residual_coding_disabled_slice_present_flag” for controlling the slice level flag “slice_ts_residual_coding_disabled_flag” is the inverse of the considered SPS flag in this document. This change the number of SH bits for different use cases. Following JVET-R0483, the truth tables for Proposal 3 and Proposal 4 are:

Proposal 3

| Use case | sps_TS_enabled | sps_tsrc_disable | sh_tsrc_disable_flag | ( sh_SDH ¦¦ sh_DQ ) | Number of SH bits |
| --- | --- | --- | --- | --- | --- |
| TS disabled | 0 | x (0 inferred) | x (0 inferred) | 0/1 | 1 |
| Lossy in CLVS* | 1 | 0 | 0 | 0/1 | 2 |
| lossless in CLVS** | 1 | 1 | x (1 inferred) | x (0 inferred) | 0 |
| Mixed LL lossy Slices | 1 | 0 | 0 | 0/1 | 2 |
| Mixed LL lossless Slices | 1 | 0 | 1 | x (0 inferred) | 1 |
| RRC only | 1 | 1 | x (1 inferred) | x (0 inferred) | 0 |

Proposal 4

| Use case | sps_TS_enabled | sps_tsrc_disable | ( sh_SDH ¦¦ sh_DQ ) | sh_tsrc_disable_flag | Number of SH bits |
| --- | --- | --- | --- | --- | --- |
| TS disabled | 0 | x (0 inferred) | 0/1 | x (0 inferred) | 1 |
| Lossy in CLVS* | 1 | 0 | (0)/1 | (0/1)x (0 inferred) | 2/1 |
| lossless in CLVS** | 1 | 1 | x (0 inferred) | x (1 inferred) | 0 |
| Mixed LL lossy Slices | 1 | 0 | 1 | x (0 inferred) | 1 |
| Mixed LL lossless Slices | 1 | 0 | 0 | 1 | 2 |
| RRC only | 1 | 1 | x (0 inferred) | x (0 inferred) | 0 |

Finally, the following table provides a comparison between SH of all different methods:

|  | Number of bits in SH |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Combination 1: Method 1 + variation 2 | Combination 2: Method 1 + variation 3 | Combination 3: Method 3 + variation 2 | Combination 4: Method 3 + variation 3 | Proposal 1 | Proposal 2 |
| TS disabled | 1 | 1 | 1 | 1 | 1 | 1 |
| Lossy in CLVS | 1 | 1 | 2 | 2/1 | 2 | 2/1 |
| lossless in CLVS** | 1 | 1 | 1 | 1 | 0 | 0 |
| Mixed LL lossy Slices | 2 | 1 | 2 | 1 | 2 | 1 |
| Mixed LL lossless Slices | 1 | 2 | 1 | 2 | 1 | 2 |
| RRC only | 1 | 1 | 1 | 1 | 0 | 0 |

Columns in blue are obtained from JVET-R0483 with the following comment (*some disagree whether such use cases are realistic.)

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).