| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0185 |
| --- | --- |

| Title: | AHG9: On syntax elements signalling in VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Hendry Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | seethal.paluri@lge.com dr.hendry@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics. |  |  |

____________________________

# Abstract

This contribution proposed the following changes to the signalling of some syntax elements in VPS:

- Condition the signalling of vps_num_dpb_params on the value of each_layer_is_an_ols_flag instead of vps_all_independent_layers_flag.

- When DPB parameter signalling is present in the VPS, constrain the number of DPB parameters present in the VPS to be greater than 0 by modifying the nomenclature of vps_num_dpb_params to vps_num_dpb_params_minus1.

- When each layer is an OLS, there is no need to signal DPB parameter in VPS since it is already present in the SPS of each layer.

- When the number of profile, tier, and level present in the VPS is the same as the number of output layer set, the signalling of mapping between profile, tier, and level structure and OLS is omitted (i.e, ols_ptl_idx[ i ] is not present). In that case, the value of ols_ptl_idx[ i ] is inferred to be equal to i.

# Proposal 1: On the condition for the signalling of vps_num_dpb_params

It is asserted that when vps_all_independent_layers_flag is equal to 1, it does not mean that each layer is an OLS by itself as the value of each_layer_is_an_ols_flag can still be equal to 0. In the 17th JVET meeting, it was agreed that DPB parameter is signalled per OLS, instead of per layer. Therefore, the current condition of signalling of vps_num_dpb_params is not accurate and need to be updated.

We propose to modify the condition for signalling the syntax element vps_num_dpb_params to be based on the value of each_layer_is_an_ols_flag, instead of vps_all_independent_layers_flag.

The spec text changes for the proposal is as shown below.

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| if( !vps_all_independent_layers_flag !each_layer_is_an_ols_flag) |  |  |
| vps_num_dpb_params | ue(v) |
| … |  |  |
| } |  |  |

# Proposal 2: On the minimum number of DPB params signalled in VPS

It is asserted that when each layer is an OLS, there is no DPB parameters need to be signalled in the VPS, otherwise, there shall be at least one DPB parameter structure present in the VPS. In the current VVC working draft, the value range of vps_num_dpb_params is from 0 to 16, which is problematic as there must be at least one DPB parameter structure when not every layer is an OLS.

We propose to change the syntax element vps_num_dpb_params to be vps_num_dpb_params_minus1 when it is present to avoid the possibility the signalled value to be equal to 0.

The spec text changes for the proposal is as shown below.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !vps_all_independent_layers_flag ) |  |
| vps_num_dpb_params_minus1 | ue(v) |
| if( vps_num_dpb_params VpsNumDpbParams > 0 && vps_max_sublayers_minus1 > 0 ) |  |
| vps_sublayer_dpb_params_present_flag | u(1) |
| for( i = 0; i < vps_num_dpb_params VpsNumDpbParams; i++ ) { |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |
| dpb_max_temporal_id[ i ] | u(3) |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < TotalNumOlss; i++ ) { |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |
| ols_dpb_pic_width[ i ] | ue(v) |
| ols_dpb_pic_height[ i ] | ue(v) |
| if(vps_num_dpb_params VpsNumDpbParams > 1 ) |  |
| ols_dpb_params_idx[ i ] | ue(v) |
| } |  |
| } |  |

vps_num_dpb_params_minus1 plus 1 specifies the number of dpb_parameters( ) syntax strutcures in the VPS. The value of vps_num_dpb_params_minus1 shall be in the range of 0 to 16, inclusive. When not present, the value of vps_num_dpb_params_minus1 is inferred to be equal to 0.

The variable VpsNumDpbParams is defined as follows:

If vps_num_dpb_params_minus1 is not present, the value of VpsNumDpbParams is equal to 0, otheriwse, the value of VpsNumDpbParams is equal to vps_num_dpb_params_minus1 + 1.

ols_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params_minus1, inclusive. When ols_dpb_params_idx[ i ] is not present, the value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

# Proposal 3: On signalling DPB params and OLS mapping

When each layer is an OLS, it is asserted that there is no need to signal DPB parameter in VPS since they are already present in the SPS of each layer.

We propose to not signalling DPB inforamtion in the VPS when each layer is an OLS. The spec text changes for the proposal is as shown below.

It is remarked that this changes may be considered as editorial changes.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if(!each_layer_is_an_ols_flag ) { |  |
| vps_num_dpb_params_minus1 | ue(v) |
| if( VpsNumDpbParams > 0 && vps_max_sublayers_minus1 > 0 ) |  |
| vps_sublayer_dpb_params_present_flag | u(1) |
| for( i = 0; i < VpsNumDpbParams; i++ ) { |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |
| dpb_max_temporal_id[ i ] | u(3) |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < TotalNumOlss; i++ ) { |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |
| ols_dpb_pic_width[ i ] | ue(v) |
| ols_dpb_pic_height[ i ] | ue(v) |
| if( VpsNumDpbParams > 1 ) |  |
| ols_dpb_params_idx[ i ] | ue(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

# Proposal 4: On signalling of ols_ptl_idx[ i ]

In the 17th JVET meeting, it was agreed that PTL index is also signalled for every OLS when VPS is present. This makes that possibility that case of the number of signalled PTL structure and the number of OLS are the same becomes common and they can be mapped one to one for more efficient signalling.

We propose that when the number of PTL structure is the same as the number of OLS, the signalling of ols_ptl_idx[ i ] is omitted and its value is inferred to be equal to i.

The spec text changes for the proposal is as shown below.

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| … | u(4) |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| if( vps_num_ptls_minus1 + 1 != TotalNumOlss && vps_num_ptls_ minus1 > 0 ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_num_dpb_params | ue(v) |
| … |  |
| } |  |

ols_ptl_idx[ i ] specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS. When present, the value of ols_ptl_idx[ i ] shall be in the range of 0 to vps_num_ptls_minus1, inclusive. When vps_num_ptls_minus1 + 1 is equal to TotalNumOlss, the value of ols_ptl_idx[ i ] is inferred to be equal to i. When vps_num_ptls_minus1 is equal to 0, the value of ols_ptl_idx[ i ] is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structure that applies to the i-th OLS is also present in the SPS referred to by the layer in the i-th OLS. It is a requirement of bitstream conformance that, when NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structures signalled in the VPS and in the SPS for the i-th OLS shall be identical.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).