| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0188-r1 |
| --- | --- |

| Title: | AHG12: On signaling of rectangular slice height and width |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seung Hwan Kim Seethal Paluri 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | dr.hendry@lge.com seunghwan3.kim@lge.com seethal.paluri@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

This contribution proposed replacing one condition (i.e., not signalling the syntax element when there is only one tile column or row) in the signalling of slice_width_in_tiles_minus1[ i ] and slice_height_in_tiles_minus1[ i ]. The replacement conditions are as follows:

- When the first tile of a rectangular slice is one of the tile(s) at the last tile column of the picture, the syntax element slice_width_in_tiles_minus1[ i ] is not present and inferred to be equal to 0.

- When the first tile (i.e., the tile at the top-left corner) of a rectangular slice is one of the tile(s) at the last tile row of the picture, the syntax element slice_height_in_tiles_minus1[ i ] is not present and inferred to be equal to 0.

It is remarked that the information about the first tile of a rectangular slice, the last tile column and the last tile row are already available in the current VVC working draft.

In the first revision of this contribution the proposed spec text was updated to remove unnecessary part and editorial update was made.

# Proposal

It is asserted that for signalling of rectangular slice, there is no need to signal the information about the slice width in tiles if the first tile of the slice is one of the tile in the last tile column of the picture. This is because the width for such rectangular slice cannot be other than 1. The same assertion applies to the signalling of rectangular slice’s height. Slice height information does not need to be signalled if the first tile for the slice is one of the tile in the last tile row of the picture.

Based on the above assertion, we propose the following changes to the signalling of syntax elements slice_width_in_tiles_minus1[ i ] and slice_height_in_tiles_minus1[ i ]:

- When the first tile of a rectangular slice is one of the tile(s) at the last tile column of the picture, the syntax element slice_width_in_tiles_minus1[ i ] is not present and inferred to be equal to 0.

- When the first tile (i.e., the tile at the top-left corner) of a rectangular slice is one of the tile(s) at the last tile row of the picture, the syntax element slice_height_in_tiles_minus1[ i ] is not present and inferred to be equal to 0.

# Proposed spec text changes

Texts that are newly added are highlighted and texts that are removed are marked as stricken-red-font text.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| ... |  |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0 ) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if( SliceTopLeftTileIdx[ i ] % NumTileColumns != NumTileColumns – 1 NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( ( SliceTopLeftTileIdx[ i ] / NumTileColumns != NumTileRows – 1 ) NumTileRows > 1 &&( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| ... |  |
| } |  |
| } |  |
| ... |  |
| } |  |
| ... |  |
| } |  |

...

slice_width_in_tiles_minus1[ i ] plus 1 specifies the width of the i-th rectangular slice in units of tile columns. The value of slice_width_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileColumns − 1, inclusive. When not present, the value of slice_width_in_tiles_minus1[ i ] is inferred to be equal to 0.

When i is less than num_slices_in_pic_minus1 and NumTileColumns is equal to 1, the value of slice_width_in_tiles_minus1[ i ] is inferred to be equal to 0.

slice_height_in_tiles_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of tile rows when num_exp_slices_in_tile[ i ] is equal to 0. The value of slice_height_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileRows − 1, inclusive.

When i is less than num_slices_in_pic_minus1 and slice_height_in_tiles_minus1[ i ] is not present, it is inferred to be equal to NumTileRows  = =  1 ? 0 : slice_height_in_tiles_minus1[ i − 1 ].

When slice_height_in_tiles_minus1[ i ] is not present, it is inferred as follows:

- If SliceTopLeftTileIdx[ i ] % NumTileColumns is equal to NumTileColumns – 1, the value of slice_height_in_tiles_minus1[ i ] is inferred to be equal to 0

- Otherwise, it is inferred to be equal to value of slice_height_in_tiles_minus1[ i − 1 ].

NOTE: the value of slice_height_in_tiles_minus1[ i ] for all slices within a tile are set to be the same.

...

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).