| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0189 |
| --- | --- |

| Title: | AHG9: On picture_header_in_slice_header_flag syntax element |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seung Hwan Kim Junghak Nam Hyeongmun Jang Jaehyun Lim 10225 Willow Creek Rd, San Diego, CA 92131, USA 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772, South Korea | Email | dr.hendry@lge.com seunghwan3.kim@lge.com junghak.nam@lge.com hm.jang@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposed the following changes related to the signalling of syntax element picture_header_in_slice_header_flag:

- When subpicture is enabled, picture header information shall not be present in slice header. It is proposed to add a constraint such that when subpicture_info_present_flag is equal to 1, the value of picture_header_in_slice_header_flag shall be equal to 0.

- When picture_header_in_slice_header_flag syntax element is equal to 1, both slice_subpic_id and slice_address syntax elements are not present in slice header as their values can be inferred.

# Proposal 1

It is asserted that when a picture consists of only one slice and has picture header be present inside slice header (i.e., picture_header_in_slice_header_flag is equal to 1), subpicture, which is designed to support bitstream extraction and merging use cases, should not be present.

In subpicture extraction case, the original bitstream would not have the value of picture_header_in_slice_header_flag to be equal to 1 because when subpicture is present to allow extraction, the number of subpicture would be greater than 1 and likewise the number of slice in each picture would be greater than 1. Therefore, from the subpicture extraction use case, when subpicture is present, the value of picture_header_in_slice_header_flag cannot be equal to 1.

In subpicture merging case, the new bitstream resulted from the merging would have at least two slices and therefore the value of picture_header_in_slice_header_flag cannot be equal to 1. Based on this observation, subpicture feature and signalling picture header in slice header feature cannot be used together.

We propose to have exclusive usage between subpicture and presence of picture header information in slice header. It is proposed to add a constraint as follows:

picture_header_in_slice_header_flag equal to 1 specifies that the PH syntax structure is present in the slice header. picture_header_in_slice_header_flag equal to 0 specifies that the PH syntax structure is not present in the slice header.

It is a requirement of bitstream conformance that the value of picture_header_in_slice_header_flag shall be the same in all coded slices in a CLVS.

When picture_header_in_slice_header_flag is equal to 1 for a coded slice, it is a requirement of bitstream conformance that no VCL NAL unit with nal_unit_type equal to PH_NUT shall be present in the CLVS.

When picture_header_in_slice_header_flag is equal to 0, all coded slices in the current picture shall have picture_header_in_slice_header_flag is equal to 0, and the current PU shall have a PH NAL unit.

It is a requirement of bitstream conformance that when the value of subpic_info_present_flag is equal to 1, the value of picture_header_in_slice_header_flag shall be equal to 0.

# Proposal 2

In the current VVC specification, when a picture has subpicture information, subpicture identification, e.g. slice_subpic_id, to which the current slice belongs to is signaled in the slice header. In addition, when a picture has more than one slice, the slice address relative to the subpicture where the slice belongs to is signaled. However, when there is only one slice in each picture in the CLVS, those syntaxes can be omitted.

We propose to update the conditions of the presence of slice_subic_id and slice_address to also consider the value of picture_header_in_slice_header_flag as shown in the table below:

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if( picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| if( !picture_header_in_slice_header_flag && subpic_info_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( !picture_header_in_slice_header_flag && ( ( rect_slice_flag && NumSlicesInSubpic[ CurrSubpicIdx ] > 1 ) ¦ ¦ ( !rect_slice_flag && NumTilesInPic > 1 ) ) ) |  |
| slice_address | u(v) |
| for( i = 0; i < NumExtraShBits; i++ ) |  |
| sh_extra_bit[ i ] | u(1) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| … |  |
| } |  |

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).