| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0190 |
| --- | --- |

| Title: | Post-filter hint based on ALF classification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Boon Teo Hai Wei Sun Chong Soon Lim | Tel: Email: | +65-6550-5417 hanboon.teo@sg.panasonic.com haiwei.sun@sg.panasonic.com chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution is a further follow-up proposal on JVET-Q0319 and JVET-P0179. In the case when ALF module is disabled through bitstream header, post-filter hint based on ALF SEI  is decoded. The proposed SEI is used to enable the ALF module to filter the reconstructed image for output using the accompanied ALF filter flags and filter coefficients. This contribution provides 3 evidences about possible application cases and their benefit. 2 alternative aspects of ALF SEI signalling were proposed: Aspect 1 uses APS id to refer to the APS containing the filter coefficients, Aspect 2 signalls the filter coefficients in the SEI itself. The source code for encoder and decoder implementations are available.

# Introduction

## ALF SEI

In the VTM8, an Adaptive Loop Filter (ALF) with block-based filter adaption is applied when ALF module is enabled in the bitstream (Figure 1).

Figure 1 – VTM8 decoder block diagram

When ALF module is disabled in the bitstream header, the decoder block diagram becomes Figure 2.

Figure 2 – VTM8 decoder block diagram when ALF is disabled

The adaptive loop filter control SEI (ALF SEI) was proposed in JVET-Q0319 [1] and JVET-P0179 [2]. The SEI is used to enable the ALF module to filter the reconstructed image for output (Figure 3).

Figure 3 – VTM8 decoder block diagram when ALF is disabled with proposed SEI

## Software implementation

For effective use of adaptive loop filter control SEI, one implementation method is suggested. In VTM8, maxDpbPicBuf size is 8. Figure 4 shows an example of VTM8 DPB buffer implementation where each buffer space consists of a flag to indicate “needed/not needed for output” and a flag to indicate “used/unused for short/long-term reference”.

Figure 4 – VTM8 DPB buffer implementation

Figure 5 shows an example of the proposed DPB buffer implementation. Each buffer space consists of:

- A set of flag and memory space to indicate “needed/not needed for output” and to store reconstructed image (before ALF), and

- A set of flag and memory space to indicate “used/unused for short/long-term reference” and to store images for output/display (e.g. ALF filtered reconstructed image).

Figure 5 – Proposed DPB buffer implementation

# Use case applications

ALF is designed in VTM to improve PSNR quality. Its filter coefficients are calculated in the encoder and the filter sets with the lowest rate distortion costs are selected and signalled in APS.

The purpose of applying post-filter is for image control, not quality improvement. The post-filter hint SEI in HEVC has the similar concept. Table 1 shows the advantages of using the proposal with VVC. Using the pre-filter approach will cause bits to increase. Post-filter hint SEI in HEVC can only apply a same filter to the whole picture. The categorizing (classification) is an important feature of ALF module and it is expected to expand the use case as describes in the following sub-sections.

Table 1 Advantages of proposed comparing to different methods of apply filters

| Num | Pre-filter | HEVC post-filter hint | Proposed |
| --- | --- | --- | --- |
| 1 | Separate module | Separate module | Built-in VVC |
| 2 | Determined by content provider, but typically: Picture level on/off No classification 1 set of filters | Picture level on/off | CTU level on/off |
| 3 |  | No classification | 4x4 level luma block classification |
| 4 |  | 1 set of filters for luma, 1 set of filters for Cb, 1 set of filters for Cr. | Up to 25 sets of luma filters, 1 set of filters for Cb, 1 set of filters for Cr. |
| 5 | Sharpening original source | At same bitrate, smaller QP used compared to pre-filter. | At same bitrate, smaller QP used compared to pre-filter. |

## Sharpening

Sharpening filter is commonly used to enhance the edges of image/video. In order not to sharpen noise, regions can be separated by selecting the classification indexes that contains details and those that contains flat area. An example is shown in Figure 6. The regions with details can be applied with sharpening filter and the regions with flat area can be applied with smoothening filter.

Selected regions with details (other indexes)

Selected regions with flat area

(indexes = 0,1,5,6,10,11,15,16,20,21)

Figure 6 – An example of separating the picture into 2 regions using ALF classification index

## Bokeh effect

foreground regions (other indexes)

background regions (indexes = 3,4,8,9,19)

Bokeh generates the effect of out-of-focus for background. This is a commonly used filter effect in photography. With classification index information, the picture can be separated into foreground and background regions. An example is shown in Figure 7. Using the sets of blurring and sharpening filters, the bokeh effect can be achieved in the video.

Figure 7 – An example of separating picture into foreground and background regions using ALF classification index

## Adaptive denoising

Noise reduction is used in video processing to remove background noise or camera noise. As noise is more noticeable in flat regions, the set of denoising filters used for detailed regions can be different then the set of denoising filters used for flat regions. With classification index information, pictures can be separated into different regions. An example of separating picture is shown in Figure 8. On flat regions, for an example, stronger denoising filters can be selected as noise is more noticeable in flat regions. On detailed regions with strong edge, for another example, weaker denoising filters can be selected to maintain edge sharpness.

detailed regions with strong edge

(other indexes)

flat regions

(indexes = 0,1,5,6,10,11,15,16,20,21)

detailed regions without strong edge

(indexes = 2,3,4,7)

Figure 8 – An example of separating picture into flat and detailed regions using ALF classification index

# Feasibility study

The relationship between ALF classification index (0~24) and regions from the above examples of applications are as follows.

As shown in Figure 9(a) for the example of sharpening, flat regions come from regions with low activity and detailed regions come from regions with high activity.

As shown in Figure 9(b) for the example of bokeh effect, grass regions generally come from the detailed regions without strong edge.

As shown in Figure 9(c) for the example of adaptive denoising, flat regions come from regions with low activity and detailed regions come from regions with high activity. In the detailed regions, strong edge comes from the regions with strong direction.

Figure 9 – The relationship between ALF classification index and regions in the above examples of applications

Based on Figure 9, activity and direction of 4x4 luma blocks can be used to separate pictures into different regions. The filters required for filtering different regions can then be signalled.

# Proposal Syntax

The syntax of ALF SEI is as follows.

## Aspect 1: Signalling of adaptation_parameter_set_id in ALF SEI

| adaptive_loop_filter_control( payloadSize ) { | Descriptor |
| --- | --- |
| alfctl_adaptation_parameter_set_id | u(3) |
| alf_luma_filter_enable_flag | u(1) |
| alf_cb_filter_enable_flag | u(1) |
| alf_cr_filter_enable_flag | u(1) |
| alf_cc_cb_filter_enable_flag | u(1) |
| alf_cc_cr_filter_enable_flag | u(1) |
| } |  |

## Aspect 2: Signalling of ALF filters in ALF SEI

| adaptive_loop_filter_control( payloadSize ) { | Descriptor |
| --- | --- |
| alf_luma_filter_enable_flag | u(1) |
| alf_cb_filter_enable_flag | u(1) |
| alf_cr_filter_enable_flag | u(1) |
| alf_cc_cb_filter_enable_flag | u(1) |
| alf_cc_cr_filter_enable_flag | u(1) |
| alf_data( ) |  |
| } |  |

The semantics of ALF SEI are as follows. Note: adaptation_parameter_set_id is applicable for aspect 1.

The adaptive loop filter control SEI message provides information to use the adaptive loop filter to modify the pictures for output. When sps_alf_enabled_flag of the bitstream is equal to 1, there shall be no adaptive loop filter control SEI messages present in the CLVS.

alfctl_adaptation_parameter_set_id, when greater than 0, specifies the value of adaptation_parameter_set_id for the APS referred to by the ALF SEI. When alfctl_adaptation_parameter_set_id is equal to 0, the ALF SEI does not refer to a APS and fixed filter coefficients AlfFixFiltCoeff[ i ][ j ] with i = 0..64, j = 0..11 are applied.

alf_luma_filter_enable_flag equal to 1 specifies that adaptive loop filter is applied to the luminance component. alf_luma_filter_enable_flag equal to 0 specifies that adaptive loop filter is not applied to the luminance component. When alf_luma_filter_enable_flag is not present, it is inferred to be equal to 0. If alf_luma_filter_enable_flag equal to 1, alf_ctb_flag[0][][] for all ctb in the luminance component equal to 1.

alf_cb_filter_enable_flag equal to 1 specifies that adaptive loop filter is applied to the Cb color component. alf_cb_filter_enable_flag equal to 0 specifies that adaptive loop filter is not applied to the Cb color component. When alf_cb_filter_enable_flag is not present, it is inferred to be equal to 0. If alf_cb_filter_enable_flag equal to 1, alf_ctb_flag[1][][] for all ctb in the Cb color component equal to 1.

alf_cr_filter_enable_flag equal to 1 specifies that adaptive loop filter is applied to the Cr color component. alf_cr_filter_enable_flag equal to 0 specifies that adaptive loop filter is not applied to the Cr color component. When alf_cr_filter_enable_flag is not present, it is inferred to be equal to 0. If alf_cr_filter_enable_flag equal to 1, alf_ctb_flag[2][][] for all ctb in the Cr color component equal to 1.

alf_cc_cb_filter_enable_flag equal to 1 specifies that cross-component filter is applied to the Cb color component. alf_cc_cb_filter_enable_flag equal to 0 specifies that cross-component filter is not applied to the Cb color component. When alf_cc_cb_filter_enable_flag is not present, it is inferred to be equal to 0.

alf_cc_cr_filter_enable_flag equal to 1 specifies that cross-component filter is applied to the Cr color component. alf_cc_cr_filter_enable_flag equal to 0 specifies that cross-component filter is not applied to the Cr color component. When alf_cc_cr_filter_enable_flag is not present, it is inferred to be equal to 0.

When ChromaArrayType is equal to 0, alf_cb_filter_enable_flag, alf_cr_filter_enable_flag, alf_cc_cb_filter_enable_flag and alf_cc_cr_filter_enable_flag shall be equal to 0. At least one of the values of alf_luma_filter_enable_flag, alf_cb_filter_enable_flag, alf_cr_filter_enable_flag, alf_cc_cb_filter_enable_flag, and alf_cc_cr_filter_enable_flag shall be equal to 1.

# Summary

The ALF SEI introduces new application uses for VVC. The filtering effects such as sharpening, bokeh and adaptive denoising can be achieved from using the ALF SEI with the advantage that no additional filtering modules are needed. We propose to adopt ALF SEI into VVC specification to expand the use case of VVC.

# References

H.-B. Teo, H.-W. Sun, C.-S. Lim, "Non-CE5: On SEI for ALF", JVET-Q0319, JVET 17th Meeting: Brussels, BE, 7–17 January 2020.

- H.-B Teo, H.-W Sun, C.-S Lim, “Non-CE5: ALF as a post-filter”, JVET-P0179, JVET 16th Meeting: Geneva, CH, 1–11 October 2019.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).