| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0195 |
| --- | --- |

| Title: | AHG8/AHG9: On HRD structure and OLS mapping signalling in VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution asserted that the signalling of HRD for OLS in VPS and SPS in the current VVC working draft is not consistent. When signalled in SPS, HRD for a single-layered OLS is optional, which is controlled by sps_general_hrd_params_present_flag. On the other hand, for signalling of HRDs for multi-layered OLSs in VPS, the signalling is either all of them are present or none of them are present.

To resolve the asserted inconsistency, it is proposed that the mapping between HRD structure and individual OLS with multiple layers is optional. To implement that, the following two options are suggested:

Option 1 is summarized as follows:

- A flag called ols_hrd_present_flag[ i ] is signalled for each OLS that contains more than one layer to specify whether ols_hrd_idx[ i ] is present or not.

- It is constrained that when vps_general_hrd_params_present_flag is equal to 1, there shall be at least one value of ols_hrd_present_flag[ i ] for i in the range from 0 to TotalNumOlss – 1, inclusive, that is equal to 1.

Option 2 is summarized as follows:

- A special value of the HRD index for OLS is assigned such that the value means the OLS has no associated HRD information / parameter. For this, the following applies:

  - Change the ols_hrd_idx[ i ] to ols_hrd_idx_plus1[ i ].

  - If ols_hrd_idx_plus1[ i ] is equal to 0, it means the i-th OLS has no associated HRD parameter structure, otherwise, the index of HRD parameter structure associated with the i-th OLS is ols_hrd_idx_plus1[ i ] – 1.

- It is constrained that when vps_general_hrd_params_present_flag is equal to 1, there shall be at least one value of ols_hrd_idx_plus1[ i ] for i in the range from 0 to TotalNumOlss – 1, inclusive, that is not equal to 0.

# Problem description

In the current VVC spec, when an OLS contains only one layer, the HRD for that OLS is present in the SPS referred to by the layer; on the other hand, when an OLS contains multiple layers, the HRD for it may be present in the VPS, together with HRD for other OLSs that contains multiple layers. It is asserted that for signalling in SPS, HRD for a single-layered OLS is optional, which is controlled by sps_general_hrd_params_present_flag. On the other hand, for signalling of HRDs for multi-layered OLSs in VPS, the signalling is either all of them are present or none of them are present.

We propose that the signalling for mapping between HRD structure and OLS be made consistent between OLS with single layer and OLSs with multiple layers.

# Proposal and spec text changes

To make the signalling of HRD structure mapping and OLS consistent, we propose that the mapping between HRD structure and individual OLS with multiple layers is optional.

## Option 1

The proposed changes to the VVC working draft for option 1 are as follow:

- A flag called ols_hrd_present_flag[ i ] is signalled for each OLS that contains more than one layer to specify whether ols_hrd_idx[ i ] is present or not.

- It is constrained that when vps_general_hrd_params_present_flag is equal to 1, there shall be at least one value of ols_hrd_present_flag[ i ] for i in the range from 0 to TotalNumOlss – 1, inclusive, that is equal to 1.

The spec text changes for the proposed option 1 is shown below.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| ... |  |
| if( !each_layer_is_an_ols_flag ) |  |
| vps_general_hrd_params_present_flag | u(1) |
| if( vps_general_hrd_params_present_flag ) { |  |
| general_hrd_parameters( ) |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |
| vps_sublayer_cpb_params_present_flag | u(1) |
| num_ols_hrd_params_minus1 | ue(v) |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |
| hrd_max_tid[ i ] | u(3) |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ] ) |  |
| } |  |
| if( num_ols_hrd_params_minus1 + 1 != TotalNumOlss && num_ols_hrd_params_minus1 > 0 ) |  |
| for( i = 1; i < TotalNumOlss; i++ ) |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |
| ols_hrd_present_flag[ i ] | u(1) |
| if( ols_hrd_present_flag[ i ] ) |  |
| ols_hrd_idx[ i ] | ue(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

...

ols_hrd_present_flag[ i ] equal 1 specifies ols_hrd_idx[ i ] is present. ols_hrd_present_flag[ i ] equal 0 specifies that ols_hrd_idx[ i ] is not present.

When vps_general_hrd_params_present_flag is equal to 1, there shall be at least one value of ols_hrd_present_flag[ i ] for i in the range from 0 to TotalNumOlss – 1, inclusive, equal to 1.

ols_hrd_idx[ i ] specifies the index, to the list of ols_hrd_parameters( ) syntax structures in the VPS, of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. The value of ols_hrd_idx[[ i ] shall be in the range of 0 to num_ols_hrd_params_minus1, inclusive.

When NumLayersInOls[ i ] is equal to 1, the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

If the value of num_ols_hrd_param_minus1 + 1 is equal to TotalNumOlss, the value of ols_hrd_idx[ i ] is inferred to be equal to i. Otherwise, when NumLayersInOls[ i ] is greater than 1 and num_ols_hrd_params_minus1 is equal to 0, the value of ols_hrd_idx[[ i ] is inferred to be equal to 0.

...

## Option 2

The proposed changes to the VVC working draft for option 2 are as follow:

- A special value of the HRD index for OLS is assigned such that the value means the OLS has not associated HRD information / parameter. For this, the following applies:

  - Change the ols_hrd_idx[ i ] to ols_hrd_idx_plus1[ i ].

  - If ols_hrd_idx_plus1[ i ] is equal to 0, it means the i-th OLS has no associated HRD parameter, otherwise, the index of HRD parameter structure associated with the i-th OLS is ols_hrd_idx_plus1[ i ] – 1.

- It is constrained that when vps_general_hrd_params_present_flag is equal to 1, there shall be at least one value of ols_hrd_idx_plus1[ i ] for i in the range from 0 to TotalNumOlss – 1, inclusive, that is not equal to 0.

The spec text changes for the proposed option 1 is shown below.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| ... |  |
| if( !each_layer_is_an_ols_flag ) |  |
| vps_general_hrd_params_present_flag | u(1) |
| if( vps_general_hrd_params_present_flag ) { |  |
| general_hrd_parameters( ) |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |
| vps_sublayer_cpb_params_present_flag | u(1) |
| num_ols_hrd_params_minus1 | ue(v) |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |
| hrd_max_tid[ i ] | u(3) |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ] ) |  |
| } |  |
| if( num_ols_hrd_params_minus1 + 1 != TotalNumOlss && num_ols_hrd_params_minus1 > 0 ) |  |
| for( i = 1; i < TotalNumOlss; i++ ) |  |
| if( NumLayersInOls[ i ] > 1 ) |  |
| ols_hrd_idx_plus1[ i ] | ue(v) |
| } |  |
| ... |  |
| } |  |

...

ols_hrd_idx_plus1[ i ] minus 1 specifies the index, to the list of ols_hrd_parameters( ) syntax structures in the VPS, of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. The value of ols_hrd_idx_plus1[ i ] shall be in the range of 0 to num_ols_hrd_params_minus1 + 1, inclusive.

When vps_general_hrd_params_present_flag is equal to 1, there shall be at least one value of ols_hrd_idx_plus1[ i ] for i in the range from 0 to TotalNumOlss – 1, inclusive, not equal to 0.

When NumLayersInOls[ i ] is equal to 1, the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

If the value of num_ols_hrd_param_minus1 + 1 is equal to TotalNumOlss, the value of ols_hrd_idx[ i ] is inferred to be equal to i. Otherwise, when NumLayersInOls[ i ] is greater than 1 and num_ols_hrd_params_minus1 is equal to 0, the value of ols_hrd_idx[[ i ] is inferred to be equal to 0.

...

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).