| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0196 |
| --- | --- |

| Title: | AHG8: On signalling of DPB parameters in the VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takahiro Nishi Kiyofumi Abe Virginie Drugeon | Tel: Email: | +49-6103-766240 nishi.takahiro@jp.panasonic.com abe.kiyo@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In the current draft specification, the authors believe that there is an issue whereby DPB parameters are not always defined for multi-layer OLSs. This document proposes some modifications to solve this issue and also proposes some clean-ups.

# Introduction

In the current draft specification, there is an issue whereby DPB parameters are not always defined for multi-layer OLSs. Specifically, this would be the case in the following two cases: 1) vps_all_independent_layers_flag is equal to 1 and there is an OLS which contains multiple layers, and case 2) vps_all_independent_layers_flag is equal to 0, vps_num_dpb_params is equal to 0 and there is an OLS which contains multiple layers.

This document proposes some modifications to solve the issues explained above and also proposes some clean-ups.

Proposed modifications are summarized as follows:

- Replace “if( !vps_all_independent_layers_flag )” with “if( !each_layer_is_an_ols_flag )”

  - DPB parameters need to be defined in the VPS if there is an OLS which contains multiple layers. There may be an OLS which contains multiple independent layers even when vps_all_independent_layers_flag is equal to 1.

  - each_layer_is_an_ols_flag seems more appropriate as condition for the presence of the DPB related syntax since it is set to 1 when each OLS contains only one layer.

- (on top of a.) Prohibit vps_num_dpb_params equal to 0, when each_layer_is_an_ols_flag is equal to 0

  - If vps_num_dpb_params is allowed to be equal to 0 even when each_layer_is_an_ols_flag is equal to 0, DPB parameters may not be defined for multi-layer OLSs.

  - At least one DPB parameters syntax structure needs to be signalled in the VPS for such a case.

- Change upper limit of vps_num_dpb_params from 16 to TotalNumOlss – 1

  - 0-th OLS never refers to any of the DPB parameters in the VPS, because it is always single layer. TotalNumOlss – 1 is considered to be sufficient.

- Remove 0-th loop for ols_dpb_pic_width[ i ], ols_dpb_pic_height[ i ] and ols_dpb_params_idx[ i ]

  - 0-th OLS always includes only one layer. DPB parameters in VPS will not be referred by such an OLS.

# Proposed changes

Proposed changes over the draft specification are presented below.

7.3.2.2	Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| if( !each_layer_is_an_ols_flag !vps_all_independent_layers_flag ){ |  |  |
| vps_num_dpb_params_minus1 | ue(v) |
| if( vps_num_dpb_params > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i <= vps_num_dpb_params_minus1; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| dpb_max_temporal_id[ i ] | u(3) |  |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 1 i = 0; i < TotalNumOlss; i++ ) { |  |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |  |
| ols_dpb_pic_width[ i ] | ue(v) |  |
| ols_dpb_pic_height[ i ] | ue(v) |  |
| if( vps_num_dpb_params_minus1 > 0 1 ) |  |  |
| ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

7.4.3.2	Video parameter set RBSP semantics

vps_num_dpb_params_minus1 plus 1 specifies the number of dpb_parameters( ) syntax strutcures in the VPS. The value of vps_num_dpb_params_minus1 shall be less than TotalNumOlss − 1. in the range of 0 to 16, inclusive. When not present, the value of vps_num_dpb_params is inferred to be equal to 0.

ols_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params_minus1 − 1, inclusive. When ols_dpb_params_idx[ i ] is not present, the value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the dpb_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

# Alternative option

The following changes over the draft specification would also work.

Instead of prohibiting vps_num_dpb_params is equal to 0, an inference rule to define DPB parameters for multi-layer OLSs is added.

7.3.2.2	Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !each_layer_is_an_ols_flagvps_all_independent_layers_flag ) |  |
| vps_num_dpb_params | ue(v) |
| if( vps_num_dpb_params > 0 && vps_max_sublayers_minus1 > 0 ) |  |
| vps_sublayer_dpb_params_present_flag | u(1) |
| for( i = 0; i < vps_num_dpb_params; i++ ) { |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |
| dpb_max_temporal_id[ i ] | u(3) |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 1 i = 0; i < TotalNumOlss; i++ ) { |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |
| ols_dpb_pic_width[ i ] | ue(v) |
| ols_dpb_pic_height[ i ] | ue(v) |
| if( vps_num_dpb_params > 1 ) |  |
| ols_dpb_params_idx[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |

7.4.3.2	Video parameter set RBSP semantics

vps_num_dpb_params specifies the number of dpb_parameters( ) syntax strutcures in the VPS. The value of vps_num_dpb_params shall be in the range of 0 to TotalNumOlss 16, inclusive. When not present, the value of vps_num_dpb_params is inferred to be equal to 0.

ols_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params − 1, inclusive. When ols_dpb_params_idx[ i ] is not present, the value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the dpb_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

When all of the following conditions are true, the dpb_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the 0-th OLS:

–	each_layer_is_an_ols_flag is equal to 0

–	the value of vps_num_dpb_params is equal to 0

–	NumLayersInOls[ i ] is greater than 1

A.4.1	General tier and level limits

- For an OLS with OLS index TargetOlsIdx, the variables PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, and PicSizeMaxInSamplesY, and the applicable dpb_parameters( ) syntax structure are derived as follows:

  - If NumLayersInOls[ TargetOlsIdx ] is equal to 1, PicWidthMaxInSamplesY is set equal to pic_width_max_in_luma_samples, PicHeightMaxInSamplesY is set equal to pic_height_max_in_luma_samples, PicSizeMaxInSamplesY is set equal to PicWidthMaxInSamplesY * PicHeightMaxInSamplesY, where  pic_width_max_in_luma_samples and pic_height_max_in_luma_samples are found in the SPS referred to by the layer in the OLS, and the applicable dpb_parameters( ) syntax structure is also found in that SPS.

  - Otherwise (NumLayersInOls[ TargetOlsIdx ] is greater than 1), if the value of vps_num_dpb_params is not equal to 0, PicWidthMaxInSamplesY is set equal to ols_dpb_pic_width[ TargetOlsIdx ], PicHeightMaxInSamplesY is set equal to ols_dpb_pic_height[ TargetOlsIdx ], PicSizeMaxInSamplesY is set equal to PicWidthMaxInSamplesY * PicHeightMaxInSamplesY, and the applicable dpb_parameters( ) syntax structure is identified by ols_dpb_params_idx[ TargetOlsIdx ] found in the VPS.

  - Otherwise (NumLayersInOls[ TargetOlsIdx ] is greater than 1 and the value of vps_num_dpb_params is equal to 0), PicWidthMaxInSamplesY is set equal to ols_dpb_pic_width[ TargetOlsIdx ], PicHeightMaxInSamplesY is set equal to ols_dpb_pic_height[ TargetOlsIdx ], PicSizeMaxInSamplesY is set equal to PicWidthMaxInSamplesY * PicHeightMaxInSamplesY, and the applicable dpb_parameters( ) syntax structure is found in the SPS referred to by the layer in the 0-th OLS.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).