| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0197 |
| --- | --- |

| Title: | AHG12: On signalling of loop filter across tiles and slices enabled flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Hendry Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, dr.hendry, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In current VVC working draft, the flags loop_filter_across_tiles_enabled_flag and loop_filter_across_slices_enabled_flag are signalled when it is specified that there is picture partitioning, regardless whether tiles and / or slices are present. It is asserted that loop_filter_across_tiles_enabled_flag is not needed when there is only one tile in the picture and, likewise, loop_filter_across_slices_enabled_flag is not needed when there is no tile that is divided into more than one rectangular slice.

This contribution proposed the following:

- Condition the presence of loop_filter_across_tiles_enabled_flag to be present only when NumTilesInPic is greater than 1.

- Condition the presence of loop_filter_across_tiles_enabled_flag to be present only when there is at least one tile that is divided into more than one rectangular slice.

It is remarked that no action needs to be taken if contribution JVET-R0069 is adopted.

# Proposed method

In the PPS, no_pic_partition_flag equal to 0 specifies each picture referring to the PPS may be partitioned into more than one tile or slice. Consequently, each picture might be partitioned into more than one tiles, more than one slices or both of them. When no_pic_partition_flag is equal to 0, loop_filter_across_tiles_enabled_flag and loop_filter_across_slices_enabled_flag are signalled to control the boundaries of tiles/slices for loop filtering.

It is observed that loop_filter_across_tiles_enabled_flag is signalled even when NumTilesInPic is equal to 1. It means that this flag, i.e., loop_filter_across_tiles_enabled_flag, is signalled redundantly as it has no effect to each picture referring this PPS. Therefore, we propose that loop_filter_across_tiles_enabled_flag is conditionally signalled based on the value of NumTilesInPic. Note that the variable NumTilesInPic has been defined in the current VVC working draft.

Likewise, it is also observed that loop_filter_across_slices_enabled_flag is signalled even when there is no tile that is divided into multiple rectangular slices. Therefore, we propose that loop_filter_across_slices_enabled_flag is conditionally signalled based on the presence of rectangular slices within a tile (i.e., any tile).

# Proposed spec text changes

In the PPS syntax, loop_filter_across_tiles_enable_flag and loop_filter_across_slices_enabled_flag are conditionally signaled as shown in the below syntax table.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| SlicesInTileFlag = 0 |  |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0 ) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| for( j = 0; j < num_exp_slices_in_tile[ i ]; j++ ) |  |
| exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| SlicesInTileFlag = ( NumSlicesInTile[ i ] > 1 ) ? 1 : SlicesInTileFlag |  |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| if( NumTilesInPic > 1 ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( SlicesInTileFlag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| …… |  |
| } |  |

loop_filter_across_slices_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across slice boundaries in pictures referring to the PPS. loop_filter_across_slice_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across slice boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_slices_enabled_flag is inferred to be equal to 1.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).