| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0198 |
| --- | --- |

| Title: | AHG9: On signalling of IDR or GDR picture flag in picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772, South Korea Hendry Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | junghak.nam@lge.com hm.jang@lge.com jaehyun.lim@lge.com dr.hendry@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, the following assertions were made:

- A GDR picture should have at least one slice that is not intra coded, otherwise, it should not be labelled as a GDR but an IRAP instead.

- In the current signalling mechanism for reference picture list (RPL) in picture header (PH), RPL still needs to be signalled in PH even when the picture associated with the picture header is IDR picture and the value of sps_idr_rpl_present_flag is equal to 0. This seems redundant and should be avoided if possible.

This contribution proposed the following changes related to the signalling of syntax elements in PH:

- When gdr_pic_flag is equal to 1, the syntax element ph_inter_slice_allowed_flag is not present because its value can easily be inferred to be equal to 1.

- When gdr_or_irap_pic_flag is equal to 1 and gdr_pic_flag is equal to 0, a new flag called idr_pic_flag is proposed to specify whether the picture associated with the picture header is an IDR picture. When sps_idr_rpl_present_flag is equal to 0 and idr_pic_flag is equal to 1, RPL signalling is not present in the PH, even when the value of rpl_info_in_ph_flag is equal to 1.

# Proposal 1

In the 17th JVET meeting, flags that specify whether signalling of syntax element for inter-predicted slices and intra-predicted slices allowed in picture header were adopted. The signalling of these two flags are early in the picture header but still after the signalling of syntax elements that specify whether the picture associated with picture header is an IRAP or GDR picture.

It is asserted that a GDR picture should have at least one slice that is not intra coded, otherwise, it should not be labelled as a GDR but an IRAP instead. Consequently, there is no need to signal ph_inter_slice_allowed_flag since its value is known.

We propose that the syntax element ph_inter_slice_allowed_flag is not present when the value of gdr_pic_flag is equal to 1.

The proposed spec text changes are shown below.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| if( !gdr_pic_flag ) |  |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| ... |  |
| } |  |

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 0 or 1. When not present, the value of ph_inter_slice_allowed_flag is inferred to be equal to 1.

# Proposal 2

We observe that reference picture list still needs to be signaled in picture header when the picture associated with the picture header is an IDR picture and the value of sps_idr_rpl_present_flag is equal to 0. This is because whether the associated picture is an IDR picture or not is still unknown when picture header is parsed.

We propose the following:

- When gdr_or_irap_pic_flag is equal to 1 and gdr_pic_flag is equal to 0, a new flag called idr_pic_flag is proposed to specify whether the picture associated with the picture header is an IDR picture.

- When sps_idr_rpl_present_flag is equal to 0 and idr_pic_flag is equal to 1, RPL signalling is not present in the PH, even when the value of rpl_info_in_ph_flag is equal to 1

The proposed spec text changes are shown below.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) { |  |
| gdr_pic_flag | u(1) |
| if( !gdr_pic_flag ) |  |
| idr_pic_flag | u(1) |
| } |  |
| … |  |
| if( rpl_info_in_ph_flag && (sps_idr_rpl_present_flag ¦ ¦ !idr_pic_flag) ) |  |
| ref_pic_lists( ) |  |
| … |  |
| } |  |

7.4.3.3 Sequence parameter set RBSP semantics

sps_idr_rpl_present_flag equal to 1 specifies that reference picture list syntax elements are present in picture header or slice headers of IDR pictures. sps_idr_rpl_present_flag equal to 0 specifies that reference picture list syntax elements are not present in picture header or slice headers of IDR pictures.

7.4.3.6 Picture header RBSP semantics

idr_pic_flag equal to 1 specifies the picture associated with the picture header is an IDR picture. idr_pic_flag equal to 0 specifies that the picture associated with the picture header is not a IDR picture. When not present, the value of idr_pic_flag is inferred to be equal to 0.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).