| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0201 |
| --- | --- |

| Title: | AHG9: On prefix and suffix APSs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naël Ouedraogo Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE 3 | Tel: Email: | +33 299876800 nael.ouedraogo@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

In this contribution, it is reported that the random access or the bitstream extraction operations starting at an IRAP picture may lead to the generation of non-conformant bitstream due to the obligation of copying all the needed APS before the IRAP. This is particularly the case when a PU with slices refers to APSs signalled in a previous PU. To solve this issue, it is proposed:

- To constrain suffix APS NAL units to be located after the last VCL NAL unit of the PU.

- To allow prefix and suffix APS NAL units with particular APS identifier and type to have different content.

Second, it is reported that the VVC Draft 8 does not constrain two slices of a PU to refer to two APSs with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type but with different contents when one of the APSs is within a previous PU. In addition to the previous constraints, it is proposed:

- To constrain prefix APS NAL unit to be located before the first VCL NAL unit of the PU.

It is reported that it would solve the current random access issue and ease the parallel decoding based on slices.

# Problem statement

VVC Draft 8 specification has the following statements regarding the APS:

- All APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a PU, regardless of whether they are prefix or suffix APS NAL units, shall have the same content.

- When PREFIX APS NAL units are present in a PU, they shall not follow the last VCL NAL unit of the PU.

- When SUFFIX APS NAL units are present in a PU, they shall not precede the first VCL NAL unit of the PU.

At the 16th Meeting of JVET, it was agreed that it was an unnecessary constraint that slice shall not refer to APS prior to an IRAP. The decision was the following:

- Remove the following constraints: [JVET-P0122][SW/Hendry] Decision (cleanup unnecessary constraints): Agreed

- For intra slices and slices in an IRAP picture, slice_alf_aps_id_luma[ i ] shall not refer to an ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

- For intra slices and slices in an IRAP picture, slice_alf_aps_id_chroma shall not refer to an ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

When decoding or extracting a bitstream starting at a random IRAP, it is thus necessary to provide/copy all the APSs that were sent prior to the IRAP to enable a correct decoding of the bitstream from the IRAP.

For example, the figure 1 represents Picture Units of a bitstream.

Figure 1: Bitstream example

In this bitstream, the PU #N+k is an IRAP. It contains a first slice that refers to an APS with an Id equal to 0 and a second slice that refers to APS with Id equal 1. These two APSs are provided as prefix and suffix APS NAL units within a PU #N that is prior to the IRAP.

When extracting the bitstream starting at the PU #N+k, the two APSs have to be provided prior to the first slice in the new bitstream as represented in the Figure 2 below:

Figure 2: Extraction of the bitstream starting at Picture Unit #N+k

It means that the two APSs of PU #N must be inserted prior to the first slice of PU #N+k. Since suffix APS NAL unit shall not be prior to first VCL NAL unit as per the VVC Draft 8 specification, it is required to rewrite the original suffix APS with Id=1 into a prefix APS NAL unit with Id=1. The rewriting operation is illustrated in Figure 3 below.

Figure 3: Bitstream after extraction

The problem is that although the original bitstream conforms to VVC8 specification, the extracted bitstream is still not conformant with the rewriting operation of Figure 3. Indeed, the Picture unit #N+k now contains two prefix APS NAL units with Id equal to 0 and 1 and two suffix APS NAL units with same Ids but with different contents which is not allowed per the VVC Draft 8 specification.

# Proposal:

## Constraints for suffix APS

This contribution proposes to modify the constraints of the bitstream for APS as follows:

- When SUFFIX APS NAL units are present in a PU, they shall not precede the first follow the last VCL NAL unit of the PU

- All APS NAL units with a particular value of nal_unit_type, with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a PU, regardless of whether they are prefix or suffix APS NAL units, shall have the same content.

The first constraint 1. ensures that APSs present in suffix APS NAL units are for use by NAL units following the current PU. This constraint is not limiting reference to APS since a prefix APS could replace a suffix APS NAL unit when provided prior to the last VCL NAL unit.

The second modification 2. allows suffix and prefix APS NAL units within a PU to contain APSs with same adaptation_parameter_set_id  and aps_params_type  values but with different content.

With this approach, the bitstream of figure 3 would be possible.

## Constraint for prefix APS

Figure 4: Slices referring to different APS content with same adaptation_parameter_set_id of aps_params_type values

The VVC Draft 8 seems not to constrain slices of a PU to refer to different APSs with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type but with different contents. This happens, for example in Figure 4, when the first APS is provided in a previous PU and the second APS is in between two slices of current PU.

In addition to random access issue (as explained above), this makes the parallel decoding (based on slices) complex since each slice may refer to different versions of an APS with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type. For some applications that provides APSs out of band, the inclusion of the APS NAL unit within the VCL NAL units is complex since needs to preserve original order of NAL units.

In addition to constraints of section 2.1, we suggest adding the following constraint to ensure that slices of a PU refer to a unique version of APS:

- When PREFIX APS NAL units are present in a PU, they shall precede the first not follow the last VCL NAL unit of the PU

This constraint allows updates of APS only prior to the first VCL NAL unit of the PU.

# Conclusion

As this proposal provides constraints that fix an issue to enable random access capability in the VVC bitstream, we suggest adopting the 3 aspects of this proposal.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).