| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0202 |
| --- | --- |

| Title: | AhG9: Syntax cleanups when Picture Header is in the Slice Header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Nael Ouedraogo Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

In order to remove some redundant signaling in the slice header, this contribution proposes some modifications of the VVC specifications when the picture header structure is inside the slice header (e.g. the picture_header_in_slice_header_flag is equal to 1) as only one slice is enabled under such condition.

- In a first aspect, when PH structure is in the SH, the syntax elements slice_lmcs_enabled_flag, slice_explicit_scaling_list_used_flag, num_tiles_in_slice_minus1 and slice_address are not signaled in the slice header and their values are inferred.

- In a second aspect, it is proposed to add an explicit constraint to set separate_colour_plane_flag and subpic_info_present_flag to 0 when PH structure is in the SH.

- In a third aspect, it is proposed to clarify the value of the flags qp_delta_info_in_ph_flag, rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, wp_info_in_ph_flag, alf_info_in_ph_flag when PH structure is in the SH by adding semantics.

- In a fourth aspect, it is proposed to avoid the overriding of the reference picture list in the SH when picture_header_in_slice_header_flag is equal to 1.

# Problem Statement

When the picture header is signaled inside the slice header, each picture of the CLVS has only one slice. When only one slice is used, some data do not need to be transmitted in the slice header. This creates additional signaling without improving the flexibility. Moreover, to clarify the VVC specification, and avoid inconsistent bitstream, some configurations should be explicitly avoided.

The 4 following proposals solves the different issues.

# Proposals

## Inferred syntax element when Ph in SH

### slice_lmcs_enabled_flag inferred when Ph in SH

In the current VVC specification, the slice_lmcs_enabled_flag enables LMCS at slice level when it is enabled in the PH. When the PH is in the SH, there is only one slice, so it is not needed to enable this tool twice and it does not give any additional flexibility.

It is proposed to not transmit slice_lmcs_enabled_flag and set it equal to ph_lmcs_enabled_flag when the PH is in SH as depicted in the following:

…

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| … |  |
| if( ph_lmcs_enabled_flag && !picture_header_in_slice_header_flag) |  |
| slice_lmcs_enabled_flag | u(1) |
| … |  |

…

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled for the current slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not enabled for the current slice. When slice_lmcs_enabled_flag is not present, and when picture_header_in_slice_header_flag is set equal to 0,  it is inferred to be equal to 0. When it is not present, and when picture_header_in_slice_header_flag is set equal to 1,  it is inferred to be equal to ph_lmcs_enabled_flag.

…

### slice_explicit_scaling_list_used_flag inferred when PH in SH

In the current VVC specification, the slice_scaling_list_enabled_flag enables the scaling list at slice level when it is enabled in the PH. When the PH is in the SH, there is only one slice, so it is not needed to enable this tool twice and it does not give any additional flexibility.

It is proposed to not transmit slice_scaling_list_enabled_flag, and set it equal to ph_scaling_list_enabled_flag when the PH is in SH as depicted in the following:

…

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| … |  |
| if( ph_explicit_scaling_list_enabled_flag && !picture_header_in_slice_header_flag) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| … |  |

…

slice_explicit_scaling_list_used_flag equal to 1 specifies that the explicit scaling list, which is signalled in the referenced scaling list APS with aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id, is used in the scaling process for transform coefficients when decoding of the current slice. slice_explicit_scaling_list_used_flag equal to 0 specifies that no explicit scaling list is not used in the scaling process for transform coefficients when decoding of the current slice. When not present, and when picture_header_in_slice_header_flag is set equal to 0, the value of slice_explicit_scaling_list_used_flag is inferred to be equal to 0. When not present, and when picture_header_in_slice_header_flag is set equal to 1,  it is inferred to be equal to ph_explicit_scaling_list_enabled_flag.

…

### num_tiles_in_slice_minus1 inferred when PH in SH

The number of tiles in the slices is signaled when the PH is in the SH, but, when the PH is in the SH, it is sure that the current picture contains only one slice. Therefore, the number of tiles in the current picture is the same as the number of tiles in the current picture (given by the variable NumTilesInPic). So, it is not needed to transmit the number of tiles in the current slice.

As the value of num_tiles_in_slice_minus1 does not need to be decoded, when the PH is in the SH, we propose the following modifications:

…

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| … |  |
| if( !rect_slice_flag && (NumTilesInPic > 1) && !picture_header_in_slice_header_flag) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| … |  |

…

num_tiles_in_slice_minus1 plus 1, when present, specifies the number of tiles in the slice. The value of num_tiles_in_slice_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive. When not present, the value of num_tiles_in_slice_minus1 is inferred to be equal to NumTilesInPic-1.

…

### slice_address inferred when PH in SH

The slice address can be signaled when the PH is in the SH, but, in that case, it is sure that the slice address is always set equal to 0 as there is only one slice when the PH is in the SH. So it is not needed to decode this flag when the PH is in the SH.

The following table shows this proposed modification.

…

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| … |  |
| if( ( rect_slice_flag && NumSlicesInSubpic[ CurrSubpicIdx ] > 1 ) ¦ ¦ ( !rect_slice_flag && NumTilesInPic > 1 ) && !picture_header_in_slice_header_flag) |  |
| slice_address | u(v) |
| … |  |

…

## Explicit constraint when PH in SH

### No subpicture when PH in SH

In the current VVC specification, it is possible to signal the subpicture when the PH is in the SH. When for example, there is only one subpicture per picture. For example, when the picture contains only one subpicture, the PH in the SH can be enabled. This seems inconsistent and should be clarified by modifying the semantics of subpic_info_present_flag as the following:

…

subpic_info_present_flag equal to 1 specifies that subpicture information is present for the CLVS and there may be one or more than one subpicture in each picture of the CLVS. subpic_info_present_flag equal to 0 specifies that subpicture information is not present for the CLVS and there is only one subpicture in each picture of the CLVS.

When res_change_in_clvs_allowed_flag is equal to 1, the value of subpic_info_present_flag shall be equal to 0.

When slice headers referring to the PPS contain the PH syntax structure, it is a requirement of bitstream conformance that subpic_info_present_flag shall be equal to 0.

…

### Explicit constraints for separate_colour_plane_flag when PH in SH

In the current VVC specification, it is possible to signal the separate colour plan when the PH is in the SH. This seems inconsistent and should be clarified by modifying the semantics of separate_colour_plane_flag as the following:

…

separate_colour_plane_flag equal to 1 specifies that the three colour components of the 4:4:4 chroma format are coded separately. separate_colour_plane_flag equal to 0 specifies that the colour components are not coded separately. When separate_colour_plane_flag is not present, it is inferred to be equal to 0. When separate_colour_plane_flag is equal to 1, the coded picture consists in three separate components, each of which consists in coded samples of one colour plane (Y, Cb, or Cr) and uses the monochrome coding syntax. In this case, each colour plane is associated with a specific colour_plane_id value.

When slice headers referring to the PPS contain the PH syntax structure, it is a requirement of bitstream conformance that separate_colour_plane_flag shall be equal to 0.

…

## All flags “xxx_info_in_ph_flag” set equal to 1 When PH in SH

In the current VVC, the following flags enable their related tools in the slice header or in the picture header:

- qp_delta_info_in_ph_flag

- rpl_info_in_ph_flag

- dbf_info_in_ph_flag

- sao_info_in_ph_flag

- wp_info_in_ph_flag

- alf_info_in_ph_flag

The current semantics of these flags don’t clearly specify the value when the PH is in the SH even if it is forbidden to signal each flag equal 0 when the SH contains the PH structure.

The current VTM implementation set the value of all these flags equal to 0 when the picture header is in the slice header which is avoided by the current semantics.

When the picture header is in the slice header, each picture of the CLVS contains only one slice. Consequently, this signaling does not provide any flexibility in that case.

To clarify that these flags shall be equal to 1 when the picture header is in the slice header, we propose to add a sentence each flag to explicitly set them equal to 1 when the PH is in the SH.

For example, the semantics of qp_delta_info_in_ph_flag is modified as the following:

…

qp_delta_info_in_ph_flag equal to 1 specifies that QP delta information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. qp_delta_info_in_ph_flag equal to 0 specifies that QP delta information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure. When slice headers referring to the PPS contain the PH syntax structure, it is a requirement of bitstream conformance that qp_delta_info_in_ph_flag shall be equal to 1.

…

All modifications for qp_delta_info_in_ph_flag, rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, wp_info_in_ph_flag and alf_info_in_ph_fla are reported in the attached JVET-Q2001-vE_JVET-R0202.docx file.

## No num_ref_idx_active_override_flag when PH in SH

The usage of the PH in the SH corresponds to some low delay applications and this signaling is very efficient for low bitrate applications. So the overriding of the reference picture list may not be useful in that case where the good strategy is to define a reference picture list set in the SPS. Therefore, it is preferable to avoid this overriding when the PH is in the SH as depicted in the following:

…

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !rpl_info_in_ph_flag && ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) |  |
| ref_pic_lists( ) |  |
| if( !picture_header_in_slice_header_flag) { |  |
| if( ( rpl_info_in_ph_flag ¦ ¦ ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) && ( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |

…

num_ref_idx_active_override_flag equal to 1 specifies that the syntax element num_ref_idx_active_minus1[ 0 ] is present for P and B slices and the syntax element num_ref_idx_active_minus1[ 1 ] is present for B slices. num_ref_idx_active_override_flag equal to 0 specifies that the syntax elements num_ref_idx_active_minus1[ 0 ] and num_ref_idx_active_minus1[ 1 ] are not present. When not present, and when picture_header_in_slice_header_flag is set equal to 0, the value of num_ref_idx_active_override_flag is inferred to be equal to 1. When not present, and when picture_header_in_slice_header_flag is set equal to 1,  it is inferred to be equal to 0.

…

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).