| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0203-r1 |
| --- | --- |

| Title: | AHG9/AHG12: On combination of NAL unit types in a picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email: | dr.hendry@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposed the following:

- When mixed_nalu_types_in_pic_flag is equal to 1, instead of specifying what NAL unit types can be mixed, specify the combination of NAL unit types that cannot be mixed. Consequently, the following applies:

  - There shall be no slice with nal_unit_type equal to GDR_NUT.

  - When one or more slice in the picture is slice with nal_unit_type equals to an IRAP NAL unit type (i.e., either CRA_NUT, IDR_N_LP, or IDR_W_RADL), the nal_unit_type of the rest of slice(s) in the picture shall be TRAIL_NUT.

- Relax the constraint such that picture with mixed NAL unit types shall have exactly two different nal_unit_types values.

In the first revision of this contribution, the following table was added to describe the proposed change for allowed combinations of NAL unit types.

| Combinations |  | Current spec | Proposed |
| --- | --- | --- | --- |
| IRAP Type | Other IRAP Type | No | No |
| IRAP Type | TRAIL_NUT | OK | OK |
| IRAP Type | STSA | No | No |
| IRAP Type | RASL_NUT | OK | No |
| IRAP Type | RADL_NUT | OK | No |
| TRAIL_NUT | STSA_NUT | OK | OK |
| TRAIL_NUT | RASL_NUT | OK | OK |
| TRAIL_NUT | RADL_NUT | OK | OK |
| RASL_NUT | RADL_NUT | OK | OK |
| RASL_NUT | STSA_NUT | OK | OK |
| RADL_NUT | STSA_NUT | OK | OK |
| GDR_NUT | Any other type | No | No |

# Problem description

In the current VVC working draft, when the value of mixed_nalu_types_in_pic_flag is equal to 1, the following constraints apply:

- The picture shall have at least two subpictures but shall have exactly two different nal_unit_type values.

- One of the nal_unit_type shall be one of the types in the list { STSA_NUT, RADL_NUT, RASL_NUT, IDR_W_RADL, IDR_N_LP, CRA_NUT } and the other nal_unit_type shall be one of the types in the list { TRAIL_NUT, RADL_NUT, RASL_NUT }

We asserted that the above constraints have the following problems:

- When nal_unit_type of one or more slices in a picture with mixed_nalu_types_in_pic_flag is equal to 1 is an IRAP types (i.e., CRA_NUT, IDR_N_LP, or IDR_W_RADL), the nal_unit_type of the rest of slice(s) in the picture can be one of leading picture NAL unit types (i.e., RASL_NUT or RADL_NUT). Such combination is only possible when the subpicture containing the slices with IRAP type has different prediction structure or different GOP size from the subpicture containing slice with leading picture NAL unit types. Allowing such combination would make the codec unnecessarily complicated.

- Restricting that there must be exactly two different nal_unit_type values in a picture with mixed_nalu_types_in_pic_flag is unnecessary. In the 17th meeting we have allowed mixing of RASL_NUT and RADL_NUT. There is no additional burden actually to allow the mixed of RASL_NUT, RADL_NUT, and TRAIL_NUT. There is no foreseeable different process for handling whether a picture contains two different NAL unit types or three different NAL unit types or four different NAL unit types.

# Proposal

To resolve the described problems, we propose the following:

- When mixed_nalu_types_in_pic_flag is equal to 1, instead of specifying what NAL unit types can be mixed, specify the combination of NAL unit types that cannot be mixed. Consequently, the following applies:

  - There shall be no slice with nal_unit_type equal to GDR_NUT. Note that this is already specified in the current working draft.

  - When one or more slice in the picture is slice with nal_unit_type equals to an IRAP NAL unit type (i.e., either CRA_NUT, IDR_N_LP, or IDR_W_RADL), the nal_unit_type of the rest of slice(s) in the picture shall be TRAIL_NUT.

- Relax the constraint such that picture with mixed NAL unit types shall have exactly two different nal_unit_types values.

# Proposed spec text changes

...

For VCL NAL units of any particular picture, the following applies:

–	If mixed_nalu_types_in_pic_flag is equal to 0, the value of nal_unit_type shall be the same for all VCL NAL units of a picture, and a picture or a PU is referred to as having the same NAL unit type as the coded slice NAL units of the picture or PU.

–	Otherwise (mixed_nalu_types_in_pic_flag is equal to 1), the picture shall have at least two subpictures and VCL NAL units of the picture shall have exactly two different nal_unit_type values as follows: the VCL NAL units of at least one subpicture of the picture shall all have a particular value of nal_unit_type equal to STSA_NUT, RADL_NUT, RASL_NUT, IDR_W_RADL, IDR_N_LP, or CRA_NUT, while the VCL NAL units of other subpictures in the picture shall all have a different particular value of nal_unit_type equal to TRAIL_NUT, RADL_NUT, or RASL_NUT. [Ed. (YK): Double check 1) whether mixing of STSA_NUT and TRAIL_NUT should be disallowed, and 2) whether mixing of an IRAP NUT and a NUT in the range of RSV_VCL_4..RSV_VCL_6 should be allowed.]

–	Otherwise (mixed_nalu_types_in_pic_flag is equal to 1), all the following applies:

–	The picture shall have at least two subpictures.

–	VCL NAL units of the picture shall have at least two different nal_unit_type values.

–	There shall be no slice in the picture with nal_unit_type_equal to GDR_NUT

–	When one or more slice in the picture has a particular value of nal_unit_type in the range from IDR_W_RADL to RSV_IRAP_12, inclusive, the rest of slices in the the picture shall have nal_unit_type equal to TRAIL_NUT.

...

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).