| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0204 |
| --- | --- |

| Title: | AHG8: On inference of index for PTL/DPB/HRD parameters in the VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takahiro Nishi Kiyofumi Abe Virginie Drugeon | Tel: Email: | +49-6103-766240 nishi.takahiro@jp.panasonic.com abe.kiyo@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This document proposes to avoid an unnecessary signaling of HRD parameters in the VPS, and to introduce into the signaling of PTLs and DPB parameters in the VPS the same skipping method that is used for the signaling of HRD parameters.

# Introduction

In the current draft specification, signalling of ols_hrd_idx[ i ] in the VPS is skipped when the assignment of OLSs to HRD parameters is obvious: If the value of num_ols_hrd_param_minus1 + 1 is equal to TotalNumOlss, the value of ols_hrd_idx[ i ] is inferred to be equal to i. However, for the case that the number of OLSs is equal to the number of HRD parameters, the 0-th HRD parameter will never be used, because the 0-th OLS is by definition always single layer and for single layer OLSs, the HRD parameters are signaled in the SPS instead.

This document proposes to avoid the above unnecessary signaling of HRD parameters, and to introduce the same method to the signaling of PTLs and DPB parameters in the VPS.

Proposed modifications are summarized as follows:

- Change upper limit of num_ols_hrd_params_minus1 from TotalNumOlss – 1 to TotalNumOlss – 2

  - 0-th OLS never refers to any of the HRD parameters in the VPS, so there is no need to send these HRD parameters as the 0-th HRD parameters syntax structure.

- Change condition for omitting signalling of ols_hrd_idx[ i ] from “num_ols_hrd_params_minus1 + 1  !=  TotalNumOlss” to “num_ols_hrd_params_minus1 + 2  !=  TotalNumOlss”

  - OLSs and HRD parameters are assigned one to one except for 0-th OLS.

  - 0-th OLS never refers to any of HRD parameters in the VPS.

- Omit signaling of ols_ptl_idx[ i ] when vps_num_ptls_minus1 + 1  is equal to TotalNumOlss

  - OLSs and PTLs are assigned one to one when there is the same number of OLSs and PTLs, since PTLs parameters are signalled also for single layer OLSs.

- Omit signaling of ols_dpb_params_idx[ i ] when vps_num_dpb_params + 1 is equal to TotalNumOlss

  - OLSs and DPB parameters are assigned one to one except for 0-th OLS.

  - 0-th OLS never refers to any of the DPB parameters in the VPS.

# Proposed changes

Proposed changes over the draft specification are presented below.

7.3.2.2	Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| vps_num_ptls_minus1 | u(8) |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( vps_num_ptls_minus1 + 1 != TotalNumOlss && vps_num_ptls_minus1 > 0 ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_num_dpb_params | ue(v) |
| if( vps_num_dpb_params > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i < vps_num_dpb_params; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| dpb_max_temporal_id[ i ] | u(3) |  |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < TotalNumOlss; i++ ) { |  |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |  |
| ols_dpb_pic_width[ i ] | ue(v) |  |
| ols_dpb_pic_height[ i ] | ue(v) |  |
| if( vps_num_dpb_params + 1 != TotalNumOlss && vps_num_dpb_params > 1 ) |  |  |
| ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| if( !each_layer_is_an_ols_flag ) |  |  |
| vps_general_hrd_params_present_flag | u(1) |  |
| if( vps_general_hrd_params_present_flag ) { |  |  |
| general_hrd_parameters( ) |  |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_cpb_params_present_flag | u(1) |  |
| num_ols_hrd_params_minus1 | ue(v) |  |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| hrd_max_tid[ i ] | u(3) |  |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ] ) |  |  |
| } |  |  |
| if( num_ols_hrd_params_minus1 + 2 + 1 != TotalNumOlss && num_ols_hrd_params_minus1 > 0 ) |  |  |
| for( i = 1; i < TotalNumOlss; i++ ) |  |  |
| if( NumLayersInOls[ i ] > 1 ) |  |  |
| ols_hrd_idx[ i ] | ue(v) |  |
| } |  |  |
| … |  |  |
| } |  |  |

7.4.3.2	Video parameter set RBSP semantics

ols_ptl_idx[ i ] specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS. When present, the value of ols_ptl_idx[ i ] shall be in the range of 0 to vps_num_ptls_minus1, inclusive. When vps_num_ptls_minus1 + 1 is equal to TotalNumOlss, ols_ptl_idx[ i ] is inferred to be equal to i. When vps_num_ptls_minus1 is equal to 0, the value of ols_ptl_idx[ i ] is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structure that applies to the i-th OLS is also present in the SPS referred to by the layer in the i-th OLS. It is a requirement of bitstream conformance that, when NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structures signalled in the VPS and in the SPS for the i-th OLS shall be identical.

ols_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params − 1, inclusive. When ols_dpb_params_idx[ i ] is not present, the value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the dpb_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

When NumLayersInOls[ i ] is greater than 1 and the value of vps_num_dpb_params + 1 is equal to TotalNumOlss, the value of ols_dpb_params_idx[ i ] is inferred to be equal to i − 1. Otherwise, when NumLayersInOls[ i ] is greater than 1 and vps_num_dpb_params is equal to 0, the value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

num_ols_hrd_params_minus1 plus 1 specifies the number of ols_hrd_parameters( ) syntax structures present in the general_hrd_parameters( ) syntax structure when vps_general_hrd_params_present_flag is equal to 1. The value of num_ols_hrd_params_minus1 shall be in the range of 0 to TotalNumOlss − 2 − 1, inclusive.

ols_hrd_idx[ i ] specifies the index, to the list of ols_hrd_parameters( ) syntax structures in the VPS, of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. The value of ols_hrd_idx[[ i ] shall be in the range of 0 to num_ols_hrd_params_minus1, inclusive.

When NumLayersInOls[ i ] is equal to 1, the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

If NumLayersInOls[ i ] is greater than 1 and the value of num_ols_hrd_param_minus1 + 2 + 1 is equal to TotalNumOlss, the value of ols_hrd_idx[ i ] is inferred to be equal to i − 1. Otherwise, when NumLayersInOls[ i ] is greater than 1 and num_ols_hrd_params_minus1 is equal to 0, the value of ols_hrd_idx[[ i ] is inferred to be equal to 0.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).