| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0205 |
| --- | --- |

| Title: | AHG9: On signalling of inter_layer_ref_pics_present_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takahiro Nishi Kiyofumi Abe Virginie Drugeon | Tel: Email: | +49-6103-766240 nishi.takahiro@jp.panasonic.com abe.kiyo@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

Conditional signalling of inter_layer_ref_pics_present_flag depending on sps_video_parameter_set_id is proposed. When sps_video_parameter_set_id is equal to 0 (i.e., the CVS contains only one layer), inter_layer_ref_pics_present_flag is not signalled in the SPS and the value of the flag is inferred to be equal to 0.

# Introduction

In the current draft specification, inter_layer_ref_pics_present_flag is always signalled in the SPS even if sps_video_parameter_set_id is equal to 0 (i.e., the CVS contains only one layer). This document proposes, as option 1, not to signal inter_layer_ref_pics_present_flag when sps_video_parameter_set_id is equal to 0. If it is preferable to keep the current SPS syntax, it is proposed to change the semantics of inter_layer_ref_pics_present_flag as described in the option 2 below.

# Proposed changes

Proposed changes over the draft specification are presented below.

## Option 1: Conditional signalling of inter_layer_ref_pics_present_flag

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_video_parameter_set_id | u(4) |
| … |  |
| if( sps_video_parameter_set_id > 0 ) |  |
| inter_layer_ref_pics_present_flag | u(1) |
| … |  |

7.4.3.3	Sequence parameter set RBSP semantics

inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CLVS. inter_layer_ref_pic_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0.

## Option 2: Restricting the value of inter_layer_ref_pics_present_flag

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_video_parameter_set_id | u(4) |
| … |  |
| inter_layer_ref_pics_present_flag | u(1) |
| … |  |

7.4.3.3	Sequence parameter set RBSP semantics

inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CLVS. inter_layer_ref_pic_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag shall is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).