| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0207-v1 |
| --- | --- |

| Title: | AHG9: General constraint information for LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Mehdi Salehifar Seung-Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com jaehyun.lim@lge.com mehdi.salehifar@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In current VVC WD text, constraint flags are defined in general constraint information syntax table, and they are aligned with sps enabling flags so that each coding tool is controlled to be disabled or enabled. Currently, a constraint flag for LFNST is missing in the table, so that it was suggested to add the flag through JVET bug tracker #760. Specifically, it was commented that an input contribution to a meeting would be required and encouraged in order to get the constraint flag flags and SPS enabling flags aligned. Therefore, it is proposed that no_lfnst_constraint_flag to be added according to the encouraged procedure.

Proposed text

The constraint flag for LFNST as other coding tools could be incorporated into general_constraint_infro (Table. 1) found in [1]. Also, its semantics could be located appropriately as in Table. 2.

Table 1  The addition of no_lfnst_constraint_flag into general_constraint_info syntax table

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| …… | u(1) |
| no_mts_constraint_flag | u(1) |
| no_sbt_constraint_flag | u(1) |
| no_lfnst_constraint_flag | u(1) |
| …… | u(1) |
| } |  |

Table 2    The semantics of no_lfnst_constraint_flag

7.4.4.2 General constraint information semantics …… no_mts_constraint_flag equal to 1 specifies that sps_mts_enabled_flag shall be equal to 0. no_mts_constraint_flag equal to 0 does not impose such a constraint. no_sbt_constraint_flag equal to 1 specifies that sps_sbt_enabled_flag shall be equal to 0. no_sbt_constraint_flag equal to 0 does not impose such a constraint. no_lfnst_constraint_flag equal to 1 specifies that sps_lfnst_enabled_flag shall be equal to 0. no_lfnst_constraint_flag equal to 0 does not impose such a constraint. ……

The existing SPS enabling flag for LFNST, sps_lfnst_enabled_flag, properly controls signalling of an LFNST index (lfnst_idx) as shown in Table 3. When sps_lfnst_enabled_flag is set as 0 and then lfnst_idx is not present, lfnst_idx is inferred as 0, which prevents LFNST from being applied (section 8.7.4.1 in [1]). Therefore, the proposed text, as shown in Table. 1 and Table. 2, is aligned with the existing spec for LFNST.

Table 3  The LFNST index (lfnst_idx) signaling allowed only if sps_lfnst_enabled_flag is equal to 1

coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) {

……

if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) {

if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 )

lfnst_idx

}

……

}

Conclusion

It is recommended that the proposed text and SW of the constraint flag for LFNST shown in Table. 1 and Table. 2 should be integrated into next VVC WD.

References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, 17th meeting, Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).