| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0208-v1 |
| --- | --- |

| Title: | AHG16: Rounding correction for ALF virtual boundary processing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Semih Esenlik Biao Wang Han Gao Elena Alshina | Tel: Email: | anand.meher.kotra@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

In VTM-8.0 [1], to avoid extreme padding for the sample rows which are immediately adjacent to the adaptive loop filter (ALF) virtual boundary, the correction value applied during the filtering is quantized by a larger value 1024 (210) instead of 128(27). However the rounding value used during the filtering is still 64. The current proposal proposes a fix by changing the rounding value to 512 when the quantization value used in the ALF filtering is 1024.

The objective results, over VTM8.0 Anchor for CTC configuration are as follows:

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | 0.00% | 0.01% | 0.01% | 100% | 99% |
| RA | 0.01% | -0.01% | 0.00% | 100% | 100% |
| LDB | 0.01% | 0.02% | -0.07% | 100% | 102% |
| LDP | 0.01% | -0.25% | -0.14% | 100% | 103% |

# Introduction

In VTM-8.0 [1], to avoid extreme padding for the sample rows which are immediately adjacent to the ALF virtual boundary, the filtering strength is reduced by increasing the quantization of the correction value used in filtering.

Table 45 and Table 46 in the VVC draft [1] show the quantization values used for Luma (alfShiftY) and Chroma ALF filtering (alfShiftC).

Table 45 – Specification of y1, y2, y3 and alfShiftY according to the vertical luma sample position y and applyAlfLineBufBoundary

| Condition | alfShiftY | y1 | y2 | y3 |
| --- | --- | --- | --- | --- |
| ( y = = CtbSizeY − 5 ¦ ¦ y = = CtbSizeY − 4 ) && ( applyAlfLineBufBoundary = = 1 ) | 10 | 0 | 0 | 0 |
| ( y = = CtbSizeY − 6 ¦ ¦ y = = CtbSizeY − 3 ) && ( applyAlfLineBufBoundary = = 1 ) | 7 | 1 | 1 | 1 |
| ( y = = CtbSizeY − 7 ¦ ¦ y = = CtbSizeY − 2 ) && ( applyAlfLineBufBoundary = = 1 ) | 7 | 1 | 2 | 2 |
| otherwise | 7 | 1 | 2 | 3 |

Table 46 – Specification of y1, y2 and alfShiftC according to the vertical chroma sample position y and applyAlfLineBufBoundary

| Condition | alfShiftC | y1 | y2 |
| --- | --- | --- | --- |
| ( y = = ctbHeightC − 2 ¦ ¦ y = = ctbHeightC − 3 ) && ( applyAlfLineBufBoundary = = 1 ) | 10 | 0 | 0 |
| ( y = = ctbHeightC − 1 ¦ ¦ y = = ctbHeightC − 4 ) && ( applyAlfLineBufBoundary = = 1 ) | 7 | 1 | 1 |
| Otherwise | 7 | 1 | 2 |

The correction value is derived as in equations (1467) and (1508) in VVC draft [2] (also shown below):

sum = curr + ( ( sum + 64 )  >>  alfShiftY  ) (1467) for Luma and

sum = curr + ( ( sum + 64 )  >>  alfShiftC ) (1508) for Chroma.

The rounding value using during quantization by a value of 2n  is normally 2n-1. Therefore the rouning value of 64 is asserted to be correct when the quanization values for alfShiftY and alfShiftC are equal to 7. However the same rounding value of 64 may reportedly result in incorrect rounding precision when alfShiftY and alfShiftC are equal to 10.

# Proposed method

The current contribution proposes a fix to the rounding value used during ALF filtering by simply changing the filtering equations (1467) and (1508) as follows:

sum = curr + ( ( sum + 2(alfShiftY-1) )  >>  alfShiftY  ) for Luma and

sum = curr + ( ( sum + 2(alfShiftC-1) )  >>  alfShiftC  ) for Chroma

# Simulation Results

The proposed rounding correction is implemented on top of VTM-8.0 and the simulation is performed under common test conditions as described in JVET-N1010 [2]. The results are as follows:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.02% | 100% | 99% |
| Class A2 | 0.00% | 0.01% | 0.01% | 100% | 99% |
| Class B | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.01% | 0.01% | 100% | 99% |
| Class E | 0.00% | 0.01% | 0.02% | 99% | 99% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.01% | 0.01% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.14% | -0.06% | 100% | 100% |
| Class A2 | 0.00% | 0.05% | 0.13% | 100% | 100% |
| Class B | 0.02% | -0.02% | -0.02% | 100% | 100% |
| Class C | 0.01% | 0.05% | -0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.06% | -0.12% | 100% | 99% |
| Class F | 0.00% | -0.04% | -0.04% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.15% | 0.12% | 101% | 103% |
| Class C | 0.00% | -0.21% | -0.15% | 100% | 100% |
| Class E | 0.06% | 0.11% | -0.29% | 100% | 102% |
| Overall | 0.01% | 0.02% | -0.07% | 100% | 102% |
| Class D | 0.00% | -0.68% | -0.45% | 99% | 100% |
| Class F | -0.03% | 0.23% | -0.18% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.05% | 0.16% | 100% | 104% |
| Class C | 0.01% | -0.14% | -0.33% | 100% | 103% |
| Class E | 0.01% | -0.72% | -0.37% | 100% | 102% |
| Overall | 0.01% | -0.25% | -0.14% | 100% | 103% |
| Class D | -0.03% | 0.10% | -0.51% | 99% | 100% |
| Class F | 0.01% | -0.11% | -0.65% | 100% | 102% |

# Conclusion

The current contribution proposes a rounding correction for the ALF virtual boundary processing. The objective results show that there is no significant change in the BD-Rate. It is suggested to adopt the proposed rounding correction.

# References

- Versatile Video Coding (Draft 8), Document JVET-Q2001-vE, B. Bross, J. Chen, S. Liu, Y.K. Wang

- “JVET common test conditions and software reference configurations”, JVET-N1010

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).