| 3Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0211 |
| --- | --- |

| Title: | AHG12: Cleanups on rectangular slices signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bae-Keun Lee | Tel: Email: | +82-31-620-1200 baekeun.lee@xris.co.kr |
| Source: | Xris corporation |  |  |

_____________________________

- Abstract

This contribution proposes to some cleanup changes on rectangular slice signalling in the PPS.

- Proposal 1: Infer the value of tile_idx_delta_present_flag if num_slices_in_pic_minus1 is less than or equal to 1.

- Proposal 2: Infer the value of tile_idx_delta_present_flag if NumTileColumns is euqal to 1 or NumTileRows is equal to 1.

- Proposal 3:

  - Infer the value of slice_width_in_tiles_minus1[i] in the i-th slice if an i-th slice is located in the last tile column.

  - Infer the value of slice_height_in_tiles_minus1[i] in the slice if an i-th slice located in the last tile row.

- Proposal 4:

  - Infer the value of exp_slice_height_in_ctus_minus1[ i ][ j ] if the tile row height of i-th slice is less than or equal to 2,

  - It is proposed that the value of exp_slice_height_in_ctus_minus1[ i ][ j ] is in the range of 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] -2

- Proposal

  - Proposal 1

If a picture has only two rectangular slices, the picture is partitioned into rectangular slice rows or rectangular slice columns in a raster scan order.

tileIdx could be derived from the variables SliceWidthInTiles[i], sliceHeightInTiles[ i ] and NumTileColumns without syntax element tile_idx_delta[i] when a picture has only two rectangular slices.

It is proposed to infer the value of tile_idx_delta_present_flag if num_slices_in_pic_minus1 is less than or equal to 1.

When tile_idx_delta_present_flag is not present, it is inferred to be equal to 0.

The proposed syntax is as follows.

Table 1

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !no_pic_partition_flag ) { |  |
| … |  |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0 ) |  |
| if( num_slices_in_pic_minus1 > 1 ) |  |
| tile_idx_delta_present_flag | u(1) |
| .. |  |
| } |  |

  - Proposal 2

If NumTileRows is equal to 1, then a picture is partitioned into rectangular slices columns in a raster scan order, and there is no need to signal tile_idx_delta_present_flag if NumTileRows is equal to 1.

It is proposed to infer the value of tile_idx_delta_present_flag if NumTileRows is equal to 1, and when tile_idx_delta_present_flag is not present, it is inferred to be equal to 0.

Similarly, if NumTileColumns is equal 1, then a picture is partitioned into rectangular slices rows in a raster scan order, and there is no need to signal tile_idx_delta_present_flag if NumTileColumns is equal to 1.

It is proposed to infer the value of tile_idx_delta_present_flag if NumTileColumns is equal to 1, and when tile_idx_delta_present_flag is not present, it is inferred to be equal to 0.

The proposed syntax is as follows.

Table 2

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !no_pic_partition_flag ) { |  |
| … |  |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0 ) |  |
| if( num_slices_in_pic_minus1 > 0&&¦ NumTileColumns > 1 && NumTileRows > 1) |  |
| tile_idx_delta_present_flag | u(1) |
| .. |  |
| } |  |

  - Proposal 3

If an i-th slice is located in the last tile column, slice_width_in_tiles_minus1[i] should be equal to 0, since there is no more tile column on the right-hand side of it. Therefore, it is proposed to infer the value of slice_width_in_tiles_minus1[i] if an i-th slice is located in the last tile column, and when slice_width_in_tiles_minus1[i] is not present, the value of slice_width_in_tiles_minus1[i] is inferred to be equal to 0.

Similarly, if an i-th slice is located in the last tile row, slice_height_in_tiles_minus1[i] should be equal to 0, since there is no more tile row on the bottom side of it. Therefore, it is proposed to infer the value of slice_height_in_tiles_minus1[i] if an i-th slice is located in the last tile row, and when slice_height_in_tiles_minus1[i] is not present, the value of slice_height_in_tiles_minus1[i] is inferred to be equal to 0.

The proposed syntax is as follows.

Table 3

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0 ) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| if( NumTileColumns > 1 && (SliceTopLeftTileIdx[i] % NumTileColumns) != (NumTileColumns -1) ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[i] % NumTileColumns = = 0 ¦ ¦ (SliceTopLeftTileIdx[i] / NumTileColumns != (NumTileRows-1) ) ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| numExpSlicesInTile = num_exp_slices_in_tile[ i ] |  |
| for( j = 0; j < numExpSlicesInTile; j++ ) |  |
| exp_slice_height_in_ctus_minus1[ j ] | ue(v) |
| i += NumSlicesInTile[ i ] |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |

slice_width_in_tiles_minus1[ i ] plus 1 specifies the width of the i-th slice in units of tile columns. The value of slice_width_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileColumns − 1, inclusive.

When i is less than num_slices_in_pic_minus1 and NumTileColumns is equal to 1, or SliceTopLeftTileIdx[i] % NumTileColumns is euqal to NumTileColumns-1, the value of slice_width_in_tiles_minus1[ i ] is inferred to be equal to 0.

slice_height_in_tiles_minus1[ i ] plus 1 specifies the height of the i-th slice in units of tile rows when num_exp_slices_in_tile[ i ] is equal to 0. The value of slice_height_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileRows − 1, inclusive.

If SliceTopLeftTileIdx[i] / NumTileColumns is equal to NumTileRows-1, it is inferred to be equal to 0.

Otherwise, if i is less than num_slices_in_pic_minus1 and slice_height_in_tiles_minus1[ i ] is not present, it is inferred to be equal to NumTileRows  = =  1 ? 0 : slice_height_in_tiles_minus1[ i − 1 ].

  - Proposal 4

If a tile row height of i-th slice is equal to 2 and a tile of i-th slices is composed of multiple slices, then the height of i-th slice should be equal to 1. Therefore, it is proposed to infer the value of exp_slice_height_in_ctus_minus1[ i ][ j ] if the tile row height of i-th slice is less than or equal to 2, and when exp_slice_height_in_ctus_minus1[ i ][ j ] is not present, it is inferred to be equal to 0.

Also, it is proposed to add constraint on the range of exp_slice_height_in_ctus_minus1[ i ][ j ]. In the WD [1], the value of exp_slice_height_in_ctus_minus1[ i ][ j ] is in the ragne of 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] -1.

If value of exp_slice_height_in_ctus_minus1[ i ][ j ] is equal to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] -1, that implies single slice tile, but exp_slice_height_in_ctus_minus1[ i ][ j ] is only signalled in case of multiple slice tile.

The proposed constraint is as follows

- The value of exp_slice_height_in_ctus_minus1[ i ][ j ] is in the range of 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] -2.

The proposed syntax is as follows.

Table 4

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 )) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| numExpSlicesInTile = num_exp_slices_in_tile[ i ] |  |
| if (RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 2) |  |
| for( j = 0; j < numExpSlicesInTile; j++ ) |  |
| exp_slice_height_in_ctus_minus1[ j ] | ue(v) |
| i += NumSlicesInTile[ i ] |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| … |  |
| } |  |

exp_slice_height_in_ctus_minus1[ i ][ j ] plus 1 specifies the height of the j-th rectangular slice in the tile containing the i-th slice in units of CTU rows. The value of exp_slice_height_in_ctus_minus1[ i ][ j ] shall be in the range of 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 12, inclusive. When not present, the value of exp_slice_height_in_ctus_minus1[ i ][ j ] is inferred to be equal to 0, inclusive.

References

[1] JVET-Q1001-VE, Versatile Video Coding (Draft8), Brussels, Jan 2020.

Patent rights declaration(s)

Xris corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).