| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0212 |
| --- | --- |

| Title: | On modes in geometric partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Du Liu, Ruoyang Yu, Jacob Ström Torshamsgatan 23 16480 Stockholm Sweden | Email: | {christopher.hollmann, du.liu, ruoyang.yu, jacob.strom} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this contribution three methods to reduce the number of modes for geometric partitioning are presented. These methods are claimed to reduce the number of modes from 64 to 50, 38 and 32, respectively. It is further claimed that these methods have a minor impact on the compression efficiency. It is also asserted that the number of combinations that are required to be tested during verification testing is reduced by up to 50%.

- Method 1 (50 modes): 0.03% RA, 0.00% LDB, 700 combinations to test (-22%)

- Method 2 (38 modes): 0.05% RA, 0.01% LDB, 532 combinations to test (-40%)

- Method 3 (32 modes): 0.07% RA, 0.06% LDB, 448 combinations to test (-50%)

# Introduction

At the 17th JVET meeting, Geometric Partitioning (GPM) was adopted to the draft of the VVC specification. The adopted version uses a total of 20 different angles with between two and four different distances, resulting in 64 total modes [1]. As GPM is allowed for 14 CU sizes, there is a total of 896 combinations that need to be tested during verification testing.

In this contribution we show that the number of combinations can be reduced substantially with only a minor loss in compression efficiency.

# Tests

Figure 1. Modes used in GPM

The original proposal for GPM used 32 different angles, as shown in Figure 1. As of the 17th JVET meeting in Brussels, Belgium, in January 2020, the following restrictions for angles exist:

- Angles 1, 6, 7, 9, 10, 15, 17, 22, 23, 25, 26 and 31 are not used.

- All angles larger than 15 have no distance 0.

- Angles 0, 8, 16 and 24 only use distances 1 and 3.

All tests were run according to CTC [2]. As GPM is only used for inter-predicted blocks, test results for the All Intra configuration are omitted, as they are not changed compared to VTM-8.0.

## Method 1: 50 modes

In the currently adopted variant of GPM the two adjacent angles on each side to the vertical angles 8 and 24 are disallowed, whereas only one adjacent angle on each side to the horizontal angles 0 and 16 is disallowed. In this method, we extend the rule for vertical angles to the horizontal angles, disallowing angles 2, 14, 18 and 30. This reduces the number of modes to 50, as no other changes are made. The impact on the compression efficiency is shown in Table 1.

As there are now 50 modes for each of the allowed CUs, the total number of combinations is reduced to 700, a reduction of around 22%.

Table 1. CTC results for method 1 with 50 modes

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | -0.01% | 100% | 100% |
| Class A2 | 0.04% | 0.08% | 0.18% | 100% | 99% |
| Class B | 0.02% | -0.05% | 0.00% | 99% | 100% |
| Class C | 0.06% | 0.21% | 0.13% | 97% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.04% | 0.07% | 99% | 99% |
| Class D | 0.02% | 0.13% | 0.02% | 97% | 100% |
| Class F | 0.02% | 0.02% | -0.03% | 98% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.23% | -0.04% | 98% | 100% |
| Class C | -0.03% | -0.08% | 0.02% | 99% | 97% |
| Class E | 0.05% | 0.36% | -0.18% | 98% | 101% |
| Overall | 0.00% | 0.16% | -0.05% | 98% | 99% |
| Class D | -0.03% | -0.47% | 0.03% | 96% | 99% |
| Class F | 0.06% | 0.63% | -0.15% | 99% | 102% |

## Method 2: 38 modes

This second method extends the previous method with 50 modes. In addition to the newly disallowed angles, this method does not use distance 2 anymore. This way, an additional 12 modes are removed, bringing the total to 38. The impact on the compression efficiency is shown in Table 2.

As there are now 38 modes for each of the allowed CUs, the total number of combinations is reduced to 532, a reduction of around 40%.

Table 2. CTC results for method 2 with 38 modes

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | -0.03% | 99% | 100% |
| Class A2 | 0.05% | 0.08% | 0.18% | 99% | 99% |
| Class B | 0.03% | -0.07% | -0.02% | 98% | 100% |
| Class C | 0.10% | 0.20% | 0.09% | 96% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.05% | 0.05% | 98% | 100% |
| Class D | 0.04% | 0.14% | -0.05% | 95% | 100% |
| Class F | 0.02% | 0.05% | -0.05% | 96% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.02% | -0.04% | 98% | 99% |
| Class C | -0.01% | -0.10% | 0.05% | 98% | 100% |
| Class E | 0.06% | 0.22% | 0.15% | 97% | 102% |
| Overall | 0.01% | 0.03% | 0.04% | 98% | 100% |
| Class D | -0.02% | -0.34% | 0.02% | 95% | 101% |
| Class F | -0.06% | 0.54% | 0.44% | 96% | 99% |

## Method 3: 32 modes

The third method extends the previous method with 38 modes. In addition to not using distance 2 for any angles anymore, this method does not use distance 0 for any angles. To summarize, this method only uses distances 1 and 3 for all remaining angles. The impact on the compression efficiency is shown in Table 3.

As there are now 32 modes for each of the allowed CUs, the total number of combinations is reduced to 448, a reduction of 50%.

Table 3. CTC results for method 3 with 32 modes

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.04% | 0.06% | 107% | 100% |
| Class A2 | 0.06% | 0.02% | 0.19% | 106% | 102% |
| Class B | 0.05% | 0.07% | 0.05% | 109% | 102% |
| Class C | 0.11% | 0.30% | 0.25% | 106% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.11% | 0.13% | 107% | 101% |
| Class D | 0.02% | 0.23% | 0.02% | 99% | 101% |
| Class F | 0.03% | 0.06% | 0.00% | 103% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.21% | -0.03% | 106% | 101% |
| Class C | 0.02% | -0.04% | 0.08% | 106% | 100% |
| Class E | 0.18% | 0.73% | -0.38% | 107% | 102% |
| Overall | 0.06% | 0.26% | -0.08% | 106% | 101% |
| Class D | 0.03% | -0.52% | -0.01% | 107% | 100% |
| Class F | 0.00% | 0.25% | -0.24% | 109% | 102% |

## Specification changes

The changes to the specification are described in the attached documents

- JVET-R0212_method1_spec_text_vs_JVET-Q2001-vE.docx

- JVET-R0212_method2_spec_text_vs_JVET-Q2001-vE.docx

- JVET-R0212_method3_spec_text_vs_JVET-Q2001-vE.docx

but they are also summarized here:

### Method 1 (50 modes)

Table 36 is changed to

Table 36 – Specification of angleIdx and distanceIdx based on merge_gpm_partition_idx.

| merge_gpm_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 0 | 0 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 8 | 8 |
| distanceIdx | 1 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 1 | 3 |
| merge_gpm_partition_idx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| angleIdx | 11 | 11 | 11 | 11 | 12 | 12 | 12 | 12 | 13 | 13 | 13 | 13 | 16 | 16 | 19 | 19 |
| distanceIdx | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 1 | 3 | 1 | 2 |
| merge_gpm_partition_idx | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| angleIdx | 19 | 20 | 20 | 20 | 21 | 21 | 21 | 24 | 24 | 27 | 27 | 27 | 28 | 28 | 28 | 29 |
| distanceIdx | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 |
| merge_gpm_partition_idx | 48 | 49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| angleIdx | 29 | 29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| distanceIdx | 2 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

And in Table 126 syntax element merge_gpm_partition_idx[ ][ ] is changed:

| merge_gpm_partition_idx[ ][ ] | FLTB | cMax = 6349 |
| --- | --- | --- |

### Method 2 (38 modes)

Table 36 is changed to

Table 36 – Specification of angleIdx and distanceIdx based on merge_gpm_partition_idx.

| merge_gpm_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 0 | 0 | 3 | 3 | 3 | 4 | 4 | 4 | 5 | 5 | 5 | 8 | 8 | 11 | 11 | 11 |
| distanceIdx | 1 | 3 | 0 | 1 | 3 | 0 | 1 | 3 | 0 | 1 | 3 | 1 | 3 | 0 | 1 | 3 |
| merge_gpm_partition_idx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| angleIdx | 12 | 12 | 12 | 13 | 13 | 13 | 16 | 16 | 19 | 19 | 20 | 20 | 21 | 21 | 24 | 24 |
| distanceIdx | 0 | 1 | 3 | 0 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 |
| merge_gpm_partition_idx | 32 | 33 | 34 | 35 | 36 | 37 |  |  |  |  |  |  |  |  |  |  |
| angleIdx | 27 | 27 | 28 | 28 | 29 | 29 |  |  |  |  |  |  |  |  |  |  |
| distanceIdx | 1 | 3 | 1 | 3 | 1 | 3 |  |  |  |  |  |  |  |  |  |  |

And in Table 126 syntax element merge_gpm_partition_idx[ ][ ] is changed:

| merge_gpm_partition_idx[ ][ ] | FLTB | cMax = 6337 |
| --- | --- | --- |

### Method 3 (32 modes)

Table 36 is changed to

Table 36 – Specification of angleIdx and distanceIdx based on merge_gpm_partition_idx.

| merge_gpm_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 0 | 0 | 3 | 3 | 4 | 4 | 5 | 5 | 8 | 8 | 11 | 11 | 12 | 12 | 13 | 13 |
| distanceIdx | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 |
| merge_gpm_partition_idx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| angleIdx | 16 | 16 | 19 | 19 | 20 | 20 | 21 | 21 | 24 | 24 | 27 | 27 | 28 | 28 | 29 | 29 |
| distanceIdx | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 |

And in Table 126 syntax element merge_gpm_partition_idx[ ][ ] is changed:

| merge_gpm_partition_idx[ ][ ] | FL | cMax = 6331 |
| --- | --- | --- |

## Summary

In Table 4 a summary of the different methods regarding compression efficiency and combinations for verification testing. Results for VTM-8.0 without GPM are taken from the test conducted by AhG 13.

Table 4. Summary of different methods

| ​ | GPM Modes​ | BDR RA​ | BDR LD​ | Combinations​ |
| --- | --- | --- | --- | --- |
| VTM-8.0​ | 64​ | 0.00%​ | 0.00%​ | 896​ |
| VTM-8.0 w/o GPM​ | 0​ | 0.66%​ | 1.50%​ | 0 (-100%) |
| Method 1​ | 50​ | 0.03%​ | 0.00%​ | 700 (-22%) |
| Method 2​ | 38​ | 0.05%​ | 0.01%​ | 532 (-40%) |
| Method 3​ | 32​ | 0.07%​ | 0.06%​ | 448 (-50%) |

# Conclusion

In this contribution we showed three different possibilities to reduce the number of modes for GPM without having a significant impact on compression efficiency. The number of combinations required for verification testing can be reduced by up to 50% by the proposed methods. We propose to adopt the method that best fits the desired trade-off between compression efficiency and verification test effort.

# References

- H. Gao et al. Integrated Text for GEO. JVET-Q0806, Brussels, Belgium, January 2020

- F. Bossen et al. JVET common test conditions and software reference configurations for SDR video. JVET-N1010, Geneva, Switzerland, March 2019

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).