| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0214 |
| --- | --- |

| Title: | AHG9: MMVD syntax modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Martin Pettersson Rickard Sjöberg Mitra Damghanian Zhi Zhang Jack Enhorn | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes the following modifications to the syntax related to merge mode with motion vector difference (MMVD) in the VVC draft specification version 8:

- Move the location of sps_fpel_mmvd_enabled_flag to directly follow sps_mmvd_enabled_flag. It is claimed that this modification makes the specification text cleaner as it groups together the MMVD related syntax elements in the SPS.

- Infer the value of sps_fpel_mmvd_enabled_flag to be equal to 0 when the flag is not present. It is reported that the current specification text does not specify the value of sps_fpel_mmvd_enabled_flag when the flag is not present. It is asserted that the current specification text is broken since an unspecified value of sps_fpel_mmvd_enabled_flag would make the presence of ph_fpel_mmvd_enabled_flag in the picture header undetermined.

- Add a no_mmvd_constraint_flag in general_constraint_info(). When this flag is equal to 1, sps_mmvd_enabled_flag shall be equal to 0. It is claimed that MMVD is a relative substantial inter coding feature. A constraint flag that allows indicating that such a feature is not used in the bitstream is claimed to be desirable to have.

# Proposal

In the current VVC draft specification, version 8, the sps_mmvd_enabled_flag specifies whether merge mode with motion vector difference (MMVD) is enabled or not. There is another flag, sps_fpel_mmvd_enabled_flag, which is conditioned on the MMVD enabling flag and it specifies whether the MMVD uses integer or fractional sample precision for the motion vector difference.

This contribution proposes the following three modifications to the syntax related to merge mode with motion vector difference (MMVD):

- Move the location of sps_fpel_mmvd_enabled_flag to directly follow sps_mmvd_enabled_flag. It is claimed this modification makes the specification text cleaner as it groups together the MMVD related syntax elements in the SPS.

- Infer the value of sps_fpel_mmvd_enabled_flag to be equal to 0 when the flag is not present. It is reported that currently the value of sps_fpel_mmvd_enabled_flag is not specified when the flag not present. Furthermore, the presence of ph_fpel_mmvd_enabled_flag in the picture header is only conditioned on the value of sps_fpel_mmvd_enabled_flag. Without the correct value of sps_fpel_mmvd_enabled_flag, the presence of ph_fpel_mmvd_enabled_flag would be undertermined.

- Add a no_mmvd_constraint_flag in general_constraint_info(). When this flag is equal to 1, sps_mmvd_enabled_flag shall be equal to 0. It is claimed that the MMVD is a relative substantial inter coding feature. It would be desirable to have a constraint flag to allow indication that such a feature is not used in the bitstream.

The three proposed modifications on top of the VVC draft (JVET-Q2001-vE) are shown below with added text marked in yellow and removed text in red strike-through.

## Move the location of sps_fpel_mmvd_enabled_flag to directly follow sps_mmvd_enabled_flag.

7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_mmvd_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| sps_isp_enabled_flag | u(1) |
| … |  |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| … |  |
| } |  |

## Infer the value of sps_fpel_mmvd_enabled_flag to be equal to 0 when the flag is not present.

7.4.3.3 Sequence parameter set RBSP semantics

sps_fpel_mmvd_enabled_flag equal to 1 specifies that merge mode with motion vector difference is using integer sample precision. sps_fpel_mmvd_enabled_flag equal to 0 specifies that merge mode with motion vector difference can use fractional sample precision. When not present, the value of sps_fpel_mmvd_enabled_flag is inferred to be equal to 0.

## Add a no_mmvd_constraint_flag in general_constraint_info().

7.3.3.2	General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| ... |  |
| no_mmvd_constraint_flag | u(1) |
| no_fpel_mmvd_constraint_flag | u(1) |
| ... |  |
| } |  |

7.4.4.2	General constraint information semantics

no_mmvd_constraint_flag equal to 1 specifies that sps_mmvd_enabled_flag shall be equal to 0. no_mmvd_constraint_flag equal to 0 does not impose such a constraint.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).