| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0216 |
| --- | --- |

| Title: | AHG9: Signalling the parallel merge level relative to the minimum coding block size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Martin Pettersson Rickard Sjöberg Mitra Damghanian Zhi Zhang Jack Enhorn Jacob Ström | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In the current VVC draft specification, the minimum luma coding block size is specified in the SPS. The allowed range for the minimum luma coding block size is from 4 to Min( 64, CTU size ). In the Brussels JVET meeting, a parallel merge mechanism was adopted in VVC. The mechanism allows parallel derivation of merge candidate lists for blocks within a certain size of a region, which is also referred to as the parallel merge level. The parallel merge level is signaled in the SPS and it indicates the size of the region wherein parallel merge is possible. The allowed range for the parallel merge level is from 4 to the CTU size.

It is reported in this contribution that the current specification allows signalling of the parallel merge level to be smaller than the minimum coding block size, which is asserted to be confusing and unnecessary. It is therefore proposed to signal the parallel merge level relative to the minimum coding block size. It is claimed that the proposed signalling cleans up the signalling of the parallel merge level and brings some minor bit-savings.

# Introduction

In the current VVC draft specification, the minimum coding block size is signalled in the SPS, as shown below.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| … |  |
| } |  |

The semantics is specified as:

log2_min_luma_coding_block_size_minus2 plus 2 specifies the minimum luma coding block size. The value range of log2_min_luma_coding_block_size_minus2 shall be in the range of 0 to Min( 4, sps_log2_ctu_size_minus5 + 3 ), inclusive.

The variables MinCbLog2SizeY, MinCbSizeY, IbcBufWidthY, IbcBufWidthC and Vsize are derived as follows:

MinCbLog2SizeY = log2_min_luma_coding_block_size_minus2 + 2

…

At the Brussels JVET meeting, a parallel merge mechanism was adopted. The mechanism allows parallel derivation of merge candidate lists for blocks within a certain size of a region, which is also referred to as the parallel merge level. The parallel merge level is currently signaled in the SPS, as shown below.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| … |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| … |  |
| } |  |

The corresponding semantics is:

log2_parallel_merge_level_minus2 plus 2 specifies the value of the variable Log2ParMrgLevel, which is used in the derivation process for spatial merging candidates as specified in clause 8.5.2.3, the derivation process for motion vectors and reference indices in subblock merge mode as specified in clause 8.5.5.2, and to control the invocation of the updating process for the history-based motion vector predictor list in clause 8.5.2.1. The value of log2_parallel_merge_level_minus2 shall be in the range of 0 to CtbLog2SizeY − 2, inclusive. The variable Log2ParMrgLevel is derived as follows:

Log2ParMrgLevel = log2_parallel_merge_level_minus2 + 2

As can be seen from above, the current specification allows the signalling of parallel merge level to be smaller than the minimum coding block size. As an example, the parallel merge level can be signaled as 8x8 or 4x4 while the minimum coding block size can be 16x16. It is asserted such allowed signalling is unnecessary and confusing and should be fixed.

# Proposal

It is proposed to signal the parallel merge level relative to the minimum coding block size. The proposed modifications on top of the VVC draft (JVET-Q2001-vE) are shown below with added text marked in yellow and removed text in red strike-through.

7.4.3.3 Sequence parameter set RBSP semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| … |  |
| log2_parallel_merge_level_minus2_min_cb_log2_size_y | ue(v) |
| … |  |
| } |  |

7.4.3.3 Sequence parameter set RBSP semantics

log2_parallel_merge_level_minus2_min_cb_log2_size_y plus 2MinCbLog2SizeY specifies the value of the variable Log2ParMrgLevel, which is used in the derivation process for spatial merging candidates as specified in clause 8.5.2.3, the derivation process for motion vectors and reference indices in subblock merge mode as specified in clause 8.5.5.2, and to control the invocation of the updating process for the history-based motion vector predictor list in clause 8.5.2.1. The value of log2_parallel_merge_level_minus2_min_cb_log2_size_y shall be in the range of 0 to CtbLog2SizeY − 2MinCbLog2SizeY, inclusive. The variable Log2ParMrgLevel is derived as follows:

Log2ParMrgLevel = log2_parallel_merge_level_minus2_min_cb_log2_size_y + 2MinCbLog2SizeY

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).