| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0217 |
| --- | --- |

| Title: | AHG8: On signalling PH RPR scaling window offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Fangjun Pu, Peng Yin, Sean McCarthy, Walt Husak, Tao Chen (Dolby) Jill Boyce (Intel) Jay N. Shingala (Ittiam) | Email: | {pyin, tlu}@dolby.com jill.boyce@intel.com jay.shingala@ittiam.com |
| Source: | Dolby, Intel, Ittiam |  |  |

_____________________________

# Abstract

This contribution proposes that alternative RPR scaling window offsets be signalled in the picture header along with an indication of whether the alternative scaling window is to be applied to the reference or current picture. If present, the alternative scaling window is used instead of the scaling window signalled in the PPS. The alternative scaling window enables a sub-region within a picture – rather than the entire picture – to be resampled to create a reference picture. Similarly, the alternative scaling window enables a sub-region of the current picture to be predicted from a reference picture. No change in the current RPR design and constraints are required.

# Proposal

The use of RPR for sub-regions (e.g. ROI) of reference and current pictures was proposed in JVET-P0336 [1] and called region-of-interest (ROI) RPR. Further simplifications were proposed in JVET-Q0199 [2]. This contribution proposes a simplified method of signalling scaling window offsets for sub-regions of either the reference or current picture, as illustrated in Figure 1.

Specifically, in the picture header, indicate if alternative scaling window offsets are present. If present, indicate that the scaling window offsets are to be applied to the reference picture(s) (Case 1 in Figure 1) or to the current picture (Case 2 in Figure 1). (Table 1 lists the proposed syntax changes over VVC Draft 8 specification [3].) The PH level scaling window offsets, when present, override the respective PPS level scaling window offsets while computing scaling ratios (RefPicScale[]) and the RPR top and left offsets for fractional sample interpolation process.

For each pair of reference and current picture, determine if the PPS IDs are the same or different. If different, apply the scaling window offsets to the indicated picture.

The scaling window indicated in the PH is constrained to be the same size or smaller than the scaling window indicated in the PPS.

Table 1 PH scaling window specification in picture header

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( ph_inter_slice_allowed_flag ) { |  |
| …… |  |
| if( res_change_in_clvs_allowed_flag ) { |  |
| ph_scaling_window_present_idc /*0: not present; 1:present in ref; 2: present in current*/ | u(2) |
| if( ph_scaling_window_present_idc ) { |  |
| ph_scaling_win_offset_prec_shift | ue(v) |
| ph_scaling_win_left_offset | ue(v) |
| ph_scaling_win_right_offset | ue(v) |
| ph_scaling_win_top_offset | ue(v) |
| ph_scaling_win_bottom_offset | ue(v) |
| } |  |
| } |  |
| } |  |
| …… |  |
| } |  |

# Simulation

The proposal is implemented on top of VTM8.0. Experiments were performed as described in JVET-Q0199, for 2X, 1X, and 0.5X. The simulations were performed on a set of pre-processed clips created from SDR CTC YUV4:2:0 clips as shown in Figure 1. The pre-processed test clips alternate between the original content and a rescaled subregion every 0.5 seconds, which is aligned with RPR common test conditions. In the example illustrated in Figure 1, the sub-region (e.g. ROI) is rescaled by 2X and cropped to the same size as the original input.

Figure 1 Pre-processed YUV for alternative scaling window (ROI) simulation

The simulation results versus VTM8.0 are listed in Table 2. It can be observed that more than 10% coding efficiency, on average, can be achieved with the proposed signalling method, which is similar to the result reported in JVET-Q0199 even though the current proposed signalling syntax is much simpler. The results indicate that use of scaling windows for sub-regions of the reference picture or the current picture improves the inter prediction quality significantly.

Table 2 Simulation results

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Proposal Over VTM8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -6.76% | -8.53% | -9.19% | 105% | 97% |
| Class C | -8.09% | -7.28% | -6.99% | 112% | 100% |
| Class E | -19.74% | -22.02% | -19.76% | 104% | 94% |
| Overall | -10.45% | -11.48% | -11.10% | 107% | 97% |
| Class D | -7.01% | -6.42% | -6.14% | 105% | 98% |
| Class F | -14.88% | -13.58% | -12.61% | 101% | 100% |

Per-frame statistics analysis indicates that the bitrate saving when the sequence switches between original pictures and rescaled sub-regions can be quite substantial. The average maximum reduction of all test clips is close to 50% and extreme maximum reduction is over 80%, which may prove helpful in reducing bitrate fluctuations and in easing buffer management.

# Discussion on computational complexity and memory bandwidth

The computational complexity of the current RPR is the same whether the scaling window offsets indicated in the PH or PPS are applied.

The worst case for memory bandwidth for RPR using the alternative scaling window (ROI) is for Case 2 when the alternative scaling window (ROI) is in current picture with picture type B and scalingRatioHor=2 and scalingRatioVer=2, where scalingRatioHor = RefPicScale[i][j][0] / 214 and scalingRatioVer = RefPicScale[i][j][1] / 214 denote the floating representation of horizontal and vertical scaling ratio respectively. (Methods for calculating memory bandwidth are described in Annex A.)

Since ROI RPR window is no larger than PPS RPR window, the worst case memory bandwidth is no greater than that for PPS RPR with the same condition.

The memory constraint specified for RPR in the current VVC draft specification are rewritten by directly using scaling ratio as follows:

- RefPicScale[ i ][ j ][ 0 ] shall be less than or equal to 215

- RefPicScale[ i ][ j ][ 0 ] shall be greater than or equal to 211

- RefPicScale[ i ][ j ][ 1 ] shall be less than or equal to 215

- RefPicScale[ i ][ j ][ 1 ] shall be greater than or equal to 211

- pic_width_max_in_luma_samples shall be greater than or equal to ( Max( 214,RefPicScale[ i ][ j ][ 0 ]) * (pic_width_in_luma_samples − Max( 8, MinCbSizeY )))  >>  14

- pic_height_max_in_luma_samples shall be greater than or equal to ( Max( 214,RefPicScale[ i ][ j ][ 1 ]) * (pic_height_in_luma_samples − Max( 8, MinCbSizeY )))  >>  14

RPR constraints are satisfied for the alternative scaling window (ROI) for Case 1 and Case 2 for either of the following alternative methods:

Alternative 1: Set pic_width_max_in_luma_samples and pic_height_max_in_luma_samples to appropriate values.

For example, set pic_width_max_in_luma_samples = max(max(1,scalingRatioHor)*pic_width_in_luma_samples) and pic_height_max_in_luma_samples = max(max(1,scalingRatioVer)*pic_height_in_luma_samples) for all pictures in a CVS.

Alternative 2: Specify a level (as specified in Table A.1 of the current VVC draft) such that:

PicSizeInSamplesY*(max(1,scalingRatioHor)*max(1,scalingRatioVer))  <= MaxLumaPs, where MaxLumaPs  is specified in Table A-1

It is noted that the RPR memory constraint in the current VVC specification draft is simplified in that it does not consider the real impact of MC interpolation filters. A third alternative method that does consider real MC memory bandwidth is described and analyzed in Annexes A and B. In that alternative method, only reference picture list 0 is used for Case 2 if scalingRatio>1.

# Conclusion

Signalling alternative scaling window offsets in the PH is proposed to simplify the use of sub-regions of current and reference pictures in RPR. RPR using alterative scaling window (ROI) satisfies existing RPR memory bandwidth constraints. The coding efficiency is similar to previous methods but the current method has much simpler syntax and semantics.

# References

- T. Lu, F. Pu, P. Yin, S. McCarty, W. Husak, and T. Chen, “AHG8: Support of ROI (Region-Of-Interest) scalability”, JVET-P0336, Geneva, CH, Oct 2019

- T. Lu, F. Pu, P. Yin, S. McCarty, W. Husak, and T. Chen, “AHG8: Support of ROI (Region-Of-Interest) RPR”, JVET-Q0119, Brussels, BE, Jan. 2020

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-Q2001-vD, Brussels, BE, Jan. 2020

# Patent rights declaration(s)

Intel. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex A: luma-inter prediction memory bandwidth analysis

Assuming block size is WxH (WidthxHeight), the interpolation filter has T taps (T=8), sR[0] and sR[1] denotes horizontal and vertical scaling ratio, respectively.

For MC without RPR, i.e., sR[0]=1 and sR[1]=1, for uni-prediction, number of reference samples is (W+T-1)*(H+T-1). For bi-prediction, number of reference samples is 2*(W+T-1)*(H+T-1). After normalization, it is 2*(W+T-1)*(H+T-1)/(W*H). Blocksize 16x4 and 4x16 has the worse memory bandwidth.

For MC with RPR, for uni-prediction, number of reference samples is ((W-1)*sR[0])+T)*((H-1)*sR[1])+T). After normalization, it is ((W-1)*sR[0])+T)*((H-1)*sR[1])+T)/(W*H). Blocksize 16x4 and 4x16 with sR[0]=sR[1]=2 has worse memory bandwidth.

For 16x4/4x16, MC without RPR, bi-prediction memory bandwidth is 7.9063.

For 16x4/4x16, MC with RPR sR[0]=sR[1]=2, uni-prediction memory bandwidth is 8.3125. Compared to MC without RPR, the additional memory bandwidth is 5% which is the worst case.

# Annex B: only list 0 used for Case 2 if scalingRatio>1

The worst case memory bandwidth is for Case 2 when all pictures have the same resolution, scalingRatioHor and scalingRatioVer are equal to 2, and 16x4 and 4x16 are used for the entire picture.

Specifying that only reference picture list 0 is to be used for Case 2 if scalingRatio>1 would result in a worst case memory bandwidth increase of 5% for the following condition:

Set  pic_width_max_in_luma_samples = max(pic_width_in_luma_samples) and pic_height_max_in_luma_samples = max(pic_height_in_luma_samples) for all pictures in the CVS.

(Methods for calculating memory bandwidth are described in Annex A.)

Results for simulation of this worst case condition are shown in Table 3. Compared to Table 2, the coding performance is 8.86% compared to 10.45%, a reduction by 1.6% by restricting to reference picture list 0 only.

Table 3 Simulation results with memory bandwidth constraint

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Proposal Over VTM8 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -5.13% | -6.49% | -7.24% | 99% | 101% |
| Class C | -6.27% | -5.43% | -4.88% | 95% | 97% |
| Class E | -18.64% | -21.10% | -19.08% | 98% | 95% |
| Overall | -8.89% | -9.79% | -9.42% | 97% | 98% |
| Class D | -5.68% | -5.06% | -4.34% | 90% | 96% |
| Class F | -13.53% | -11.74% | -11.05% | 92% | 102% |