| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0220-r2 |
| --- | --- |

| Title: | AHG9: Weighted prediction syntax cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiancong Luo, Jie Chen, Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | jiancong.luo@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

In VVC draft 8, when weighted bi-prediction is enabled and pred_weight_table syntax is signalled as part of PH syntax structure inside a slice header, weighted prediction parameters for list 1 are unnecessarily signalled for P slices. Two methods are proposed in this contribution. In the first method, when picture_header_in_slice_header_flag is equal to 1, regardless of the value of wp_info_in_ph_flag, pred_weight_table syntax structure is signaled in slice header, but not as part of picture_header syntax structure. In the second method, an encoder constraint is proposed to disallow both picture_header_in_slice_header_flag and wp_info_in_ph_flag being equal to 1.

# Problem Statement

In VVC draft 8 [1], weighted prediction parameters are signaled in pred_weight_table syntax structure for each picture when weighted prediction is enabled. pred_weight_table is signaled in either picture header (PH) syntax structure or slice header. A flag (wp_info_in_ph_flag) in PPS controls whether the pred_weight_table is signaled in PH syntax structure or in slice header. A flag (picture_header_in_slice_header_flag) in slice header indicates whether the PH syntax structure is signaled in slice header or in a separate PH NAL unit in a picture unit (PU).

The values of picture_header_in_slice_header_flag and wp_info_in_ph_flag control where the weighted prediction parameters are signaled as in the following 4 cases:

Case 1: pred_weight_table() in picture_header() inside PH NAL unit;

Case 2: pred_weight_table() in slice header without picture_header() in it;

Case 3: pred_weight_table() in picture_header() inside slice header

Case 4: pred_weight_table() in slice header after picture_header().

In case 3 ( both picture_header_in_slice_header_flag and wp_info_in_ph_flag are equal to 1), the and list 1 weighted prediction parameters (including num_l1_weights and the weights and offsets when num_l1_weights is larger than 0 ) are unnecessary signaled for P slice; such redundant signaling should be avoid. It is also asserted that when picture_header() is in slice_header(), it is unnecessary to support both case 3 and case 4, since they serve the same purpose, i.e. provide weighted prediction parameters for a single slice.

# Proposed methods

In this contribution, we proposed two methods to address the problem.

In method 1, it is proposed to merge case 3 to case 4, i.e. when picture_header() is present in slice header, signal pred_weight_table in slice header but not as part of picture_header() regardless of the value of wp_info_in_ph_flag. The syntax changes are shown in the followings:

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| PictureHeaderInSliceHeaderFlag = 0 |  |
| picture_header_structure( ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && wp_info_in_ph_flag && !PictureHeaderInSliceHeaderFlag ) |  |
| pred_weight_table( ) |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| PictureHeaderInSliceHeaderFlag = picture_header_in_slice_header_flag |  |
| if( picture_header_in_slice_header_flag PictureHeaderInSliceHeaderFlag ) |  |
| picture_header_structure( ) |  |
| … |  |
| if( slice_type != I ) { |  |
| … |  |
| if( ( PictureHeaderInSliceHeaderFlag ¦ ¦ !wp_info_in_ph_flag ) && ( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| … |  |
| } |  |

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_weighted_bipred_flag && wp_info_in_ph_flag && !PictureHeaderInSliceHeaderFlag) |  |
| num_l1_weights | ue(v) |
| for( i = 0; i < NumWeightsL1; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| … |  |
| } |  |

In method 2, it is proposed to disallow case 3 by adding the following encoder constraint:

It is a requirement of bitstream conformance that the values of wp_info_in_ph_flag and picture_header_in_slice_header_flag shall not be both equal to 1.

# Conclusion

In this contribution, it is proposed to address the redundant signaling of list 1 weighted parameters for P slice in VVC draft 8. In the first method, when picuture_header_in_slice_header is equal to 1, regardless of the value of wp_info_in_ph_flag, pred_weight_table syntax structure is signalled in slice header, but not as part of PH syntax structure. In the second method, an conformance constraint is proposed to disallow both picture_header_in_slice_header_flag and wp_info_in_ph_flag being equal to 1.

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).