| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0222 |
| --- | --- |

| Title: | AHG9: SPS sublayer syntax cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiancong Luo, Jie Chen, Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | jiancong.luo@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

This contribution includes two aspects for sublayer syntax cleanup in SPS:

- It is proposed to set inference for syntax element vps_max_sublayers_minus1 when sps_video_parameter_set_id is equal to 0;

- It is proposed to signal sps_sublayer_dpb_params_flag only when sps_ptl_dpb_hrd_params_present_flag is equal to 1.

# Proposal

## Aspect 1

In VVC draft 8, when syntax element sps_video_parameter_set_id is equal to 0, the following applies:

–	The SPS does not refer to a VPS.

–	No VPS is referred to when decoding each CLVS referring to the SPS.

–	The value of vps_max_layers_minus1 is inferred to be equal to 0.

–	The CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS shall have the same value of nuh_layer_id).

–	The value of GeneralLayerIdx[ nuh_layer_id ] is inferred to be equal to 0.

–	The value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is inferred to be equal to 1.

And the value of sps_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1. While vps_max_sublayers_minus1 is signaled in VPS, when sps_video_parameter_set_id is equal to 0, vps_max_sublayers_minus1 is not available for parsing sps_max_sublayers_minus1.

It is proposed to assign an inferred value for sps_max_sublayers_minus1 when sps_video_parameter_set_id is equal to 0, so that the maximum value of sps_max_sublayers_minus1 is still available when SPS does not refer to any VPS. Since the value of vps_max_sublayers_minus1 is in range of 0 to 6, it is proposed to set the range of sps_max_sublayers_minus1 to be the same as that of vps_max_sublayers_minus1 when the SPS does not refer to a VPS.

The specification text change is shown in the following:

sps_video_parameter_set_id, when greater than 0, specifies the value of vps_video_parameter_set_id for the VPS referred to by the SPS.

When sps_video_parameter_set_id is equal to 0, the following applies:

–	The SPS does not refer to a VPS.

–	No VPS is referred to when decoding each CLVS referring to the SPS.

–	The value of vps_max_layers_minus1 is inferred to be equal to 0.

–	The value of vps_max_sublayers_minus1 is inferred to be equal to 6.

–	The CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS shall have the same value of nuh_layer_id).

–	The value of GeneralLayerIdx[ nuh_layer_id ] is inferred to be equal to 0.

–	The value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is inferred to be equal to 1.

## Aspect 2

In SPS, sps_sublayer_dpb_params_flag is only use in dpb_parameters() syntax structure signaled in the SPS. sps_sublayer_dpb_params_flag is signaled when sps_max_sublayers_minus1 is larger than 0. Therefore, the signaling of sps_sublayer_dpb_params_flag is redundant when sps_max_sublayers_minus1 is larger than 0 and sps_ptl_dpb_hrd_params_present_flag is equal to 0, since dpb_parameters() is not signaled under this condition.

It is proposed to signal sps_sublayer_dpb_params_flag when sps_ptl_dpb_hrd_params_present_flag is equal to 1 and sps_max_sublayers_minus1 is larger than 0.

The proposed syntax change is shown as following:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_dpb_params_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ){ |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_dpb_params_flag | u(1) |
| dpb_parameters( sps_max_sublayers_minus1, sps_sublayer_dpb_params_flag ) |  |
| } |  |
| … |  |
| } |  |

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).