| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0227-v1 |
| --- | --- |

| Title: | AHG9: General constraint information semantics constraints and a flag for PH in SH |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Ruoyang Yu Martin Pettersson Mitra Damghanian Zhi Zhang Jack Enhorn | Email: | rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The contribution proposes the following modifications to the general constraint information:

- Mandate the value of no_ccalf_constraint_flag to be equal to 1 when no_alf_constraint_flag is equal to 1. It is reported that when no_alf_constraint_flag is equal to 1, sps_alf_enabled_flag is equal to 0 which disables the adaptive loop filter (ALF) tool, including cross-component ALF.

- Mandate the value of no_sbtmvp_constraint_flag to be equal to 1 when no_temporal_mvp_constraint_flag is equal to 1. It is reported that when no_temporal_mvp_constraint_flag is equal to 1, sps_temporal_mvp_enabled_flag is equal to 0 which disables temporal motion vector prediction, including SbTMVP.

- Rename no_qp_delta_constraint_flag to no_cu_qp_delta_constraint_flag. It is reported that the current name is misleading since no_qp_delta_constraint_flag is only constraining cu_qp_delta_enabled_flag and not ph_qp_delta or slice_qp_delta.

- Add a general constraint information flag no_picture_header_in_slice_header_constraint_flag for the feature to include picture header syntax in the slice header. It is reported that a constraint flag for this feature that was added at the previous meeting in Brussels is missing.

# Proposed text

The four proposed modifications on top of the VVC draft (JVET-Q2001-vE) are shown below with added text marked in yellow.

#### 7.3.3.2 General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| ... | ... |
| no_ccalf_constraint_flag | u(1) |
| ... |  |
| no_sbtmvp_constraint_flag | u(1) |
| ... |  |
| no_cu_qp_delta_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| no_picture_header_in_slice_header_constraint_flag | u(1) |
| no_mixed_nalu_types_in_pic_constraint_flag | u(1) |
| ... |  |
| } |  |

#### 7.4.4.2	General constraint information semantics

...

no_ccalf_constraint_flag equal to 1 specifies that sps_ccalf_enabled_flag shall be equal to 0. no_ccalf_constraint_flag equal to 0 does not impose such a constraint. When no_alf_constraint_flag is equal to 1, the value of no_ccalf_constraint_flag shall be equal to 1.

...

no_sbtmvp_constraint_flag equal to 1 specifies that sps_sbtmvp_enabled_flag shall be equal to 0. no_sbtmvp_constraint_flag equal to 0 does not impose such a constraint. When no_temporal_mvp_constraint_flag is equal to 1, the value of no_sbtmvp_constraint_flag shall be equal to 1.

...

no_cu_qp_delta_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that cu_qp_delta_enabled_flag shall be equal to 0. no_qp_delta_constraint_flag equal to 0 does not impose such a constraint.

...

no_picture_header_in_slice_header_constraint_flag equal to 1 specifies that picture_header_in_slice_header_flag shall be equal to 0 in each slice header in OlsInScope. no_picture_header_in_slice_header_constraint_flag equal to 0 does not impose such a constraint.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).