| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0228-v3 |
| --- | --- |

| Title: | AHG11: Bugfix to deblocking filter boundary strength setting for palette |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Yan Ye, Mohammed Golam Sarwer | Tel: Email: | {ruling.lrl, yan.ye, m.sarwer}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In VVC draft 8, the declocking filter boundary strength is set according to the coding modes of two neighboring blocks along a deblocking edge. However, it is reported that, when one of neighboring blocks is coded in palette and the other is coded in IBC or inter mode, the boundary filtering strength is not clearly defined in the spec. It is also reported that in VTM-8.0, this ambiguity partially exists as well, and can cause the software mismatch in encoder and decoder in some cases. In this contribution, it is proposed to clearly define the boundary filtering strength for the aforementioned case by setting it to one of 1 or 2. It is reported that, as compared to VTM-8.0 with palette off and dual tree on, the overall coding performance impact for {Y, U, V} in 4:4:4 color format is:

- VTM8:{-6.25%,-6.42%,-6.26%}for AI,{-5.04%,-6.81%,-6.86%}for RA,{-3.18%,-4.39%,-4.60%}for LB

- bS = 1:{-6.25%,-6.42%,-6.26%}for AI,{-5.03%,-6.74%,-6.79%}for RA,{-3.16%,-4.22%,-4.37%}for LB

- bS = 2:{-6.25%,-6.42%,-6.26%}for AI,{-5.02%,-6.76%,-6.81%}for RA,{-3.14%,-4.26%,-4.41%}for LB

For class F in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- VTM8:{-1.24%,-0.43%,-0.47%}for AI,{-1.29%,-0.47%,-0.56%}for RA,{-0.56%,-0.24%,-1.24%}for LB

- bS = 1:{-1.24%,-0.43%,-0.48%}for AI,{-1.29%,-0.50%,-0.52%}for RA,{-0.61%,-0.06%,-1.10%}for LB

- bS = 2:{-1.24%,-0.43%,-0.47%}for AI,{-1.32%,-0.53%,-0.52%}for RA,{-0.60%,-0.38%,-0.98%}for LB

For class TGM in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- VTM8:{-6.50%,-5.18%,-4.94%}for AI,{-3.78%,-2.45%,-2.42%}for RA,{-1.22%,-0.59%,-0.66%}for LB

- bS = 1:{-6.50%,-5.18%,-4.94%}for AI,{-3.78%,-2.30%,-2.28%}for RA,{-1.20%,-0.39%,-0.54%}for LB

- bS = 2:{-6.50%,-5.18%,-4.94%}for AI,{-3.76%,-2.36%,-2.39%}for RA,{-1.28%,-0.68%,-0.69%}for LB

In terms of subjective quality, no significant visual difference was observed in two cases based on the informal subjective viewing conducted.

Introduction

In VVC draft 8, the boundary filtering strength (bS) is set according to the following rules:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_luma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is greater than 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_chroma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a coding block edge and the sample p0 or q0 is in a coding block with ciip_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a transform block which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode), bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if cIdx is equal to 0, edgeFlags[ xDi ][ yDj ] is equal to 2, and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two coding sublocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a coding subblock with top-left sample covering ( xSb, ySb ), is equal to PredFlagL0[ xSb ][ ySb ] + PredFlagL1[ xSb ][ ySb ].

      - One motion vector is used to predict the coding subblock containing the sample p0 and one motion vector is used to predict the coding subblock containing the sample q0, and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 8 in units of 1/16 luma samples.

      - Two motion vectors and two different reference pictures are used to predict the coding subblock containing the sample p0, two motion vectors for the same two reference pictures are used to predict the coding subblock containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two coding subblocks for the same reference picture is greater than or equal to 8 in units of 1/16 luma samples.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

        - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

        - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the coding subblock containing the sample p0 and the list 1 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the coding subblock containing the sample p0 and list 0 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples.

    - Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.

In case that one of blocks is coded in palette mode and the other is coded in IBC or inter mode, the boundary filtering strength is not defined due to two reasons:

- It is not clear whether palette mode is part of intra prediction mode or not.

- If palette mode is not part of intra prediction mode, the following condition check is not statisfied due to the description in brackets:

Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode), bS[ xDi ][ yDj ] is set equal to 1.

In VTM-8.0, for single tree case in B-slice, the boundary filtering strength is set equal to 1 when one of blocks is coded in palette mode and the other block is coded in IBC or inter mode according to the following code:

However, the code has problem when current slice is I-slice or a block is coded in local dual tree. In the local dual tree case, when the luma block is IBC mode and the chroma block is palette mode, the reference index is set to 16 for both luma and chroma blocks, where the reference picture with index 16 is not defined. So when deciding the boundary filtering strength for chroma block, the code may have mismatch in encoder and decoder.

Proposed methods

In this contribution, we have tested two different setting for boundary filtering strength, which are 1 and 2 when one of blocks is coded in palette mode and the other is coded in IBC or inter mode.

Firstly, palette mode is treated as part of intra prediction mode. Therefore, the boundary filtering strength is set equal to 2.

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_luma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is greater than 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_chroma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode or pred_mode_plt_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 2.

    - …

Secondly, palette mode is treated as an independent mode. Therefore, when the coding modes of two blocks are not identical, the boundary filtering strength is set equal to 1.

    - …

    - Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode), bS[ xDi ][ yDj ] is set equal to 1.

    - …

Simulation results

The proposed methods are implemented on top of VTM-8.0. The simulation results are tested under CTC and the coding performance impact for Y component are summarized in Table 1. It is noted that all the anchor used in this contribution are VTM-8.0 with dual tree on and palette mode off for two reasons: 1) the palette mode does not work correctly in current software; 2) the palette mode is off in common test condition for 4:2:0 color format. The results show that there is minor difference in objective compression performance for different boundary filtering strength setting.

Table 1. Summary of coding performance impact of Y component

| bS setting |  | Class F for 420 color format |  |  | TGM for 420 color format |  |  | Overall for 444 color format |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI | RA | LB | AI | RA | LB | AI | RA | LB |
| VTM-8.0 | -1.24% | -1.29% | -0.56% | -6.50% | -3.78% | -1.22% | -6.25% | -5.04% | -3.18% |
| bS = 1 | -1.24% | -1.29% | -0.61% | -6.50% | -3.78% | -1.20% | -6.25% | -5.03% | -3.16% |
| bS = 2 | -1.24% | -1.32% | -0.60% | -6.50% | -3.76% | -1.28% | -6.25% | -5.02% | -3.14% |

Table 2. VTM-8.0 simulation results for sequences in 4:2:0 color format

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.24% | -0.43% | -0.47% | 104% | 98% |
| Class TGM | -6.50% | -5.18% | -4.94% | 99% | 90% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.29% | -0.47% | -0.56% | 105% | 100% |
| Class TGM | -3.78% | -2.45% | -2.42% | 100% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.56% | -0.24% | -1.24% | 107% | 99% |
| Class TGM | -1.22% | -0.59% | -0.66% | 104% | 100% |

Table 3. VTM-8.0 simulation results for sequences in 4:4:4 color format

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -13.65% | -15.05% | -14.32% | 101% | 87% |
| TGM 720p | -7.72% | -6.91% | -6.31% | 102% | 94% |
| Animation | -0.54% | -0.94% | -1.15% | 107% | 100% |
| Mixed content | -4.32% | -4.05% | -4.76% | 106% | 98% |
| Camera-Captured content | 0.03% | 0.07% | 0.07% | 108% | 100% |
| Overall | -6.25% | -6.42% | -6.26% | 104% | 95% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -10.42% | -13.99% | -12.92% | 99% | 94% |
| TGM 720p | -6.77% | -8.61% | -9.16% | 101% | 97% |
| Animation | -0.50% | -0.54% | -1.05% | 103% | 100% |
| Mixed content | -3.44% | -5.61% | -6.01% | 104% | 101% |
| Camera-Captured content | 0.00% | -0.05% | -0.10% | 100% | 100% |
| Overall | -5.04% | -6.81% | -6.86% | 101% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -7.34% | -9.54% | -9.12% | 104% | 103% |
| TGM 720p | -3.58% | -4.54% | -5.28% | 106% | 102% |
| Animation | -0.52% | -0.94% | -1.31% | 105% | 101% |
| Mixed content | -1.91% | -3.66% | -3.95% | 109% | 105% |
| Camera-Captured content | 0.02% | -0.02% | -0.13% | 101% | 105% |
| Overall | -3.18% | -4.39% | -4.60% | 105% | 103% |

Table 4. Simulation results for sequences in 4:2:0 color format when bS is set to 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.24% | -0.43% | -0.48% | 104% | 98% |
| Class TGM | -6.50% | -5.18% | -4.94% | 100% | 90% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.29% | -0.50% | -0.52% | 105% | 99% |
| Class TGM | -3.78% | -2.30% | -2.28% | 100% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.61% | -0.06% | -1.10% | 107% | 97% |
| Class TGM | -1.20% | -0.39% | -0.54% | 104% | 98% |

Table 5. Simulation results for sequences in 4:4:4 color format when bS is set to 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -13.65% | -15.05% | -14.32% | 101% | 86% |
| TGM 720p | -7.72% | -6.91% | -6.31% | 102% | 93% |
| Animation | -0.54% | -0.94% | -1.15% | 106% | 99% |
| Mixed content | -4.32% | -4.05% | -4.76% | 106% | 96% |
| Camera-Captured content | 0.03% | 0.07% | 0.07% | 107% | 99% |
| Overall | -6.25% | -6.42% | -6.26% | 104% | 93% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -10.38% | -13.73% | -12.69% | 98% | 95% |
| TGM 720p | -6.75% | -8.53% | -9.04% | 100% | 97% |
| Animation | -0.49% | -0.54% | -1.05% | 102% | 99% |
| Mixed content | -3.49% | -5.67% | -6.09% | 103% | 100% |
| Camera-Captured content | 0.01% | -0.09% | -0.12% | 100% | 100% |
| Overall | -5.03% | -6.74% | -6.79% | 101% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -7.26% | -9.13% | -8.70% | 103% | 103% |
| TGM 720p | -3.56% | -4.41% | -4.99% | 105% | 99% |
| Animation | -0.53% | -1.00% | -1.35% | 105% | 100% |
| Mixed content | -1.93% | -3.48% | -3.70% | 106% | 103% |
| Camera-Captured content | 0.01% | 0.02% | 0.01% | 101% | 103% |
| Overall | -3.16% | -4.22% | -4.37% | 104% | 102% |

Table 6. Simulation results for sequences in 4:2:0 color format when bS is set to 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.24% | -0.43% | -0.47% | 104% | 100% |
| Class TGM | -6.50% | -5.18% | -4.94% | 100% | 90% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.32% | -0.53% | -0.52% | 105% | 100% |
| Class TGM | -3.76% | -2.36% | -2.39% | 100% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.60% | -0.38% | -0.98% | 106% | 95% |
| Class TGM | -1.28% | -0.68% | -0.69% | 104% | 97% |

Table 7. Simulation results for sequences in 4:4:4 color format when bS is set to 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -13.65% | -15.05% | -14.32% | 101% | 86% |
| TGM 720p | -7.72% | -6.91% | -6.31% | 103% | 94% |
| Animation | -0.54% | -0.94% | -1.15% | 106% | 99% |
| Mixed content | -4.32% | -4.05% | -4.76% | 106% | 96% |
| Camera-Captured content | 0.03% | 0.07% | 0.07% | 108% | 100% |
| Overall | -6.25% | -6.42% | -6.26% | 104% | 94% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -10.41% | -13.86% | -12.81% | 98% | 94% |
| TGM 720p | -6.72% | -8.55% | -9.06% | 101% | 98% |
| Animation | -0.49% | -0.53% | -1.04% | 103% | 99% |
| Mixed content | -3.47% | -5.58% | -6.05% | 104% | 100% |
| Camera-Captured content | 0.00% | -0.05% | -0.10% | 100% | 100% |
| Overall | -5.02% | -6.76% | -6.81% | 101% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -7.24% | -9.31% | -8.83% | 103% | 102% |
| TGM 720p | -3.47% | -4.34% | -4.74% | 104% | 98% |
| Animation | -0.54% | -0.98% | -1.41% | 105% | 101% |
| Mixed content | -1.90% | -3.54% | -3.95% | 106% | 102% |
| Camera-Captured content | 0.02% | -0.02% | -0.13% | 100% | 102% |
| Overall | -3.14% | -4.26% | -4.41% | 104% | 101% |

Subjective quality

The subjective quality of some decoded sequences are checked. We did not identify any difference between different settings of boundary filtering strength.

Conclusion

In this contribution, two different settings of boundary filtering strength are tested to address the issue of undefined boundary filtering strength when one block is coded in palette mode and the other block is coded in IBC or inter mode. It shows that there is minor impact on coding performance and subjective quality. It is recommend to adopt one of settings to VVC draft.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).