| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0229-v3 |
| --- | --- |

| Title: | AHG11: Fixed number of reuse flags for palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Yan Ye, Mohammed Golam Sarwer | Tel: Email: | {ruling.lrl, yan.ye, m.sarwer}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In the VVC draft 8, number of reuse flags to be decoded is related to the size of the palette predictor. In the case that two neighboring blocks are both coded in palette mode, the syntax of the second block cannot be parsed until the new palette predictor size is obtained. In this contribution, the number of reuse flags is set to a fixed value, that is, 63 and 31 for single tree slice and dual tree slice, respectively. Therefore, the syntax of second block can be parsed with no dependency on how large the previous block’s palette predictor is. Fixed number of reuse flags can be achieved by two methods: 1) initializing the palette predictor size to the fixed value; 2) add a requirement of bitstream conformance to palette_predictor_run signaling. It is reported that, as compared to VTM-8.0 with palette on, the overall coding performance impact for {Y, U, V} in 4:4:4 color format is:

- #1:{0.01%,0.01%,0.01%}for AI,{0.03%,0.05%,0.01%}for RA,{0.02%,0.04%,-0.02%}for LB

- #2:{0.01%,0.00%,0.02%}for AI,{0.03%,0.04%,-0.01%}for RA,{0.04%,0.03%,0.09%}for LB

For class F in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- #1:{-0.01%,-0.07%,0.03%}for AI,{0.01%, 0.08%,0.07%}for RA,{0.07%,-0.24%,-0.22%}for LB

- #2:{-0.02%,-0.04%,0.05%}for AI,{-0.02%,0.10%,0.08%}for RA,{0.12%,-0.34%,-0.15%}for LB

For class TGM in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- #1:{0.00%,0.01%,0.00%}for AI,{0.05%,-0.06%,-0.04%}for RA,{0.01%,-0.09%,0.11%}for LB

- #2:{0.01%,0.00%,-0.01%}for AI,{0.05%,-0.04%,-0.04%}for RA,{0.07%,-0.06%,0.07%}for LB

Introduction

The palette table syntax design in VVC draft 8 is shown in Table 1, where the reuse flags of a palette coded CU are firstly decoded followed by the component values of the new palette entries. Before parsing the reuse flags, the number of palette predictor entries (i.e. “PredictorPaletteSize[ startComp ]” in Table 1) need to be known. Currently, there are two ways to obtain the palette predictor size: 1) performing the palette predictor update process in the CABAC parsing stage; 2) calculating the size using the following equation:

new palette predictor size=min⁡(maxNumPalettePredictorSize,
                     num_signalled_palette_entries+PredictorPaletteSize startComp )

Table 1. Syntax table for palette mode

| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| startComp = ( treeType = = DUAL_TREE_CHROMA ) ? 1 : 0 |  |
| numComps = ( treeType = = SINGLE_TREE ) ? ( ChromaArrayType = = 0 ? 1 : 3 ) : ( treeType = = DUAL_TREE_CHROMA ) ? 2 : 1 |  |
| maxNumPaletteEntries = ( treeType = = SINGLE_TREE ) ? 31 : 15 |  |
| palettePredictionFinished = 0 |  |
| NumPredictedPaletteEntries = 0 |  |
| for( predictorEntryIdx = 0; predictorEntryIdx < PredictorPaletteSize[ startComp ] && !palettePredictionFinished && NumPredictedPaletteEntries < maxNumPaletteEntries; predictorEntryIdx++ ) { |  |
| palette_predictor_run | ae(v) |
| if( palette_predictor_run != 1 ) { |  |
| if( palette_predictor_run > 1 ) |  |
| predictorEntryIdx += palette_predictor_run − 1 |  |
| PalettePredictorEntryReuseFlags[ predictorEntryIdx ] = 1 |  |
| NumPredictedPaletteEntries++ |  |
| } else |  |
| palettePredictionFinished = 1 |  |
| } |  |
| if( NumPredictedPaletteEntries < maxNumPaletteEntries ) |  |
| num_signalled_palette_entries | ae(v) |
| for( cIdx = startComp; cIdx < ( startComp + numComps ); cIdx++ ) |  |
| for( i = 0; i < num_signalled_palette_entries; i++ ) |  |
| new_palette_entries[ cIdx ][ i ] | ae(v) |

Proposed method

It is proposed to set the number of reuse flags to be equal to the maximum number of palette predictor size which is 63 for single tree slice and 31 for dual tree slice. Since the new palette predictor size is larger than or equal to the current palette predictor size, fixed number of reuse flags can be achieved by initializing the palette predictor size to maximum value. Besides, the component values for each entry are all set to 0. With the proposed method, it does not need to perform palette predictor update process or calculate the new palette predictor size during CABAC parsing. Although there is some reuse flag signaling redundancy in the proposed simplification, upon further analysis, the impact is found to be very small because the coding method of reuse flag in VVC draft 8 is very robust and efficient. The reuse flags are coded using run-length coding, and as shown in Figure 2, when the coded run-length value is equal to 1, it is interpreted as “end of run,” i.e., the remaining reuse flags are all set to 0. This analysis is supported by simulation results presented in the next section.

Figure 2. Example of reuse flags binarization

Instead of initializing the palette predictor size to maximum value, alternative way is to add a requirement of bitstream conformance to the palette_predictor_run signaling. It is required that the value of reuse flags for the palette predictor entries that are larger than the current palette predictor size should be 0.

Simulation results

The proposed methods are implemented on top of VTM-8.0. The simulation results are tested under CTC and the coding performance impact for Y component are summarized in Table 3. It is noted that all the anchor used in this contribution are VTM-8.0 with palette mode on. Besides, the deblocking filter boundary strength for palette mode is set to 1 in both anchor and proposed methods (i.e. the bugfix proposed in JVET-R0228). The results show that there is minor difference in objective compression performance.

#1. Initializing the palette predictor size to maximum value

#2. Adding bitstream conformance requirement to the palette_predictor_run signaling

Table 3. Summary of coding performance impact of Y component

|  | Overall for 444 color format |  |  | Class F for 420 color format |  |  | SCC for 420 color format |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LB | AI | RA | LB | AI | RA | LB |
| #1 | 0.01% | 0.03% | 0.02% | -0.01% | 0.01% | 0.07% | 0.00% | 0.05% | 0.01% |
| #2 | 0.01% | 0.03% | 0.04% | -0.02% | -0.02% | 0.12% | 0.01% | 0.05% | 0.07% |

Table 4. Fixed number of reuse flags in 4:2:0 color format using method 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 with PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.01% | -0.07% | 0.03% | 100% | 100% |
| Class TGM | 0.00% | 0.01% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.01% | 0.08% | 0.07% | 101% | 101% |
| Class TGM | 0.05% | -0.06% | -0.04% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.07% | -0.24% | -0.22% | 100% | 99% |
| Class TGM | 0.01% | -0.09% | 0.11% | 100% | 100% |

Table 5. Fixed number of reuse flags in 4:4:4 color format using method 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 with PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | 0.00% | -0.01% | 100% | 100% |
| TGM 720p | 0.03% | 0.02% | 0.06% | 100% | 100% |
| Animation | 0.02% | 0.00% | 0.05% | 100% | 100% |
| Mixed content | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Camera-Captured content | 0.02% | -0.01% | -0.04% | 100% | 100% |
| Overall | 0.01% | 0.01% | 0.01% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.04% | 0.04% | 0.06% | 100% | 100% |
| TGM 720p | 0.02% | 0.04% | -0.14% | 100% | 101% |
| Animation | -0.02% | -0.10% | -0.01% | 100% | 100% |
| Mixed content | 0.08% | 0.16% | 0.16% | 100% | 100% |
| Camera-Captured content | 0.03% | 0.10% | 0.01% | 100% | 100% |
| Overall | 0.03% | 0.05% | 0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.06% | -0.03% | -0.02% | 101% | 100% |
| TGM 720p | 0.02% | 0.24% | 0.08% | 100% | 100% |
| Animation | 0.05% | 0.04% | 0.03% | 100% | 100% |
| Mixed content | 0.09% | -0.07% | -0.17% | 101% | 99% |
| Camera-Captured content | 0.04% | -0.07% | -0.05% | 100% | 101% |
| Overall | 0.02% | 0.04% | -0.02% | 100% | 100% |

Table 6. Fixed number of reuse flags in 4:2:0 color format using method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 with PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.02% | -0.04% | 0.05% | 100% | 99% |
| Class TGM | 0.01% | 0.00% | -0.01% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.02% | 0.10% | 0.08% | 100% | 101% |
| Class TGM | 0.05% | -0.04% | -0.04% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.12% | -0.34% | -0.15% | 100% | 99% |
| Class TGM | 0.07% | -0.06% | 0.07% | 100% | 99% |

Table 7. Fixed number of reuse flags in 4:4:4 color format using method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 with PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.02% | -0.01% | -0.01% | 100% | 100% |
| TGM 720p | 0.03% | 0.01% | 0.08% | 100% | 99% |
| Animation | 0.03% | -0.01% | 0.04% | 100% | 100% |
| Mixed content | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Camera-Captured content | 0.02% | -0.01% | -0.04% | 100% | 100% |
| Overall | 0.01% | 0.00% | 0.02% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.04% | 0.04% | 0.06% | 100% | 99% |
| TGM 720p | 0.04% | 0.04% | -0.14% | 100% | 101% |
| Animation | -0.02% | -0.10% | -0.02% | 100% | 100% |
| Mixed content | 0.05% | 0.12% | 0.08% | 100% | 100% |
| Camera-Captured content | 0.03% | 0.10% | 0.01% | 100% | 99% |
| Overall | 0.03% | 0.04% | -0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 with PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.03% | 0.02% | 0.05% | 100% | 99% |
| TGM 720p | 0.08% | 0.21% | 0.34% | 100% | 99% |
| Animation | 0.06% | 0.04% | 0.06% | 100% | 101% |
| Mixed content | 0.00% | -0.17% | -0.05% | 100% | 101% |
| Camera-Captured content | 0.04% | -0.07% | -0.05% | 100% | 101% |
| Overall | 0.04% | 0.03% | 0.09% | 100% | 100% |

Conclusion

This contribution proposes to set the number of reuse flags to a fixed value of 63 for single tree slice and 31 for dual tree slice. It is asserted that this proposed method has the advantage of decoupling the reuse flag parsing from the palette predictor size, and the size of new palette predictor does not need to be known during CABAC parsing of palette coded CU. Analysis of the reuse flag coding method, as well as thee simulation results show that there is negligible coding performance impact. It is recommended to adopt this proposal to VVC draft.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).