| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0230 |
| --- | --- |

| Title: | AHG2: Syntax clean-up for cross component adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In VVC draft 8, cross component adaptive loop filter (CC-ALF) is adopted to refine chroma components by using luma samples. Filter coefficients of CC-ALF are signalled in adaptation parameter sets (APSs). In an APS, a Cb (Cr resp.) CC-ALF filter set with up to 4 filters could be signalled and a filter from this Cb (Cr resp.) filter set could be applied to a Cb (Cr resp.) coding tree block. On the other hand, in an APS, a chroma adaptive loop filter (ALF) filter set with up to 8 filters could be signalled and a filter from this chroma filter set could be applied to a chroma coding tree block. In this contribution, CC-ALF filters are unified for the two chroma components. Cb and Cr components share the same CC-ALF filter set in an APS, which is the same method used in a regular chroma ALF. Under common test conditions, compared with VTM-8.0, the average BD rate of the proposed method is

AI   : 0.03%(Y), -0.17%(U), -0.20%(V)

RA  : 0.01%(Y), -0.10%(U), 0.06%(V)

LDB: -0.04%(Y), -0.16%(U), -0.04%(V)

LDP: -0.06%(Y), -0.09%(U), 0.21%(V)

# Introduction

In VVC draft 8, cross component adaptive loop filter (CC-ALF) is adopted to refine chroma components by using luma samples. Filter coefficients are signalled in adaptive loop filter (ALF) adaptation parameter sets (APS). In one APS, a Cb (Cr resp.) filter set with up to 4 filters could be signalled and a Cb (Cr resp.) filter from this Cb (Cr resp.) set could be applied to a Cb (Cr resp.) coding tree block (CTB). On the other hand, in an ALF APS, a chroma ALF filter set with up to 8 filters could be signalled. Both chroma components share the same filter set. A filter from a chroma filter set in an ALF APS could be applied to a chroma (either Cb or Cr) coding tree block.

# Proposed method

In this contribution, it is proposed to cleanup the CC-ALF syntax elements and unify the filter storage between chroma ALF and CC-ALF by using the same method applied in a regular chroma ALF. As a result, Cb and Cr components share the same CC-ALF filter set in an ALF APS.  To be specific, an ALF APS could signal CC-ALF filter set with up to 8 CC-ALF filters, as the following modified syntax table.

| alf_cc_cb_filter_signal_flag | u(1) |
| --- | --- |
| alf_cc_cr_filter_signal_flag | u(1) |
| if( alf_cc_cb_filter_signal_flag ) { |  |
| alf_cc_cb_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < alf_cc_cb_filters_signalled_minus1 + 1; k++ ) { |  |
| for( j = 0; j < 7; j++ ) { |  |
| alf_cc_cb_mapped_coeff_abs[ k ][ j ] | u(3) |
| if( alf_cc_cb_mapped_coeff_abs[ k ][ j ] ) |  |
| alf_cc_cb_coeff_sign[ k ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_cc_cr_filter_signal_flag ) { |  |
| alf_cc_cr_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < alf_cc_cr_filters_signalled_minus1 + 1; k++ ) { |  |
| for( j = 0; j < 7; j++ ) { |  |
| alf_cc_cr_mapped_coeff_abs[ k ][ j ] | u(3) |
| if( alf_cc_cr_mapped_coeff_abs[ k ][ j ] ) |  |
| alf_cc_cr_coeff_sign[ k ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |

In a picture or slice header, in VVC draft 8, an ALF APS index is signalled for each chroma component to indicate which APS is used for CCALF. In this contribution, it is proposed to signal one APS index for both chroma components, which is the same method used in a regular chroma ALF. The modified syntax table in slice header is shown as below.

| slice_cc_alf_cb_enabled_flag | u(1) |
| --- | --- |
| if( slice_cc_alf_cb_enabled_flag ) |  |
| slice_cc_alf_cb_aps_id | u(3) |
| slice_cc_alf_cr_enabled_flag | u(1) |
| if( slice_cc_alf_cb_enabled_flag ¦¦ slice_cc_alf_cr_enabled_flag ) |  |
| slice_cc_alf_cr_aps_id | u(3) |

A chroma (either Cb or Cr) CTB can utilize any CC-ALF filter in this ALF APS.

# Experimental results

The proposed method is implemented on top of VTM-8.0 and tested under common test condition defined in JVET-N1010.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.09% | -0.04% | 101% | 100% |
| Class A2 | 0.04% | -0.05% | -0.78% | 101% | 97% |
| Class B | 0.03% | -0.46% | 0.06% | 101% | 99% |
| Class C | 0.02% | -0.08% | -0.19% | 100% | 99% |
| Class E | 0.01% | -0.03% | -0.21% | 100% | 95% |
| Overall | 0.03% | -0.17% | -0.20% | 100% | 98% |
| Class D | 0.01% | -0.02% | -0.16% | 101% | 96% |
| Class F | 0.01% | -0.08% | -0.03% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | 0.16% | 101% | 100% |
| Class A2 | 0.02% | -0.07% | -0.19% | 101% | 100% |
| Class B | 0.02% | -0.19% | 0.31% | 102% | 100% |
| Class C | 0.00% | -0.07% | -0.13% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.10% | 0.06% | 101% | 100% |
| Class D | 0.00% | -0.11% | 0.00% | 101% | 101% |
| Class F | 0.03% | 0.02% | -0.14% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.11% | 0.02% | 100% | 98% |
| Class C | -0.08% | -0.39% | 0.09% | 101% | 103% |
| Class E | -0.03% | 0.06% | -0.33% | 101% | 105% |
| Overall | -0.04% | -0.16% | -0.04% | 101% | 101% |
| Class D | -0.06% | -0.09% | -0.45% | 101% | 104% |
| Class F | 0.00% | 0.34% | 0.29% | 100% | 99% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.15% | 0.76% | 101% | 100% |
| Class C | -0.05% | -0.06% | -0.05% | 100% | 103% |
| Class E | -0.11% | -0.04% | -0.37% | 99% | 98% |
| Overall | -0.06% | -0.09% | 0.21% | 100% | 100% |
| Class D | 0.02% | 0.42% | 0.22% | 101% | 104% |
| Class F | -0.04% | -0.34% | -1.09% | 101% | 104% |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).