| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0231 |
| --- | --- |

| Title: | AHG2: Rounding offsets for adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Virtual boundary (VB) processing is adopted to VVC to avoid line buffer increment for adaptive loop filter (ALF) and cross component adaptive loop filter (CC-ALF). Symmetrical sample padding is applied when VB processing is applied to ALF and CC-ALF. In some cases, extreme padding for the closest row on each side of a VB may introduce visual artifacts. In VVC, to account for that in ALF when ALF is applied to samples on the rows adjacent to a VB, filter strength is reduced by increasing the right shift for ALF filtering. However, the rounding offset of the right shift is not changed and is kept the same for all values of the right shift. In addition, the filter strength for samples on the rows adjacent to a virtual boundary is not changed in CC-ALF. In this contribution, at first, the rounding offset is changed for different right shift values for ALF. In another aspect, the filter strength is reduced when applying CC-ALF to the samples on the rows adjacent to a VB. Compared to VTM-8.0, the average BD-rate for the proposed methods is as follows:

| Results over VTM-8.0 | All Intra |  |  | Random Access |  |  | Low Delay B |  |  | Low Delay P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Method 1 | 0.00% | 0.01% | 0.01% | 0.01% | -0.01% | 0.00% | 0.01% | 0.02% | -0.07% | 0.01% | -0.25% | -0.14% |
| Method 2 | 0.00% | 0.05% | 0.05% | 0.00% | -0.01% | 0.00% | -0.05% | 0.00% | -0.09% | -0.01% | -0.31% | 0.20% |
| Method 3 | 0.00% | 0.07% | 0.07% | 0.01% | 0.01% | 0.04% | -0.04% | 0.09% | 0.05% | 0.01% | -0.32% | -0.17% |

# Introduction

In VVC draft 8, virtual boundary (VB) is defined for adaptive loop filter (ALF) and cross component adaptive loop filter (CC-ALF). The position of a VB is 4 lines of luma samples and 2 lines of chroma samples above a horizontal coding tree unit boundary. Symmetry sample padding is applied to avoid using samples below a VB when filtering a sample above the VB, and to avoid using samples above a VB when filtering a sample below the VB. When filtering samples immediately adjacent to a VB, ALF virtual boundary processing can give rise to visual artifacts. To reduce the artifact, in ALF filtering, when filtering samples in rows that adjacent to an ALF VB, filter strength is reduced by increasing the right shift in filtering process. However, the rounding offset is not increased when right shift value is increased. In addition, when applying CC-ALF to samples on rows adjacent to a VB, the filtering strength is not changed.

# Proposed Methods

In this contribution, three methods are proposed.

## Method 1

When right shift of ALF filtering is increased, the rounding offset is increased as the highlighted text change:

sum = curr + ( ( sum + 64(1<<(alfShiftY - 1)) )  >>  alfShiftY  )   	(1467)

sum = curr + ( ( sum + 64(1<<(alfShiftC - 1)) )  >>  alfShiftC )	(1508)

## Method 2

When CC-ALF is applied to samples on rows adjacent to a VB, the right shift is increased, as the highlighted text change:

scaledSum = Clip3( −( 1 << ( BitDepth − 1 ) ), ( 1 << ( BitDepth  − 1 ) ) − 1, ( sum + 64 ) >> 7ccalfShift)       (1533)

Table 47 – Specification of ccalfShift, yP1 and yP2 according to the vertical luma sample position ( y * subHeightC ) and applyAlfLineBufBoundary

| Condition | ccalfShift | yP1 | yP2 |
| --- | --- | --- | --- |
| ( y * subHeightC = = CtbSizeY − 5 ¦ ¦ y * subHeightC = = CtbSizeY − 4 ) && applyAlfLineBufBoundary = = 1 | 10 | 0 | 0 |
| ( y * subHeightC = = CtbSizeY − 6 ¦ ¦ y * subHeightC = = CtbSizeY − 3 ) && applyAlfLineBufBoundary = = 1 | 7 | 1 | 1 |
| Otherwise | 7 | 1 | 2 |

## Method 3

When CC-ALF is applied to samples on rows adjacent to a VB, the right shift is increased. In both CC-ALF and ALF, when the right shift is increased, the rounding offset is increased as well. The text change is highlighted below:

sum = curr + ( ( sum + 64(1<<(alfShiftY - 1)) )  >>  alfShiftY  )   	(1467)

sum = curr + ( ( sum + 64(1<<(alfShiftC - 1)) )  >>  alfShiftC )	(1508)

scaledSum = Clip3( −( 1 << ( BitDepth − 1 ) ), ( 1 << ( BitDepth  − 1 ) ) − 1, ( sum + 64(1<<(ccalfShift - 1))  ) >> 7ccalfShift)       (1533)

Table 47 – Specification of ccalfShift, yP1 and yP2 according to the vertical luma sample position ( y * subHeightC ) and applyAlfLineBufBoundary

| Condition | ccalfShift | yP1 | yP2 |
| --- | --- | --- | --- |
| ( y * subHeightC = = CtbSizeY − 5 ¦ ¦ y * subHeightC = = CtbSizeY − 4 ) && applyAlfLineBufBoundary = = 1 | 10 | 0 | 0 |
| ( y * subHeightC = = CtbSizeY − 6 ¦ ¦ y * subHeightC = = CtbSizeY − 3 ) && applyAlfLineBufBoundary = = 1 | 7 | 1 | 1 |
| Otherwise | 7 | 1 | 2 |

# Experimental Results

The proposed methods are implemented on top of VTM-8.0 and tested under common test condition defined in JVET-N1010.

## Method 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.02% | 99% | 99% |
| Class A2 | 0.00% | 0.01% | 0.01% | 99% | 98% |
| Class B | 0.00% | 0.01% | 0.01% | 99% | 98% |
| Class C | 0.00% | 0.01% | 0.01% | 99% | 101% |
| Class E | 0.00% | 0.01% | 0.02% | 99% | 97% |
| Overall | 0.00% | 0.01% | 0.01% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Class F | 0.00% | 0.01% | 0.01% | 100% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.14% | -0.06% | 100% | 100% |
| Class A2 | 0.00% | 0.05% | 0.13% | 100% | 100% |
| Class B | 0.02% | -0.02% | -0.02% | 100% | 100% |
| Class C | 0.01% | 0.05% | -0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.06% | -0.12% | 101% | 101% |
| Class F | 0.00% | -0.04% | -0.04% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.15% | 0.12% | 100% | 97% |
| Class C | 0.00% | -0.21% | -0.15% | 100% | 105% |
| Class E | 0.06% | 0.11% | -0.29% | 102% | 104% |
| Overall | 0.01% | 0.02% | -0.07% | 101% | 101% |
| Class D | 0.00% | -0.68% | -0.45% | 101% | 103% |
| Class F | -0.03% | 0.23% | -0.18% | 100% | 98% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.05% | 0.16% | 100% | 98% |
| Class C | 0.01% | -0.14% | -0.33% | 100% | 104% |
| Class E | 0.01% | -0.72% | -0.37% | 99% | 95% |
| Overall | 0.01% | -0.25% | -0.14% | 100% | 99% |
| Class D | -0.03% | 0.10% | -0.51% | 100% | 101% |
| Class F | 0.01% | -0.11% | -0.65% | 100% | 101% |

## Method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.04% | 99% | 102% |
| Class A2 | 0.00% | 0.06% | 0.10% | 100% | 98% |
| Class B | 0.00% | 0.10% | 0.07% | 100% | 98% |
| Class C | 0.00% | 0.01% | 0.01% | 100% | 101% |
| Class E | 0.00% | 0.02% | 0.04% | 100% | 99% |
| Overall | 0.00% | 0.05% | 0.05% | 100% | 100% |
| Class D | 0.00% | 0.02% | 0.01% | 100% | 100% |
| Class F | 0.00% | 0.03% | 0.04% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.13% | -0.02% | 99% | 99% |
| Class A2 | 0.00% | 0.06% | 0.13% | 100% | 100% |
| Class B | 0.00% | 0.01% | -0.03% | 100% | 100% |
| Class C | -0.01% | 0.00% | -0.07% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class D | -0.01% | 0.00% | 0.02% | 101% | 101% |
| Class F | 0.02% | 0.00% | -0.01% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.15% | -0.06% | 100% | 96% |
| Class C | -0.09% | -0.01% | -0.13% | 100% | 99% |
| Class E | -0.05% | -0.24% | -0.11% | 100% | 101% |
| Overall | -0.05% | 0.00% | -0.09% | 100% | 98% |
| Class D | -0.09% | -0.82% | -0.34% | 100% | 101% |
| Class F | -0.06% | 0.57% | 0.20% | 100% | 99% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.10% | 0.39% | 100% | 98% |
| Class C | 0.00% | -0.29% | 0.13% | 100% | 102% |
| Class E | -0.04% | -0.67% | -0.01% | 99% | 99% |
| Overall | -0.01% | -0.31% | 0.20% | 100% | 100% |
| Class D | -0.01% | 0.03% | 0.76% | 100% | 100% |
| Class F | 0.03% | -0.16% | -0.64% | 101% | 102% |

## Method 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | 0.07% | 100% | 100% |
| Class A2 | 0.00% | 0.09% | 0.12% | 101% | 98% |
| Class B | 0.00% | 0.11% | 0.08% | 100% | 101% |
| Class C | 0.00% | 0.02% | 0.03% | 99% | 98% |
| Class E | 0.00% | 0.03% | 0.06% | 100% | 96% |
| Overall | 0.00% | 0.07% | 0.07% | 100% | 99% |
| Class D | 0.00% | 0.02% | 0.02% | 99% | 97% |
| Class F | 0.00% | 0.04% | 0.04% | 99% | 97% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.02% | 102% | 101% |
| Class A2 | 0.02% | -0.04% | 0.21% | 101% | 100% |
| Class B | 0.01% | 0.05% | 0.04% | 102% | 101% |
| Class C | 0.00% | 0.03% | -0.08% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.04% | 101% | 101% |
| Class D | 0.00% | 0.14% | -0.08% | 101% | 102% |
| Class F | 0.00% | 0.00% | -0.08% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.06% | 0.26% | 101% | 99% |
| Class C | -0.06% | -0.15% | -0.12% | 100% | 102% |
| Class E | -0.04% | 0.47% | -0.08% | 102% | 106% |
| Overall | -0.04% | 0.09% | 0.05% | 101% | 102% |
| Class D | -0.06% | -0.18% | -0.41% | 101% | 103% |
| Class F | -0.06% | 0.49% | 0.37% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.15% | 0.20% | 100% | 99% |
| Class C | 0.01% | -0.29% | -0.10% | 100% | 101% |
| Class E | 0.00% | -0.64% | -0.89% | 100% | 101% |
| Overall | 0.01% | -0.32% | -0.17% | 100% | 100% |
| Class D | -0.04% | -0.41% | -0.01% | 100% | 101% |
| Class F | -0.05% | -0.32% | -0.54% | 101% | 104% |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).