| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0233 |
| --- | --- |

| Title: | AHG16: Line buffer problem of CC-ALF for 4:2:2 and 4:4:4 sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In VVC draft 8, cross component adaptive loop filter (CC-ALF) is adopted to refine chroma components by using luma samples. To get an offset for a chroma sample, CC-ALF is applied to luma samples (after applying luma sample adaptive offset) where the centre of the filter is the co-located luma sample of the current chroma sample.

To reduce the line buffer requirement for adaptive loop filter (ALF) and CC-ALF, virtual boundary (VB) processing is applied to both ALF and CC-ALF. The position of a VB is 4 lines of luma samples and 2 lines of chroma samples above a horizontal coding tree unit (CTU) boundary. When ALF or CC-ALF is applied, a current to-be-filtered sample above (below resp.) a VB can not use samples below (above resp.) the VB.

However, when CC-ALF is applied to 4:2:2 and 4:4:4 video sequences, in some cases, when current to-be-filtered chroma sample is above a VB, its co-located luma sample is below the VB, which conflicts the design of VB.

In this contribution, three methods are proposed to solve the problem. Compared to VTM-8.0, the average BD-rate for the proposed methods is as follows:

| Results over VTM-8.0 | All Intra |  |  | Random Access |  |  | Low Delay B |  |  | Low Delay P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Method 1 | 0.00% | 0.00% | 0.01% | 0.01% | -0.03% | 0.02% | 0.01% | -0.06% | 0.01% | -0.02% | -0.02% | -0.04% |
| Method 2 | 0.00% | 0.02% | 0.06% | 0.02% | 0.04% | 0.11% | 0.00% | 0.03% | 0.08% | -0.06% | 0.00% | 0.04% |
| Method 3 | 0.00% | -0.01% | -0.02% | 0.00% | -0.06% | -0.03% | 0.01% | -0.07% | -0.05% | -0.02% | -0.04% | -0.13% |

# Introduction

In VVC draft 8, deblocking filter (DBF), sample adaptive offset (SAO) and adaptive loop filter (ALF) are three cascaded in-loop filters.

When applying DBF to horizontal boundaries of coding tree units (CTU), asymmetrical filtering may be applied as shown in Figure 1. For any chroma format, at most 3 bottom lines (lines D, E and F in Figure 1) of luma samples and 1 bottom line (line O in Figure 1) of chroma samples in the upper CTU (CTU0 in Figure 1) may be modified. However, DBF cannot be applied to horizontal CTU boundaries until availability of top lines of the below CTU (lines G, H, I etc. in CTU1 in Figure 1). In addition, SAO cannot be applied to line C and line N until DBF has been applied to lines D, E, F and O. Therefore, in VVC draft 8, the bottom 4 lines of luma samples and 2 lines of chroma samples of a CTU are required as line buffers which require the availability of below CTU. To reduce the line buffer requirement for ALF and cross component adaptive loop filter (CC-ALF, which can be regarded as a part of ALF in pipeline design), virtual boundary (VB) processing is used in VVC draft 8. The position of a VB is 4 lines of luma samples and 2 lines of chroma samples above a horizontal CTU boundary. When ALF or CC-ALF is applied to a sample above (below resp.) a VB, the samples below (above resp.) the VB cannot be utilized. Therefore, the samples above a VB (for example lines A, B, L and M in CTU0 in Figure 1) can finish all in-loop filtering without using samples from the below CTU (CTU1 in Figure 1). In another word, when in-loop filtering (DBF, SAO and ALF) is pending on lines C, D, E, F, N and O, in-loop filters are done with lines A, B, L and M in Figure 1.

In VVC draft 8, when CC-ALF is applied to a chroma sample, its co-located luma sample is found at first. Then a 4x3 diamond filter shape is applied to the luma samples surrounding this co-located luma sample. This process is described in Figure 2 and 3. (Note that the sample padding for the VB processing is not shown in Figure 2 and Figure 3.)

Figure 1. Virtual boundaries and CTU boundaries in VVC draft 8

Figure 2.  CC-ALF is applied to a chroma sample in a 4:2:0 sequence.

Figure 3. CC-ALF is applied to a chroma sample in a 4:2:2 or 4:4:4 sequence.

# Problem

In a 4:2:2 or 4:4:4 sequence, when CC-ALF is applied to a chroma sample above a VB. DBF and SAO of this chroma sample have finished. However, the co-located luma sample may be below the VB. As such, the 4x3 diamond filtering of CC-ALF cannot be applied to this co-located luma sample until its DBF and SAO finish. Therefore, additional lines of chroma samples above a VB should be stored as extra line buffers.

Figure 4. In a 4:2:2 or 4:4:4 sequence, CC-ALF is applied to chroma samples above a VB and their co-located luma samples are below the VB.

For example, as shown in Figure 4, in CTU0 of a 4:2:2 or 4:4:4 sequence, when CC-ALF is applied to a chroma sample in line L (M resp.), its co-located luma sample is in line C (D resp.). However, by this time, DBF and SAO of samples in line C (D resp.) may have not started since the below CTU (CTU1) is unavailable. Therefore, chroma samples in lines L and M will be stored as additional line buffers until DBF and SAO are done with luma samples in lines C and D.

# Solutions

To solve that problem, three methods are proposed.

## Method 1

Figure 5. In a 4:2:2 or 4:4:4 sequence, when CC-ALF is applied to two chroma samples above a VB, their co-located luma samples are below the VB. Their co-located luma samples are then changed into the closest ones above the VB, which are in line B.

In method 1, when CC-ALF is applied to a chroma sample above a VB and its co-located luma sample is below the VB, the position of the co-located luma sample is then changed into the nearest luma sample above the VB.

For example, in Figure 5, when CC-ALF is applied to chroma sample in line L (M resp.), its co-located luma sample is in line C (D resp.). Then the closest luma sample in line B will used as the new co-located luma sample.

## Method 2

In method 2, when CC-ALF is applied to a chroma sample above a VB and its co-located luma sample is below the VB, CC-ALF is not applied to this chroma sample.

## Method 3

In method 3, for 4:2:2 and 4:4:4 sequences, the position of a VB is changed into 4 chroma lines above a CTU boundary, as shown in Figure 6. Compared to VVC draft 8, line buffer increases by 2 lines or chroma samples.

Figure 6. In a 4:2:2 or 4:4:4 sequence, the position of a VB is changed into 4 chroma lines above a CTU boundary.

# Experimental Results

The proposed methods are implemented on top of VTM-8.0 and are tested under the test condition for 4:2:2 and 4:4:4 natural sequences defined in JVET-Q2013.

## Method 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.00% | 0.00% | 0.02% | 100% | 99% |
| YUV 4:2:2 | 0.00% | 0.00% | 0.02% | 100% | 100% |
| RGB 4:4:4 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.03% | -0.05% | 0.05% | 100% | 100% |
| YUV 4:2:2 | -0.01% | -0.03% | 0.02% | 100% | 101% |
| RGB 4:4:4 | 0.01% | -0.01% | -0.01% | 100% | 100% |
| Overall | 0.01% | -0.03% | 0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | -0.01% | -0.10% | -0.03% | 100% | 98% |
| YUV 4:2:2 | 0.02% | -0.08% | 0.04% | 100% | 101% |
| RGB 4:4:4 | 0.03% | -0.01% | 0.01% | 100% | 101% |
| Overall | 0.01% | -0.06% | 0.01% | 100% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | -0.03% | -0.01% | -0.17% | 100% | 102% |
| YUV 4:2:2 | -0.01% | -0.07% | 0.04% | 100% | 100% |
| RGB 4:4:4 | -0.03% | 0.01% | 0.01% | 99% | 98% |
| Overall | -0.02% | -0.02% | -0.04% | 100% | 100% |

## Method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.00% | 0.01% | 0.08% | 100% | 99% |
| YUV 4:2:2 | 0.00% | 0.04% | 0.09% | 100% | 100% |
| RGB 4:4:4 | 0.00% | 0.00% | 0.01% | 100% | 99% |
| Overall | 0.00% | 0.02% | 0.06% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.05% | 0.01% | 0.17% | 100% | 101% |
| YUV 4:2:2 | 0.00% | 0.09% | 0.15% | 100% | 100% |
| RGB 4:4:4 | 0.01% | 0.01% | 0.02% | 100% | 100% |
| Overall | 0.02% | 0.04% | 0.11% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | -0.01% | -0.06% | 0.02% | 100% | 100% |
| YUV 4:2:2 | 0.01% | 0.16% | 0.16% | 100% | 101% |
| RGB 4:4:4 | 0.00% | 0.00% | 0.06% | 100% | 100% |
| Overall | 0.00% | 0.03% | 0.08% | 100% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | -0.06% | -0.08% | 0.01% | 100% | 100% |
| YUV 4:2:2 | -0.01% | 0.07% | 0.10% | 100% | 100% |
| RGB 4:4:4 | -0.10% | 0.02% | 0.00% | 100% | 99% |
| Overall | -0.06% | 0.00% | 0.04% | 100% | 99% |

## Method 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.00% | -0.02% | -0.04% | 100% | 100% |
| YUV 4:2:2 | 0.00% | 0.00% | -0.02% | 100% | 98% |
| RGB 4:4:4 | 0.00% | -0.01% | 0.00% | 100% | 99% |
| Overall | 0.00% | -0.01% | -0.02% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.01% | -0.06% | 0.01% | 100% | 101% |
| YUV 4:2:2 | 0.01% | -0.11% | -0.08% | 100% | 101% |
| RGB 4:4:4 | -0.02% | -0.02% | -0.02% | 100% | 100% |
| Overall | 0.00% | -0.06% | -0.03% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.04% | -0.04% | -0.16% | 100% | 100% |
| YUV 4:2:2 | 0.00% | -0.15% | 0.01% | 100% | 101% |
| RGB 4:4:4 | -0.01% | -0.03% | 0.00% | 100% | 101% |
| Overall | 0.01% | -0.07% | -0.05% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | -0.01% | -0.02% | -0.27% | 100% | 100% |
| YUV 4:2:2 | 0.02% | -0.05% | -0.05% | 100% | 101% |
| RGB 4:4:4 | -0.06% | -0.04% | -0.07% | 100% | 100% |
| Overall | -0.02% | -0.04% | -0.13% | 100% | 101% |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).