| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0235-v1 |
| --- | --- |

| Title: | Removal of LFNST for chroma components |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Alican Nalci, Vadim Seregin, Wei-Jung Chien, Ted Hsieh, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Email: | hegilmez@qti.qualcomm.com analci@qti.qualcomm.com vseregin@qti.qualcomm.com wchien@qti.qualcomm.com thsieh@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In VVC draft 8, LFNST is disabled for chroma components in the single-tree partition mode and only applied for luma components. This contribution proposes to further disable LFNST for chroma in dual-tree partition mode. Thus, the LFNST process for chroma components is completely removed. The experimental results under common test conditions (CTC) show that the proposed simplification leads to average BD-rates of “Y:0.12%, U:1.14%, V:1.37%” in AI, and “Y:0.07%, U: 0.28%, V: 0.37%” in RA configurations. Disabling LFNST for chroma also lead to about 6% encoder speed-up in AI configurations. Additional experimental results with a slightly different encoder setting show that the BD-rate losses can be reduced to “Y:0.02%, U:0.81%, V:1.01%” in AI and “Y:0.02%, U:0.15%, V:0.24%” in RA configurations with similar average encoding runtimes as compared to the VTM-8.0 anchor.

# Introduction

In the VVC specification working draft (JVET-Q2001-vE), the LFNST index (lfnst_idx) is signalled after residual coding of all color components based on the following set of conditions:

| LfnstDcOnly = 1 |  |
| --- | --- |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsDcOnly = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| … |  |
| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |

| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| --- | --- |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |

where LFNST index (lfnst_idx) signalled in single-tree partition mode is only used determine the LFNST applied for luma component only, and the LFNST process is skipped for chroma components. This is reflected in JVET-Q2001-vE by using ApplyLfnstFlag variable as follows:

The variable ApplyLfnstFlag is derived as follows:

- If treeType is equal to SINGLE_TREE, the following applies:

ApplyLfnstFlag = ( lfnst_idx > 0 && cIdx  = =  0 ) ? 1 : 0	(176)

- Otherwise, the following applies:

ApplyLfnstFlag = ( lfnst_idx > 0 ) ? 1 : 0		(177)

# Proposed Method

This contribution proposes to disable the LFNST process for chroma in dual-tree (separate-tree) partitioning mode. In other words, the LFNST process for chroma components are completely removed and is only applied to luma blocks. Thus, the proposal provides a unified design with the single-tree partition mode and reduces the latency due to the additional stage of LNST processing for chroma components.

The proposed changes are reflected on top of the VVC specification working draft (JVET-Q2001-vE) as follows:

| LfnstDcOnly = 1 |  |
| --- | --- |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsDcOnly = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |

| if( treeType != DUAL_TREE_CHROMA && Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| --- | --- |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && MtsDcOnly = = 0 ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |

…

The variable ApplyLfnstFlag is derived as follows:

- If treeType is equal to SINGLE_TREE, the following applies:

ApplyLfnstFlag = ( lfnst_idx > 0 && cIdx  = =  0 ) ? 1 : 0	(176)

- Otherwise, the following applies:

ApplyLfnstFlag = ( lfnst_idx > 0 ) ? 1 : 0		(177)

…

# Experimental Results

The proposed method is implemented on top of VTM-8.0 and evaluated under common test conditions (CTC). Note that low-delay B results are omitted, since LFNST is disabled in CTC for low-delay B.

Table 1 shows the results of proposed simplification without any encoder changes.

Table 2 presents the results of proposed normative changes with the following encoder modifications:

- The maximum “TT” partition for chroma I-slices is set to 64 (i.e., MaxTTChromaISlice=64).

- The g_aucIntraModeNumFast_UseMPM_2D table is modified as:

const uint8_t g_aucIntraModeNumFast_UseMPM_2D[7 - MIN_CU_LOG2 + 1][7 - MIN_CU_LOG2 + 1] =

{

{5, 5, 4, 3, 2, 2},  //   4x4,   4x8,   4x16,   4x32,   4x64,   4x128,

{5, 4, 4, 3, 3, 2},  //   8x4,   8x8,   8x16,   8x32,   8x64,   8x128,

{4, 4, 4, 3, 3, 2},  //  16x4,  16x8,  16x16,  16x32,  16x64,  16x128,

{3, 3, 3, 3, 3, 2},  //  32x4,  32x8,  32x16,  32x32,  32x64,  32x128,

{2, 3, 3, 3, 3, 2},  //  64x4,  64x8,  64x16,  64x32,  64x64,  64x128,

{2, 2, 2, 2, 2, 3},  // 128x4, 128x8, 128x16, 128x32, 128x64, 128x128,

};

Table 1.  Results of the proposed method over VTM-8.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 1.56% | 1.97% | 95% | 100% |
| Class A2 | -0.04% | 1.58% | 1.60% | 91% | 99% |
| Class B | 0.12% | 1.05% | 1.47% | 96% | 102% |
| Class C | 0.25% | 0.66% | 0.92% | 92% | 99% |
| Class E | 0.22% | 1.08% | 0.98% | 94% | 99% |
| Overall | 0.12% | 1.14% | 1.37% | 94% | 100% |
| Class D | 0.26% | 0.86% | 0.98% | 94% | 103% |
| Class F | 0.23% | 0.54% | 0.49% | 97% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.48% | 0.52% | 94% | 94% |
| Class A2 | 0.02% | 0.34% | 0.61% | 94% | 95% |
| Class B | 0.05% | 0.61% | 0.81% | 98% | 96% |
| Class C | 0.16% | -0.34% | -0.45% | 98% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.28% | 0.37% | 96% | 95% |
| Class D | 0.18% | 0.61% | 0.66% | 100% | 101% |
| Class F | 0.23% | -0.16% | 0.17% | 100% | 100% |

Table 2.  Results of the proposed method with additional encoder only changes over VTM-8.0 under CTC.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 1.38% | 1.82% | 101% | 98% |
| Class A2 | -0.13% | 1.10% | 1.19% | 99% | 96% |
| Class B | 0.03% | 0.70% | 1.01% | 104% | 99% |
| Class C | 0.12% | 0.54% | 0.74% | 101% | 93% |
| Class E | 0.04% | 0.52% | 0.38% | 101% | 93% |
| Overall | 0.02% | 0.81% | 1.01% | 101% | 96% |
| Class D | 0.10% | 0.68% | 0.81% | 102% | 100% |
| Class F | 0.06% | 0.24% | 0.27% | 101% | 96% |
|  | Random Access |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.38% | 0.48% | 100% | 100% |
| Class A2 | -0.03% | 0.34% | 0.49% | 96% | 95% |
| Class B | 0.01% | 0.31% | 0.55% | 101% | 100% |
| Class C | 0.11% | -0.38% | -0.52% | 98% | 92% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.15% | 0.24% | 99% | 97% |
| Class D | 0.17% | 0.47% | 0.37% | 102% | 101% |
| Class F | 0.12% | -0.45% | 0.00% | 102% | 102% |

# Conclusions

This contribution proposes to remove LFNST for chroma components in dual-tree partitioning mode in order to provide a unified design with the single-tree partitioning mode as well as to reduce the latency introduced by the additional stage of LFNST. Since the additional gain of applying LFNST for chroma is limited and requires about 6% more encoder run-time, it is recommended to adopt the proposed method in VVC.

The file “JVET-Q2001-vE_JVET_R0235-spec.docx” attached to this submission includes the proposed changes over the VVC specification working draft (JVET-Q2001-vE).

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” JVET-Q2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 Jan. 2020.

- H.E. Egilmez, A. Nalci, M. Coban, V. Seregin, M. Karczewicz, “Chroma LFNST Simplification and Signaling,” JVET-Q0686, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 Jan. 2020.

- H.E. Egilmez, A. Nalci, A.K. Ramasubramonian, M. Coban, V. Seregin, M. Karczewicz, T. Hashimoto, T. Chujoh, E. Sasaki, T. Ikai, “Combination of JVET-Q0106, JVET-Q0686 and JVET-Q0133 for LFNST Signaling, Latency Reduction and Scaling Process,” JVET-Q0784, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 Jan. 2020.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).