| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0238 |
| --- | --- |

| Title: | AHG12: A fix on subpicture-level slice indexing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Li Zhang1, Ye-Kui Wang1 Zhipin Deng2, Kui Fan1, Jizheng Xu1 Hongbin Liu2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the current VVC text, to derive the value of the variable NumCtusInCurrSlice, which relies on a conversion from the subpicture-level slice index to the picture-level slice index of a slice, it is assumed that the order of increasing picture-level slice index values is the same as the decoding order of slices. However, when some slices are resulted from splitting one tile, it is claimed that the assumption may be violated. Consequenlty, the parsing of slice data would be problematic and the decoded sample values won't be correct.

It is proposed to fix the problem by deriving a mapping between the picture-level slice index and the subpicture-level slice index and using the mapping in the derivation of the value of the variable NumCtusInCurrSlice.

# Problem Statement

In the current VVC text, when rect_slice_flag is equal to 1, a slice may be referenced by either a subpicture-level index or a picture-level index, which are defined as follows:

- A subpicture-level slice index is the index of a slice in the list of slices belonging to a subpicture in the order as they are signalled in the PPS.

- A picture-level slice index is the index of a slice in the list of slices belonging to a picture in the order as they are signalled in the PPS.

When signalling a slice, slice_address represents the subpicture-level slice index of the slice. For any two slices belonging to two different subpictures, the one associated with the less subpicture index must be signalled earlier in decoding order; and for any two slices belonging to the same subpicture, the one with the less subpicture-level slice index must be signalled earlier in decoding order.

The variable NumCtusInCurrSlice, which specifies the number of CTUs in the current slice, is derived by Equation 116 in the current VVC text as follows:

for( j = 0; j < CurrSubpicIdx; j++ )
		picLevelSliceIdx  +=  NumSlicesInSubpic[ j ]
	NumCtusInCurrSlice = NumCtusInSlice[ picLevelSliceIdx ]

In the equation, it is assumed that the order of increasing picture-level slice index values is the same as the decoding order of slices, so that picLevelSliceIdx is equal to the picture-level slice index.

However, when some slices are resulted from splitting one tile, some of the above aspects may be violated. As an example shown in Figure 6, when a picture is partitioned into two tiles by a vertical tile boundary, and each of the two tiles is partitioned into two slices by the same horizontal boundary across the entire picture, and the upper two slices are included into the first subpicture and the lower two slices are included into the second subpicture. In this case, according to the current VVC text, the picture-level slice index values of the four slices in slice raster scan order would be 0, 2, 1, 3, while the decoding order index values of the four slices in slice raster scan order would be 0, 1, 2, 3. Consequently, the derivation of NumCtusInCurrSlice would be incorrect, and in turn the parsing of slice data would be problematic, the decoded sample values won't be correct.

Figure 1. An example of picture partitioning. Blue lines represent boundaies of tiles; green lines represent boudnaries of slices and Red dashed lines represet boundaries of subpictures. The picture-level indices, decoding order indices, subpicture-level indices of the four slices and the indices of subpictures and tiles are indicated in the figure.

# Proposed solution

It is proposed to fix the problem by deriving a mapping between the picture-level slice index and the subpicture-level slice index and using the mapping in the derivation of the NumCtusInCurrSlice variable.

The proposed text changes based on JVET-Q2001-vE are as follows, with the deleted text and newly added text highlighted in turquois and yellow, respectively.

### 3 Definitions

subpicture-level slice index: An index, defined when rect_slice_flag is equal to 1, of a slice to the list of slices in a subpicture in the order as they are signalled in the PPS.

subpicture-level slice index: The index of a slice to the list of slices in a subpicture in their decoding order when the rect_slice_flag is equal to 1.

### 6.5.1 CTB raster scanning, tile scanning, and subpicture scanning processes

…

The list NumSlicesInSubpic[ i ], specifying the number of rectangular slices in the i-th subpicture,the variable NumSlicesInSubpic[ i ], specifying the number of rectangular slices in the i-th subpicture, the variable SubpicIdxOfSlice[ j ], specifying the subpicture index of the slice with picture-level slice index j, the variable SubpicLevelSliceIdx[ j ], specifying the subpicture-level slice index of the slice with picture-level slice index j, and the variable PicLevelSliceIdx[ m ][ n ], specfiying the picture-level slice index for the slice with subpicture-level slice index n in the m-th subpicture, are is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ ) {
	NumSlicesInSubpic[ i ] = 0
	for( j = 0; j  <=  num_slices_in_pic_minus1; j++ ) {
		posX = CtbAddrInSlice[ j ][ 0 ] % PicWidthInCtbsY
		posY = CtbAddrInSlice[ j ][ 0 ] / PicWidthInCtbsY
		if( ( posX  >=  subpic_ctu_top_left_x[ i ] )  &&	(32)
				( posX < subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1 )  &&
				( posY  >=  subpic_ctu_top_left_y[ i ] )  &&
				( posY < subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1 ) )					SubpicIdxOfSlice[ j ] = i
			SubpicLevelSliceIdx[ j ] = NumSlicesInSubpic[ i ]
			PicLevelSliceIdx[ i ][ SubpicLevelSliceIdx[ j ] ] = j
			NumSlicesInSubpic[ i ]++
}

…

### 7.4.8.1 General slice header semantics

...

slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0.

If rect_slice_flag is equal to 0, the following applies:

- The slice address is the raster scan tile index of the first tile in the slice.

- The length of slice_address is Ceil( Log2 ( NumTilesInPic ) ) bits.

- The value of slice_address shall be in the range of 0 to NumTilesInPic − 1, inclusive.

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the subpicture-level slice index of the slice as specified by the variable SubpicLevelSliceIdx[ i ] derived in Equation 32, where i is the picture-level slice index of the slice.

- The length of slice_address is Ceil( Log2( NumSlicesInSubpic[ CurrSubpicIdx ] ) ) bits.

- The value of slice_address shall be in the range of 0 to NumSlicesInSubpic[ CurrSubpicIdx ] − 1, inclusive.

It is a requirement of bitstream conformance that the following constraints apply:

- If rect_slice_flag is equal to 0 or subpic_info_present_flag is equal to 0, the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

- Otherwise, the pair of slice_subpic_id and slice_address values shall not be equal to the pair of slice_subpic_id and slice_address values of any other coded slice NAL unit of the same coded picture.

- The shapes of the slices of a picture shall be such that each CTU, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded CTU(s).

...

num_tiles_in_slice_minus1 plus 1, when present, specifies the number of tiles in the slice. The value of num_tiles_in_slice_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive.

The variable NumCtusInCurrSlice, which specifies the number of CTUs in the current slice, and the list CtbAddrInCurrSlice[ i ], for i ranging from 0 to NumCtusInCurrSlice − 1, inclusive, specifying the picture raster scan address of the i-th CTB within the slice, are derived as follows:

if( rect_slice_flag ) {
	picLevelSliceIdx = PicLevelSliceIdx[ CurrSubpicIdx ][ slice_address ]
	for( j = 0; j < CurrSubpicIdx; j++ )
		picLevelSliceIdx  +=  NumSlicesInSubpic[ j ]
	NumCtusInCurrSlice = NumCtusInSlice[ picLevelSliceIdx ]
	for( i = 0; i < NumCtusInCurrSlice; i++ )
		CtbAddrInCurrSlice[ i ] = CtbAddrInSlice[ picLevelSliceIdx ][ i ]	(116)
} else {
	NumCtusInCurrSlice = 0
	for( tileIdx = slice_address; tileIdx  <=  slice_address + num_tiles_in_slice_minus1; tileIdx++ ) {
		tileX = tileIdx % NumTileColumns
		tileY = tileIdx / NumTileColumns
		for( ctbY = tileRowBd[ tileY ]; ctbY < tileRowBd[ tileY + 1 ]; ctbY++ ) {
			for( ctbX = tileColBd[ tileX ]; ctbX < tileColBd[ tileX + 1 ]; ctbX++ ) {
				CtbAddrInCurrSlice[ NumCtusInCurrSlice ] = ctbY * PicWidthInCtb + ctbX
				NumCtusInCurrSlice++
			}
		}
	}
}

…

# Conclusion

In this contribution, it is proposed to fix a bug in the process of slice partitioning in the VVC text, by deriving a mapping between the picture-level slice index and the subpicture-level slice index and using the mapping in the derivation of the value of the variable NumCtusInCurrSlice. It is recommended to adopt the bug fix.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.