| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0239 |
| --- | --- |

| Title: | AHG9: Cleanups on signalling of tiles, slices, and subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Li Zhang1, Ye-Kui Wang1, Zhipin Deng2, Jizheng Xu1, Hongbin Liu2 18910 University Center Lane San Diego, CA, U.S.A. 92122 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes regarding the signalling of tiles, slices, and subpictures:

- The semantics of tile_column_width_minus1[ i ] is clarified to specify the width of the i-th tile column only when num_exp_tile_columns_minus1 is greater than 0. A simimar modification is also applied to tile_row_height_minus1[ i ].

- When the picture width is less than or equal to one CTB size, num_exp_tile_columns_minus1 is not signalled and inferred to be 0. Likewise, when the picture height is less than or equal to one CTB size, num_exp_tile_rows_minus1 is not signalled and inferred to be 0.

- When num_exp_tile_columns_minus1 is equal to PicWidthInCtbsY − 1, tile_column_width_minus1[ i ] is not signalled and inferred to be 0. Likewise, when num_exp_tile_rows_minus1 is equal to PicHeightInCtbsY − 1, tile_row_height_minus1[ i ] is not signalled and inferred to be 0.

- The semantis of exp_slice_height_in_ctus_minus1[ i ] is clarified that exp_slice_height_in_ctus_minus1[ i ][ num_exp_slices_in_tile[ i ] − 1 ] is used to derive the uniform slice height, instead of specifying the height of the (num_exp_slices_in_tile[ i ] − 1) -th slice.

- When the maximum picture width and height are both less than or equal to one CTB size, sps_num_subpics_minus1 is not signaled and inferred to be 0.

# Problem statement

In the current VVC text, there are the following problems in the syntax/semantics related to tile/slice/subpicture partitioning:

- It is declared that “tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs for i in the range of 0 to num_exp_tile_columns_minus1 − 1, inclusive.” However, num_exp_tile_columns_minus1 may be equal to 0, which cuase contradictions. The same issue also applies to tile_row_height_minus1[ i ].

- If the picture width is smaller than or equal to one CTB size, the signaling of num_exp_tile_columns_minus1 is unnecessary. Likewise, if the picture height is smaller than or equal to one CTB size, the signaling of num_exp_tile_rows_minus1 is unnecessary.

- If num_exp_tile_columns_minus1 is equal to PicWidthInCtbsY − 1, tile_column_width_minus1[ i ] must be equal to 0 and the signaling is unnecessary. The same issue also applies to tile_row_height_minus1[ i ].

- It is declared that “exp_slice_height_in_ctus_minus1[ i ][ j ] plus 1 specifies the height of the j-th rectangular slice in the tile containing the i-th slice in units of CTU rows.” However, in section 6.5.1, exp_slice_height_in_ctus_minus1[ i ][ num_exp_slices_in_tile[ i ] − 1 ] is used to derive the uniform slice height, instead of specifying the height of the (num_exp_slices_in_tile[ i ] − 1) -th slice.

- When the maximum picture width and height are both less than or equal to one CTB size, signalling of sps_num_subpics_minus1 is unnecessary.

# Proposed solution

To address the problems, Four aspects are proposed.

- The text is modified as “tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs for i in the range of 0 to num_exp_tile_columns_minus1 − 1, inclusive, when num_exp_tile_columns_minus1 is greater than 0. A simimar modification is also applied to tile_row_height_minus1[ i ].

- If the picture width is smaller than or equal to one CTB size, num_exp_tile_columns_minus1 is not signaled and inferred to be 0. Likewise, if the picture height is less than or equal to one CTB size, num_exp_tile_rows_minus1 is not signaled and inferred to be 0.

- If num_exp_tile_columns_minus1 is equal to PicWidthInCtbsY − 1, tile_column_width_minus1[ i ] is not signaled and inferred to be 0. Likewise, if num_exp_tile_rows_minus1 is equal to PicHeightInCtbsY – 1, tile_row_height_minus1[ i ] is not signaled and inferred to be 0.

- The text is modified to clarify that exp_slice_height_in_ctus_minus1[ i ][ num_exp_slices_in_tile[ i ] − 1 ] is used to derive the uniform slice height, instead of specifying the height of the (num_exp_slices_in_tile[ i ] − 1) -th slice.

- When the maximum picture width and height are both less than or equal to one CTB size, sps_num_subpics_minus1 is not signaled and inferred to be 0.

The working draft changes based on JVET-Q2001-vE are as follows, with the deleted text and newly added text highlighted in turquois and yellow, respectively.

Aspect 1)

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs for i in the range of 0 to num_exp_tile_columns_minus1 – 1, inclusive, when num_exp_tile_columns_minus1 is greater than 0. tile_column_width_minus1[ num_exp_tile_columns_minus1 ] is used to derive the width of the tile columns with index greater than or equal to num_exp_tile_columns_minus1 as specified in clause 6.5.1. The value of tile_column_width_minus1[ i ] shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of tile_column_width_minus1[ 0 ] is inferred to be equal to PicWidthInCtbsY − 1.

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs for i in the range of 0 to num_exp_tile_rows_minus1 − 1, inclusive, when num_exp_tile_rows_minus1 is greater than 0. tile_row_height_minus1[ num_exp_tile_rows_minus1 ] is used to derive the height of the tile rows with index greater than or equal to num_exp_tile_rows_minus1 as specified in clause 6.5.1. The value of tile_row_height_minus1[ i ] shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of tile_row_height_minus1[ 0 ] is inferred to be equal to PicHeightInCtbsY − 1.

Aspect 2)

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … |  |
| if( pic_width_in_luma_samples > CtbSizeY ) |  |
| num_exp_tile_columns_minus1 | ue(v) |
| if( pic_heigh_in_luma_samples > CtbSizeY ) |  |
| num_exp_tile_rows_minus1 | ue(v) |
| … |  |

num_exp_tile_columns_minus1 plus 1 specifies the number of explicitly provided tile column widths. The value of num_exp_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present no_pic_partition_flag is equal to 1, the value of num_exp_tile_columns_minus1 is inferred to be equal to 0.

num_exp_tile_rows_minus1 plus 1 specifies the number of explicitly provided tile row heights. The value of num_exp_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present no_pic_partition_flag is equal to 1, the value of num_exp_tile_rows_minus1 is inferred to be equal to 0.

Aspect 3)

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … |  |
| for( i = 0; i <= num_exp_tile_columns_minus1 && num_exp_tile_columns_minus1 != PicWidthInCtbsY − 1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1 && num_exp_tile_rows_minus1 != PicHeightInCtbsY − 1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| … |  |

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs for i in the range of 0 to num_exp_tile_columns_minus1 − 1, inclusive. tile_column_width_minus1[ num_exp_tile_columns_minus1 ] is used to derive the width of the tile columns with index greater than or equal to num_exp_tile_columns_minus1 as specified in clause 6.5.1. The value of tile_column_width_minus1[ i ] shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of tile_column_width_minus1[ i0 ] is inferred to be equal to no_pic_partition_flag ? PicWidthInCtbsY − 1 : 0 for i in the range of 0 to Max( 0, num_exp_tile_columns_minus1 − 1 ), inclusive.

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs for i in the range of 0 to num_exp_tile_rows_minus1 − 1, inclusive. tile_row_height_minus1[ num_exp_tile_rows_minus1 ] is used to derive the height of the tile rows with index greater than or equal to num_exp_tile_rows_minus1 as specified in clause 6.5.1. The value of tile_row_height_minus1[ i ] shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of tile_row_height_minus1[ i0 ] is inferred to be equal to no_pic_partition_flag ? PicHeightInCtbsY − 1 : 0 for i in the range of 0 to Max( 0, num_exp_tile_rows_minus1 − 1 ), inclusive.

Aspect 4)

exp_slice_height_in_ctus_minus1[ i ][ j ] plus 1 specifies the height of the j-th rectangular slice in the tile containing the i-th slice in units of CTU rows, for j in the range of 0 to num_exp_slices_in_tile[ i ] − 1, inclusive, when num_exp_slices_in_tile[ i ] is greater than 0. exp_slice_height_in_ctus_minus1[ i ][ num_exp_slices_in_tile[ i ]] is used to derive the height of the k-th rectangular slice with k greater than or equal to num_exp_slices_in_tile[ i ] in the tile containing the i-th slice in units of CTU rows, as specified in clause 6.5.1. The value of exp_slice_height_in_ctus_minus1[ i ][ j ] shall be in the range of 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 1, inclusive.

Aspect 5)

#### 7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( subpic_info_present_flag ) { |  |
| if( pic_width_max_in_luma_samples > CtbSizeY ¦ ¦ pic_height_max_in_luma_samples > CtbSizeY ) |  |
| sps_num_subpics_minus1 | ue(v) |
| ... |  |
| } |  |

# Conclusion

In this contribution, five aspects are proposed to make the VVC text clearer. It is recommended to adopt them into the next version of the VVC text.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.