| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0242-v3 |
| --- | --- |

| Title: | AHG9/AHG12: Decoded subpicture hash SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Lidong Xu | Email: | jill.boyce@intel.com lidong.xu@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

A decoded subpicture hash SEI message is proposed. It is similar in spirit to the decoded picture hash SEI message but signals hash values for each subpicture individually.

The -v3 version includes a software implementation of the proposed SEI message, with macro JVET_R0242_DECODED_SUBPICTURE_HASH_SEI.

# Proposal

The SEI specification in JVET-Q2007 includes a decoded picture hash SEI message, which has the same syntax as for HEVC and updated semantics reflecting the separation of the SEI spec from the main VVC specification.

A similar SEI message is proposed to signal hash values per subpicture. It is asserted that providing such an SEI message will assist in the implementation of bitstream extraction and mergers (BEAMers) and decoders operating on extracted bitstreams to confirm that they are handling subpictures properly.

If a bitstream has undergone a bitstream extraction process in which some subpictures are retained and some subpictures are removed, the hash values in the existing decoded picture hash SEI message will not be correct for the extracted bitstream. A decoder would be able to utilize the proposed SEI message to check the hash values for the retained sub-pictures.

In the proposed syntax, the number of subpictures for which hash values are present is explicitly signalled in the SEI message. No requirement is imposed that the SEI message number of subpictures matches that in the bitstream.  The subpicture ID may optionally be explicitly signalled.

A bitstream extractor would not be required to modify the contents of the SEI message when performing bitstream extraction to remove some subpictures, although it may choose to.

## Decoded subpicture hash SEI message

### Decoded subpicture hash SEI message syntax

| decoded_subpicture_hash( payloadSize ) { | Descriptor |
| --- | --- |
| dsh_hash_type | u(2) |
| dsh_num_subpics_minus1 | ue(v) |
| dsh_subpic_id_explicitly_signalled_flag | u(1) |
| if( dsh_subpic_id_explicitly_signalled_flag ) { |  |
| dsh_subpic_id_len_minus1 | ue(v) |
| for( s = 0; s <= dsh_num_subpics_minus1; s++ ) |  |
| dsh_subpic_id[ s ] | u(v) |
| } |  |
| byte_align( ) |  |
| for( s = 0; s <= dsh_num_subpics_minus1; s++ ) |  |
| for( cIdx = 0; cIdx < ( ChromaFormatIdc = = 0 ? 1 : 3 ); cIdx++ ) |  |
| if( hash_type = = 0 ) |  |
| for( i = 0; i < 16; i++) |  |
| dsh_subpicture_md5[ s ] [ cIdx ][ i ] | b(8) |
| else if( hash_type = = 1 ) |  |
| dsh_subpicture_crc[ s ] [ cIdx ] | u(16) |
| else if( hash_type = = 2 ) |  |
| dsh_subpicture_checksum[ s ] [ cIdx ] | u(32) |
| } |  |

### Decoded picture hash SEI message semantics

This message provides a hash for each colour component of each subpicture of the current decoded picture.

Use of this SEI message requires the definition of the following variables:

–	A subpicture width and subpicture height in units of luma samples, for each subpicture s, denoted herein by SubPicWidthInLumaSamples[ s ] and SubPicHeightInLumaSamples[ s ], respectively.

–	A CTU size, CtuSize

–	A subpicture top left vertical and horizontal position in units of CTUs, for each subpicture s, denoted herein by SubPicCtuTopLeftY[ s ] and SubPicCtuTopLeftX[ s ], respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.2.

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY, and when ChromaFormatIdc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein by BitDepthC.

–	For each colour component cIdx, a two-dimensional array ComponentSamples[ cIdx ][ y ][ x ] of decoded picture sample values in unsigned integer representation.

Prior to computing the hash, the decoded subpicture data are arranged into one or three strings of bytes called subpictureData[ s ][ cIdx ] of lengths dataLen[ s ][ cIdx ] as follows:

for( s= 0; s < dsh_num_subpics_minus1; s++ ) {
	for( cIdx = 0; cIdx < ( ChromaFormatIdc  = =  0 ) ? 1 : 3; cIdx++ ) {
		if( cIdx  = =  0 ) {

compSubpicWidth[ s ][ cIdx ] = SubPicWidthInLumaSamples
			compSubpicHeight[ s ][ cIdx ] = SubPicHeightInLumaSamples

compPosX[ s ][ cIdx ] = SubPicCtuTopLeftX[ s ] * CtuSize
			compPosY[ s ][ cIdx ] = SubPicCtuTopLeftY[ s ] * CtuSize
			compDepth[ s ] [ cIdx ] = BitDepthY
		} else {
			compSubpicWidth[ s ] [ cIdx ] = SubPicWidthInLumaSamples / SubWidthC
			compSubpicHeight[ s ] [ cIdx ] = SubPicHeightInLumaSamples / SubHeightC

compPosX[ s ][ cIdx ] = SubPicCtuTopLeftX[ s ] * CtuSize / SubWidthC
			compPosY[ s ][ cIdx ] = SubPicCtuTopLeftY[ s ] * CtuSize / SubHeightC
			compDepth[ s ] [ cIdx ] = BitDepthC	
		}
		iLen = 0
		for( y = CompPosY[ s ][ cIdx ]; y < CompPosY[ s ][ cIdx ] + compSubpicHeight [ s ] [ cIdx ] ; y++) {

for (x = CompPosX[ s ][ cIdx ]; x < CompPosX[ s ][ cIdx ] + compSubpicWidth [ s ][ cIdx ]; x++ ) {
				subpictureData[ s ] [ cIdx ][ iLen++ ] = ComponentSamples[ cIdx ][ y ][ x ] & 0xFF
				if( compDepth[ cIdx ] > 8 )
					subpictureData[ s ] [ cIdx ][ iLen++ ] = ComponentSamples[ cIdx ][ y ][ x ] >>  8

}
}
dataLen[ s ] [ cIdx ] = iLen

}

}

.

dsh_hash_type indicates the method used to calculate the checksum as specified in Table 9 . Values of dsh_hash_type that are not listed in in Table 9  are reserved for future use by ITU-T | ISO/IEC and shall not be present in payload data conforming to this version of this Specification. Decoders shall ignore decoded subpicture hash SEI messages that contain reserved values of dsh_hash_type.

Table 9 –  Interpretation of dsh_hash_type

| dsh_hash_type | Method |
| --- | --- |
| 0 | MD5 (IETF RFC 1321) |
| 1 | CRC |
| 2 | Checksum |

dsh_num_subpics_minus1 + 1 specifies the number subpictures for which hash values will be signalled.

dsh_subpic_id_explicitly_signalled_flag equal to 1 specifies that the dsh_subpic_id[ s] syntax element will be present. dsh_subpic_id_explicitly_signalled_flag equal to 0 specifies that the dsh_subpic_id[ s] syntax element will not be present.

dsh_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element dsh_subpic_id[ i ]. The value of dsh_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive. The value of 1  <<  ( dsh_subpic_id_len_minus1 + 1 ) shall be greater than or equal to dsh_num_subpics_minus1 + 1.

dsh_subpic_id[ s ] specifies the subpicture ID of the s-th subpicture. The length of the dsh_subpic_id[ s ] syntax element is dsh_subpic_id_len_minus1 + 1 bits. When not present, the value of dsh_subpic_id[ s ] is inferred to be equal to s.

dsh_subpicture_md5[ s ] [ cIdx ][ i ] is the 16-byte MD5 hash of the cIdx-th colour component of the s-th decoded subpicture. The value of dsh_subpicture_md5[ s ] [ cIdx ][ i ] shall be equal to the value of digestVal[ s ] [ cIdx ] obtained as follows, using the MD5 functions defined in IETF RFC 1321:

MD5Init( context )
MD5Update( context, subpictureData[ s ] [ cIdx ], dataLen[ s ] [ cIdx ] )	(XX)
MD5Final( digestVal[ s ] [ cIdx ], context )

dsh_subpicture_crc[ s ] [ cIdx ] is the cyclic redundancy check (CRC) of the colour component cIdx of the s-th decoded subpicture. The value of dsh_subpicture_crc[ s ] [ cIdx ] shall be equal to the value of crcVal[s][ cIdx ] obtained as follows:

crc = 0xFFFF
subpictureData[ s ] [ cIdx ][  dataLen[ s ] [ cIdx ] ] = 0
subpictureData[ s ] [ cIdx ][  dataLe[ s ] n[ cIdx ] + 1 ] = 0
for( bitIdx = 0; bitIdx < ( dataLen[ s ] [ cIdx ]  + 2 ) * 8; bitIdx++ ) {	(XX)
	dataByte = subpictureData[ s ] [ cIdx ][ bitIdx  >>  3 ]
	crcMsb = ( crc  >>  15 ) & 1
	bitVal = ( dataByte  >>  ( 7 − ( bitIdx & 7 ) ) ) & 1
	crc = ( ( ( crc  <<  1 ) + bitVal ) & 0xFFFF ) ^ ( crcMsb * 0x1021 )
}
crcVal[s][ cIdx ] = crc

NOTE – The same CRC specification is found in Rec. ITU-T H.271.

dsh_subpicture_checksum[ s ] [ cIdx ] is the checksum of the colour component cIdx of the s-th decoded subpicture. The value of dsh_subpicture_checksum[ s ] [ cIdx ] shall be equal to the value of checksumVal[s][ cIdx ] obtained as follows:

sum = 0
for( y = compPosY[ s ][ cIdx ]; y < compPosY[ s ][ cIdx ] + compSubpicHeight[s][ cIdx ]; y++ )
	for( x = compPosX[ s ][ cIdx ]; x < compPosX[ s ][ cIdx ] + compSubpicWidth[s][ cIdx ]; x++ ) {
		xorMask = ( x & 0xFF ) ^ ( y & 0xFF ) ^ ( x  >>  8 ) ^ ( y  >>  8 )
		sum = ( sum + ( ( ComponentSamples [ cIdx ][ y ][x ] & 0xFF ) ^	(XX)
				xorMask ) ) & 0xFFFFFFFF
		if( compDepth[ cIdx ] > 8 )
			sum = ( sum + ( ( ComponentSamples [ cIdx ][ y ][ x ]  >>  8 ) ^
					xorMask ) ) & 0xFFFFFFFF
	}
checksumVal[s][ cIdx ] = sum

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).