| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0243 |
| --- | --- |

| Title: | AHG9: 4:4:4 vs. 4:2:0 bit-rate in VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Steve Keating Karl Sharman | Email: | steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

This contribution compares bit-rates for 4:4:4 and 4:2:0 encoding. It is for information/discussion only.

# Introduction

Another contribution, JVET-R0244, proposes changes to the CpbVclFactor and MinCrScaleFactor for Main 4:4:4 10 profile.  This contribution specifies that the maximum bit-rate of 4:4:4 should be twice the maximum bit-rate of 4:2:0.

# 4:4:4 bit-rate

Various 4:4:4 sequences were trial encoded with VTM8.0 and the (RA) bit-rates compared with their 4:2:0 versions (obtained using the HDR Tools functions), these are shown below:

|  |  | 420 |  |  |  | 444 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  | Main tier | High tier |  |  | bitrate ratio |
|  | Level | max kbps | max kbps | kbps |  | max kbps | max kbps | kbps |  |  |
| Traffic_QP22 | 5 | 25000 | 100000 | 11807 |  | 62500 | 250000 | 22108 |  | 1.9 |
| Traffic_QP23 |  |  |  | 9479 |  |  |  | 17053 |  | 1.8 |
| Traffic_QP27 |  |  |  | 4716 |  |  |  | 7392 |  | 1.6 |
| Traffic_QP32 |  |  |  | 2273 |  |  |  | 3021 |  | 1.3 |
| Traffic_QP37 |  |  |  | 1164 |  |  |  | 1378 |  | 1.2 |
| BirdsInCage_QP22 | 4.1 | 20000 | 50000 | 3223 |  | 50000 | 125000 | 20728 |  | 6.4 |
| BirdsInCage_QP27 |  |  |  | 738 |  |  |  | 2697 |  | 3.7 |
| BirdsInCage_QP32 |  |  |  | 315 |  |  |  | 469 |  | 1.5 |
| BirdsInCage_QP37 |  |  |  | 166 |  |  |  | 199 |  | 1.2 |
| CrowdRun_QP22 | 4.1 | 20000 | 50000 | 45468 |  | 50000 | 125000 | 67343 |  | 1.5 |
| CrowdRun_QP24 |  |  |  | 31255 |  |  |  | 42521 |  | 1.4 |
| CrowdRun_QP27 |  |  |  | 19145 |  |  |  | 23703 |  | 1.2 |
| CrowdRun_QP32 |  |  |  | 8989 |  |  |  | 10306 |  | 1.1 |
| CrowdRun_QP37 |  |  |  | 4154 |  |  |  | 4591 |  | 1.1 |
| EBULupoCandlelight_QP22 | 4.1 | 20000 | 50000 | 3195 |  | 50000 | 125000 | 3486 |  | 1.1 |
| EBULupoCandlelight_QP27 |  |  |  | 1437 |  |  |  | 1458 |  | 1.0 |
| EBULupoCandlelight_QP32 |  |  |  | 733 |  |  |  | 725 |  | 1.0 |
| EBULupoCandlelight_QP37 |  |  |  | 377 |  |  |  | 366 |  | 1.0 |
| EBURainFruits_QP22 | 4.1 | 20000 | 50000 | 7647 |  | 50000 | 125000 | 8171 |  | 1.1 |
| EBURainFruits_QP27 |  |  |  | 3625 |  |  |  | 3705 |  | 1.0 |
| EBURainFruits_QP32 |  |  |  | 1820 |  |  |  | 1847 |  | 1.0 |
| EBURainFruits_QP37 |  |  |  | 917 |  |  |  | 926 |  | 1.0 |
| Ferns_QP22 | 4 | 12000 | 30000 | 7024 |  | 30000 | 75000 | 8876 |  | 1.3 |
| Ferns_QP27 |  |  |  | 2958 |  |  |  | 3244 |  | 1.1 |
| Ferns_QP32 |  |  |  | 1561 |  |  |  | 1626 |  | 1.0 |
| Ferns_QP37 |  |  |  | 859 |  |  |  | 852 |  | 1.0 |
| Genesis_QP22 | 4 | 12000 | 30000 | 3497 |  | 30000 | 75000 | 9131 |  | 2.6 |
| Genesis_QP27 |  |  |  | 1354 |  |  |  | 2097 |  | 1.5 |
| Genesis_QP32 |  |  |  | 696 |  |  |  | 791 |  | 1.1 |
| Genesis_QP37 |  |  |  | 372 |  |  |  | 413 |  | 1.1 |
| Kimono_QP22 | 4 | 12000 | 30000 | 5369 |  | 30000 | 75000 | 12519 |  | 2.3 |
| Kimono_QP27 |  |  |  | 2168 |  |  |  | 3346 |  | 1.5 |
| Kimono_QP32 |  |  |  | 1059 |  |  |  | 1258 |  | 1.2 |
| Kimono_QP37 |  |  |  | 516 |  |  |  | 579 |  | 1.1 |
| Pool_QP22 | 4 | 12000 | 30000 | 17839 |  | 30000 | 75000 | 27142 |  | 1.5 |
| Pool_QP27 |  |  |  | 7212 |  |  |  | 9688 |  | 1.3 |
| Pool_QP32 |  |  |  | 2899 |  |  |  | 3545 |  | 1.2 |
| Pool_QP37 |  |  |  | 1044 |  |  |  | 1201 |  | 1.2 |
| VenueVu_QP22 | 4 | 12000 | 30000 | 4841 |  | 30000 | 75000 | 5840 |  | 1.2 |
| VenueVu_QP27 |  |  |  | 2382 |  |  |  | 2751 |  | 1.2 |
| VenueVu_QP32 |  |  |  | 1262 |  |  |  | 1396 |  | 1.1 |
| VenueVu_QP37 |  |  |  | 661 |  |  |  | 707 |  | 1.1 |
| View_QP22 | 4 | 12000 | 30000 | 3473 |  | 30000 | 75000 | 12383 |  | 3.6 |
| View_QP27 |  |  |  | 1070 |  |  |  | 2264 |  | 2.1 |
| View_QP32 |  |  |  | 521 |  |  |  | 627 |  | 1.2 |
| View_QP37 |  |  |  | 261 |  |  |  | 294 |  | 1.1 |
| DucksAndLegs_QP22 | 4.1 | 20000 | 50000 | 34445 |  | 50000 | 125000 | 146278 |  | 4.2 |
| DucksAndLegs_QP25 |  |  |  | 16336 |  |  |  | 52992 |  | 3.2 |
| DucksAndLegs_QP27 |  |  |  | 9857 |  |  |  | 26310 |  | 2.7 |
| DucksAndLegs_QP32 |  |  |  | 4036 |  |  |  | 6679 |  | 1.7 |
| DucksAndLegs_QP37 |  |  |  | 1946 |  |  |  | 2343 |  | 1.2 |
| OldTownCross_QP22 | 4.1 | 20000 | 50000 | 21846 |  | 50000 | 125000 | 65907 |  | 3.0 |
| OldTownCross_QP23 |  |  |  | 14784 |  |  |  | 42409 |  | 2.9 |
| OldTownCross_QP27 |  |  |  | 4231 |  |  |  | 10018 |  | 2.4 |
| OldTownCross_QP32 |  |  |  | 1279 |  |  |  | 1534 |  | 1.2 |
| OldTownCross_QP37 |  |  |  | 645 |  |  |  | 706 |  | 1.1 |
| ParkScene_QP22 | 4 | 12000 | 30000 | 7445 |  | 30000 | 75000 | 16000 |  | 2.1 |
| ParkScene_QP27 |  |  |  | 3067 |  |  |  | 4434 |  | 1.4 |
| ParkScene_QP32 |  |  |  | 1358 |  |  |  | 1687 |  | 1.2 |
| ParkScene_QP37 |  |  |  | 604 |  |  |  | 695 |  | 1.2 |
| Seeking_QP22 | 4.1 | 20000 | 50000 | 30794 |  | 50000 | 125000 | 57520 |  | 1.9 |
| Seeking_QP23 |  |  |  | 23392 |  |  |  | 40773 |  | 1.7 |
| Seeking_QP24 |  |  |  | 18771 |  |  |  | 30757 |  | 1.6 |
| Seeking_QP27 |  |  |  | 10270 |  |  |  | 14385 |  | 1.4 |
| Seeking_QP32 |  |  |  | 4443 |  |  |  | 5250 |  | 1.2 |
| Seeking_QP37 |  |  |  | 2052 |  |  |  | 2261 |  | 1.1 |

Note: bit-rate figures are highlighted when they exceed the main tier (green) or high tier (blue) max. rates. These maximum bit-rates are specified in Table A.2.

As can be seen, although many of these 4:4:4 sequences require less than twice the bit-rate of their 4:2:0 versions some of them require significantly more at the lower QPs.  Further investigation of these sequences showed that they had a significant amount of high frequency energy in the U channel so the variance of the Y, U and V channels was calculated and compared:

|  | 444 variance |  |  | bitrate ratio | U:Y var ratio |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  | QP |
| Traffic | 55 | 30 | 5 | 1.9 | 0.5 | 22 |
| BirdsInCage | 27 | 72 | 16 | 6.4 | 2.7 | 22 |
| CrowdRun | 350 | 72 | 54 | 1.5 | 0.2 | 22 |
| EBULupoCandlelight | 44 | 4 | 2 | 1.1 | 0.1 | 22 |
| EBURainFruits | 95 | 4 | 3 | 1.1 | 0.0 | 22 |
| Ferns | 164 | 12 | 16 | 1.3 | 0.1 | 22 |
| Genesis | 22 | 48 | 18 | 2.6 | 2.1 | 22 |
| Kimono | 14 | 32 | 10 | 2.3 | 2.4 | 22 |
| Pool | 74 | 35 | 40 | 1.5 | 0.5 | 22 |
| VenueVu | 72 | 8 | 21 | 1.2 | 0.1 | 22 |
| View | 32 | 59 | 26 | 3.6 | 1.8 | 22 |
| DucksAndLegs | 85 | 266 | 72 | 4.2 | 3.1 | 22 |
| OldTownCross | 112 | 111 | 59 | 3.0 | 1.0 | 22 |
| ParkScene | 66 | 42 | 28 | 2.1 | 0.6 | 22 |
| Seeking | 143 | 64 | 54 | 1.9 | 0.4 | 22 |

And plotting bit-rate against the ratio of U and Y variance:

Bit-rate are shown at QP22 and at a QP that meets the Main Tier max. bit-rate specification

Examining a section of the source image BirdsInCage shows a chroma noise pattern:

Source RGB

Source Y channel

Source U channel

Source V channel

The noise pattern seen in the U channel is likely due to a lack of anti-aliasing in the de-Bayering process.

## Reducing the chroma noise

A 1,2,1 horizontal low-pass filter was applied to the U channel prior to encoding which brought the required bit-rate much closer to twice that required for 4:2:0 encoding:

Of course this low-pass filtering has a detrimental effect on the chroma PSNR and hence the chroma BD rate but improves the luma BD rate due to the reduced bit-rate.  Experiments have shown that adjusting the chroma QP offset can mitigate this chroma loss to some extent. It may also be the case that the subjective quality is not so much affected by filtering

# Conclusion

It has been shown that the Main 4:4:4 10 profile can work with twice the bit-rate of the Main 10 profile provided chroma noise in the source is limited.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, B. Bross, J. Chen, S. Liu and Y-K. Wang