| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0244 |
| --- | --- |

| Title: | AHG9: Coded Picture Buffer sizes and MinCr in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Keating Karl Sharman | Email: | steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

This contribution describes proposed changes to maximum coded picture buffer (CPB) size and MinCrScalingFactor in order to guarantee that the buffer can always store a full picture when compressed at the minimum compression ratio (MinCr).

# Introduction

Table A.1 in the current specification specifies the maximum coded picture buffer (CPB) size, MaxCPB, for each level. It is a requirement that a coded picture must also be stored in its entirety inside the CPB, pulled out atomically in an instant, from Annex C1:

If DecodingUnitHrdFlag is equal to 0, the HRD operates at AU level and each DU is an AU. Otherwise the HRD operates at DU level and each DU is a subset of an AU.

NOTE 5 – If the HRD operates at AU level, each time when some bits are removed from the CPB, a DU that is an entire AU is removed from the CPB. Otherwise (the HRD operates at DU level), each time when some bits are removed from the CPB, a DU that is a subset of an AU is removed from the CPB. Regardless of whether the HRD operates at access unt level or DU level, each time when some picture is output from the DPB, an entire decoded picture is output from the DPB, though the picture output time is derived based on the differently derived CPB removal times and the differently signalled DPB output delays.

In addition, Table A.2 of the current specification specifies a MinCrBase, which, along with the MinCrScalingFactor, allows the minimum compression ratio (MinCr) to be calculated. MinCr is used to guarantee that a picture is compressed by at least the specified amount.

However, MinCr is not always the limiting factor. It is therefore proposed to either remove MinCr from the specification, or to adjust the values in Annex A to make MinCr once again useful.

This contribution will show that, for example, an 8K picture coded at the minimum compression ratio of minCr using Main 10 profile, level 6, main tier cannot be stored in the CPB; for Main 4:4:4 10 profile, such is the case for several levels.

Tables A.1 through A.3 are included below for reference:

Table A.1 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slices per picture MaxSlicesPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 |

Table A.2 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1 | 552 960 | 128 | - | 2 | 2 |
| 2 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 6 | 4 |

Table A.3 – Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor

| Profile | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main 10 | 1 000 | 1 100 | 1.875 | 1.0 |
| Main 4:4:4 10 | 2 500 | 2 750 | 3.750 | 0.5 |

# Problem

The table below shows the max picture size, max CPB size, max coded picture size at MinCr and the minimum number of pictures in the CPB for Main 10 profile:

| Level | Max luma picture size MaxLumaPs (samples) | Example max luma size | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) | Max CPB size MaxCPB Main10 (bits) | Min compression ratio MinCrBase | Max coded picture size Main10 (bits @ 15bits/sample) | Min coded pictures in CPB Main10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | Main tier | Main tier | Main tier | Main tier |
| 1 | 36,864 |  | 350 | 350,000 | 2 | 276,480 | 1.3 |
| 2 | 122,880 |  | 1,500 | 1,500,000 | 2 | 921,600 | 1.6 |
| 2.1 | 245,760 |  | 3,000 | 3,000,000 | 2 | 1,843,200 | 1.6 |
| 3 | 552,960 |  | 6,000 | 6,000,000 | 2 | 4,147,200 | 1.4 |
| 3.1 | 983,040 |  | 10,000 | 10,000,000 | 2 | 7,372,800 | 1.4 |
| 4 | 2,228,224 | 2048x1088 | 12,000 | 12,000,000 | 4 | 8,355,840 | 1.4 |
| 4.1 | 2,228,224 | 2048x1088 | 20,000 | 20,000,000 | 4 | 8,355,840 | 2.4 |
| 5 | 8,912,896 | 4096x2176 | 25,000 | 25,000,000 | 6 | 22,282,240 | 1.1 |
| 5.1 | 8,912,896 | 4096x2176 | 40,000 | 40,000,000 | 8 | 16,711,680 | 2.4 |
| 5.2 | 8,912,896 | 4096x2176 | 60,000 | 60,000,000 | 8 | 16,711,680 | 3.6 |
| 6 | 35,651,584 | 8192x4352 | 60,000 | 60,000,000 | 8 | 66,846,720 | 0.9 |
| 6.1 | 35,651,584 | 8192x4352 | 120,000 | 120,000,000 | 8 | 66,846,720 | 1.8 |
| 6.2 | 35,651,584 | 8192x4352 | 240,000 | 240,000,000 | 6 | 89,128,960 | 2.7 |

Note that the fields highlighted in green are from Table A.1 or Table A.2 in the spec., other fields are derived from these, CpbVclFactor and MinCrScaleFactor (Table A.3).

The next table shows the same for Main 444 10 profile:

| Level | Max luma picture size MaxLumaPs (samples) | Example max luma size | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) | Max CPB size MaxCPB Main444_10 (bits) | Min compression ratio MinCrBase | Max coded picture size Main444_10 (bits @30bits/sample) | Min coded pictures in CPB Main444_10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | Main tier | Main tier | Main tier | Main tier |
| 1 | 36,864 |  | 350 | 875,000 | 2 | 1,105,920 | 0.8 |
| 2 | 122,880 |  | 1,500 | 3,750,000 | 2 | 3,686,400 | 1.0 |
| 2.1 | 245,760 |  | 3,000 | 7,500,000 | 2 | 7,372,800 | 1.0 |
| 3 | 552,960 |  | 6,000 | 15,000,000 | 2 | 16,588,800 | 0.9 |
| 3.1 | 983,040 |  | 10,000 | 25,000,000 | 2 | 29,491,200 | 0.8 |
| 4 | 2,228,224 | 2048x1088 | 12,000 | 30,000,000 | 4 | 33,423,360 | 0.9 |
| 4.1 | 2,228,224 | 2048x1088 | 20,000 | 50,000,000 | 4 | 33,423,360 | 1.5 |
| 5 | 8,912,896 | 4096x2176 | 25,000 | 62,500,000 | 6 | 89,128,960 | 0.7 |
| 5.1 | 8,912,896 | 4096x2176 | 40,000 | 100,000,000 | 8 | 66,846,720 | 1.5 |
| 5.2 | 8,912,896 | 4096x2176 | 60,000 | 150,000,000 | 8 | 66,846,720 | 2.2 |
| 6 | 35,651,584 | 8192x4352 | 60,000 | 150,000,000 | 8 | 267,386,880 | 0.6 |
| 6.1 | 35,651,584 | 8192x4352 | 120,000 | 300,000,000 | 8 | 267,386,880 | 1.1 |
| 6.2 | 35,651,584 | 8192x4352 | 240,000 | 600,000,000 | 6 | 356,515,840 | 1.7 |

Note that only Main tier is shown as the problem does not occur at High tier.

In the above two tables, the yellow cells highlight the cases where MinCr is irrelevant for the nominal picture formats.

# Proposed Changes

It is proposed that either MinCr constraint is removed, as it is often not required, or the values used to derive the CPB size are adjusted so that MinCr becomes always relevant again.

## Main 10

The table below shows a modified max CPB size for level 6:

| Level | Max luma picture size MaxLumaPs (samples) | Example max luma size | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) | Max CPB size MaxCPB Main10 (bits) | Min compression ratio MinCrBase | Max coded luma picture size Main10 (bits @15bits/sample) | Min coded pictures in CPB Main10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | Main tier | Main tier | Main tier | Main tier |
| 1 | 36,864 |  | 350 | 350,000 | 2 | 276,480 | 1.3 |
| 2 | 122,880 |  | 1,500 | 1,500,000 | 2 | 921,600 | 1.6 |
| 2.1 | 245,760 |  | 3,000 | 3,000,000 | 2 | 1,843,200 | 1.6 |
| 3 | 552,960 |  | 6,000 | 6,000,000 | 2 | 4,147,200 | 1.4 |
| 3.1 | 983,040 |  | 10,000 | 10,000,000 | 2 | 7,372,800 | 1.4 |
| 4 | 2,228,224 | 2048x1088 | 12,000 | 12,000,000 | 4 | 8,355,840 | 1.4 |
| 4.1 | 2,228,224 | 2048x1088 | 20,000 | 20,000,000 | 4 | 8,355,840 | 2.4 |
| 5 | 8,912,896 | 4096x2176 | 25,000 | 25,000,000 | 6 | 22,282,240 | 1.1 |
| 5.1 | 8,912,896 | 4096x2176 | 40,000 | 40,000,000 | 8 | 16,711,680 | 2.4 |
| 5.2 | 8,912,896 | 4096x2176 | 60,000 | 60,000,000 | 8 | 16,711,680 | 3.6 |
| 6 | 35,651,584 | 8192x4352 | 80,000 | 80,000,000 | 8 | 66,846,720 | 1.2 |
| 6.1 | 35,651,584 | 8192x4352 | 120,000 | 120,000,000 | 8 | 66,846,720 | 1.8 |
| 6.2 | 35,651,584 | 8192x4352 | 240,000 | 240,000,000 | 6 | 89,128,960 | 2.7 |

## Main 4:4:4 10

A simple fix for this profile is to adjust MinCrScaleFactor, the table below shows the same modification for level 6 MaxCPB but also with MinCrScaleFactor=0.75:

| Level | Max luma picture size MaxLumaPs (samples) | Example max luma size | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) | Max CPB size MaxCPB Main444_10 (bits) | Min compression ratio MinCrBase | Max coded luma picture size Main444_10 (bits @30bits/sample) | Min coded pictures in CPB Main444_10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | Main tier | Main tier | Main tier | Main tier |
| 1 | 36,864 |  | 350 | 875,000 | 2 | 737,280 | 1.2 |
| 2 | 122,880 |  | 1,500 | 3,750,000 | 2 | 2,457,600 | 1.5 |
| 2.1 | 245,760 |  | 3,000 | 7,500,000 | 2 | 4,915,200 | 1.5 |
| 3 | 552,960 |  | 6,000 | 15,000,000 | 2 | 11,059,200 | 1.4 |
| 3.1 | 983,040 |  | 10,000 | 25,000,000 | 2 | 19,660,800 | 1.3 |
| 4 | 2,228,224 | 2048x1088 | 12,000 | 30,000,000 | 4 | 22,282,240 | 1.3 |
| 4.1 | 2,228,224 | 2048x1088 | 20,000 | 50,000,000 | 4 | 22,282,240 | 2.2 |
| 5 | 8,912,896 | 4096x2176 | 25,000 | 62,500,000 | 6 | 59,419,307 | 1.1 |
| 5.1 | 8,912,896 | 4096x2176 | 40,000 | 100,000,000 | 8 | 44,564,480 | 2.2 |
| 5.2 | 8,912,896 | 4096x2176 | 60,000 | 150,000,000 | 8 | 44,564,480 | 3.4 |
| 6 | 35,651,584 | 8192x4352 | 80,000 | 200,000,000 | 8 | 178,257,920 | 1.1 |
| 6.1 | 35,651,584 | 8192x4352 | 120,000 | 300,000,000 | 8 | 178,257,920 | 1.7 |
| 6.2 | 35,651,584 | 8192x4352 | 240,000 | 600,000,000 | 6 | 237,677,227 | 2.5 |

However, Table A.3 specifies the CpbVclFactor for Main 444 10 to be 2.5 times that for Main 10, this is in part due to the fact that the CpbVclFactor for Main 10 was taken from HEVC’s Main 10 profile where it wasn’t increased over the original 8-bit profile.  To keep the CpbVclFactor consistent with the FormatCapabilityFactor it is proposed to change the CpbVclFactor for Main 444 10 to be twice that for Main 10, it is also proposed to keep the MinCrScaleFactor at 1.0 for both profiles:

Table A.3 – Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor

| Profile | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main 10 | 1 000 | 1 100 | 1.875 | 1.0 |
| Main 4:4:4 10 | 2 000 | 2 200 | 3.750 | 1.0 |

The next table shows the effect of the proposed modifications:

| Level | Max luma picture size MaxLumaPs (samples) | Example max luma size | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) | Max CPB size MaxCPB Main444_10 (bits) | Min compression ratio MinCrBase | Max coded luma picture size Main444_10 (bits @30bits/sample) | Min coded pictures in CPB Main444_10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | Main tier | Main tier | Main tier | Main tier |
| 1 | 36,864 |  | 350 | 700,000 | 2 | 552,960 | 1.3 |
| 2 | 122,880 |  | 1,500 | 3,000,000 | 2 | 1,843,200 | 1.6 |
| 2.1 | 245,760 |  | 3,000 | 6,000,000 | 2 | 3,686,400 | 1.6 |
| 3 | 552,960 |  | 6,000 | 12,000,000 | 2 | 8,294,400 | 1.4 |
| 3.1 | 983,040 |  | 10,000 | 20,000,000 | 2 | 14,745,600 | 1.4 |
| 4 | 2,228,224 | 2048x1088 | 12,000 | 24,000,000 | 4 | 16,711,680 | 1.4 |
| 4.1 | 2,228,224 | 2048x1088 | 20,000 | 40,000,000 | 4 | 16,711,680 | 2.4 |
| 5 | 8,912,896 | 4096x2176 | 25,000 | 50,000,000 | 6 | 44,564,480 | 1.1 |
| 5.1 | 8,912,896 | 4096x2176 | 40,000 | 80,000,000 | 8 | 33,423,360 | 2.4 |
| 5.2 | 8,912,896 | 4096x2176 | 60,000 | 120,000,000 | 8 | 33,423,360 | 3.6 |
| 6 | 35,651,584 | 8192x4352 | 80,000 | 160,000,000 | 8 | 133,693,440 | 1.2 |
| 6.1 | 35,651,584 | 8192x4352 | 120,000 | 240,000,000 | 8 | 133,693,440 | 1.8 |
| 6.2 | 35,651,584 | 8192x4352 | 240,000 | 480,000,000 | 6 | 178,257,920 | 2.7 |

JVET-R0243 compares 4:4:4 and 4:2:0 bit-rates for various sequences in VTM8.0.

Note that theses proposed changes are independent of, but compatible with, the level coding method proposed in JVET-R0245.

# Conclusion

It is proposed that either MinCr is removed, or else values in Annex A are adjusted so that it is useful for all levels and profiles.

If MinCr is still to be useful then the Max CPB size for level 6 should be increased to guarantee the buffer is big enough to always store a full picture.  The MinCrScalingFactor for the Main 4:4:4 10 profile must also be increased for the same reason. It is also proposed to modify the CpbVclFactor and CpbNalFactor for the Main 4:4:4 10 profile to be twice those of the Main 10 profile.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Specification Changes

Changes to tables A.1 and A.3 as detailed above.

# References

- “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, B. Bross, J. Chen, S. Liu and Y-K. Wang