| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0245 |
| --- | --- |

| Title: | AHG9: Level coding in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Keating Karl Sharman | Email: | steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

This document describes a modified method of encoding the level and a modified decoder constraint allowing increased picture sizes and frame rates to be accommodated.

# Current Method

In the current specification various levels are defined from 1 to 6.2 with the highest possible level being 8.5; the level is coded in 8 bits as Level*30, hence level 8.5 being coded as 255.  There is also the condition:

Decoders conforming to the *** profile at a specific level of a specific tier shall be capable of decoding all bitstreams for which all of the following conditions apply:

- …

- The bitstream is indicated to conform to a level that is not level 8.5 and is lower than or equal to the specified level.

The currently defined levels with example picture sizes and frame rates are shown below:

|  | JVET-Q2001-vE Table A.1 |  |  | JVET-Q2001-vE Table A.2 |  |
| --- | --- | --- | --- | --- | --- |
| Level | Max luma picture size MaxLumaPs (samples) | Example max luma size |  | Max luma sample rate MaxLumaSr (samples/sec) | Max frame rate of max size MaxLumaSr/MaxLumaPs (frames/sec) |
| 1 | 36,864 |  |  | 552,960 |  |
| 2 | 122,880 |  |  | 3,686,400 |  |
| 2.1 | 245,760 |  |  | 7,372,800 |  |
| 3 | 552,960 |  |  | 16,588,800 |  |
| 3.1 | 983,040 |  |  | 33,177,600 |  |
| 4 | 2,228,224 | 2048x1088 |  | 66,846,720 | 30 |
| 4.1 | 2,228,224 | 2048x1088 |  | 133,693,440 | 60 |
| 5 | 8,912,896 | 4096x2176 |  | 267,386,880 | 30 |
| 5.1 | 8,912,896 | 4096x2176 |  | 534,773,760 | 60 |
| 5.2 | 8,912,896 | 4096x2176 |  | 1,069,547,520 | 120 |
| 6 | 35,651,584 | 8192x4352 |  | 1,069,547,520 | 30 |
| 6.1 | 35,651,584 | 8192x4352 |  | 2,139,095,040 | 60 |
| 6.2 | 35,651,584 | 8192x4352 |  | 4,278,190,080 | 120 |

There are several issues with the current specification of levels:

- As defined, it is not worthwhile to create new levels between the different existing levels

For example, level 5.2 and level 6 have the same MaxLumaSr. The only possible change is to have some intermediate picture size, but this does not seem to make sense.

- It is not possible to go beyond 120fps for a given level picture size.

The only method is to use a level with a larger picture size.

For example, a new level 5.3 cannot be defined for 4K at 240fps because this would require twice the sample rate of the current level 6, and it is a current requirement that a level 6 decoder would have to decode a level 5.X stream.

- The maximum level is 8.5, implying a maximum image size of 32K

Although this might not seem to be an immediate issue, it may cause problems, for example, for future omnidirectional applications that use subpictures.

# Proposed Method

To address these issues, an alternative method of coding the levels is proposed. In addition, an alternative constraint on the ability to decode streams is also proposed.

## Coding of level

The alternative coding of levels is proposed so that the levels do not waste codes of levels that cannot be ever defined. This in turn extends the highest possible value from 8.5 to 15.15. As an aside, the alternative coding also makes the level code instantly visible in a hex dump.

The level should be coded in 8 bits as major*16+minor, e.g. level 4.1 is coded as 65 and the maximum level is 15.15.

## Constraint of decoding of levels.

To address the issue of higher frame rates not currently being possible without using a higher level (and therefore mandating the use of longer line buffers etc.), it is proposed to change the constraint on the ability to decode streams,

New minor levels for higher frame rates may be added without requiring a decoder for the next major level to be able to decode the higher sample rate if the condition is changed to:

Decoders conforming to the *** profile at a specific level, D.d  of a specific tier shall be capable of decoding all bitstreams for which all of the following conditions apply:

- …

- The bitstream is indicated to conform to a level S.s that is not level 15.15 and S is lower than or equal to D and S*2+s is lower than or equal to D*2+d..

This constraint is completely backwards compatible with all currently defined levels. It also does not impact any decoder. However, it does now permit the generation of high frame rate decoders at a lower layer than currently possible.

As an example, adding a new level 5.3:

|  | JVET-Q2001-vE Table A.1 |  |  | JVET-Q2001-vE Table A.2 |  |
| --- | --- | --- | --- | --- | --- |
| Level | Max luma picture size MaxLumaPs (samples) | Example max luma size |  | Max luma sample rate MaxLumaSr (samples/sec) | Max frame rate of max size MaxLumaSr/MaxLumaPs (frames/sec) |
| 1 | 36,864 |  |  | 552,960 |  |
| 2 | 122,880 |  |  | 3,686,400 |  |
| 2.1 | 245,760 |  |  | 7,372,800 |  |
| 3 | 552,960 |  |  | 16,588,800 |  |
| 3.1 | 983,040 |  |  | 33,177,600 |  |
| 4 | 2,228,224 | 2048x1088 |  | 66,846,720 | 30 |
| 4.1 | 2,228,224 | 2048x1088 |  | 133,693,440 | 60 |
| 5 | 8,912,896 | 4096x2176 |  | 267,386,880 | 30 |
| 5.1 | 8,912,896 | 4096x2176 |  | 534,773,760 | 60 |
| 5.2 | 8,912,896 | 4096x2176 |  | 1,069,547,520 | 120 |
| 5.3 | 8,912,896 | 4096x2176 |  | 2,139,095,040 | 240 |
| 6 | 35,651,584 | 8192x4352 |  | 1,069,547,520 | 30 |
| 6.1 | 35,651,584 | 8192x4352 |  | 2,139,095,040 | 60 |
| 6.2 | 35,651,584 | 8192x4352 |  | 4,278,190,080 | 120 |

A level 5.3 stream would be decodable by a level 6.1 decoder as 5 is lower than or equal to 6 and 5*2+3 is lower than or equal to 6*2+1.

# Conclusion.

An alternative method for coding the level is proposed which increases the maximum level from 8.5 to 15.15.  An alternative constraint is also proposed which allows levels with higher frame rates to be added without requiring all higher level decoders to be able to decoder the increased bitrate.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Specification Changes

A.3.1	Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Referenced SPSs shall have chroma_format_idc equal to 0 or 1.

- Referenced SPSs shall have bit_depth_minus8 in the range of 0 to 2, inclusive.

- Referenced SPSs shall have sps_palette_enabled_flag equal to 0.

- general_level_idc and sublayer_level_idc[ i ] for all values of i in the VPS (when available) and in the referenced SPSs shall not be equal to 255 (which indicates level 8.5 15.15).

- The tier and level constraints specified for the Main 10 profile in clause A.4, as applicable, shall be fulfilled.

Conformance of a bitstream to the Main 10 profile is indicated by general_profile_idc being equal to 1.

Decoders conforming to the Main 10 profile at a specific level, D.d of a specific tier shall be capable of decoding all bitstreams for which all of the following conditions apply:

- The bitstream is indicated to conform to the Main 10 profile.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

- The bitstream is indicated to conform to a level S.s that is not level 8.5 15.15and is lower than or equal to the specified level D and S*2+s is lower than or equal to D*2+d.

A.3.2	Main 4:4:4 10 profile

Bitstreams conforming to the Main 4:4:4 10 profile shall obey the following constraints:

- Referenced SPSs shall have chroma_format_idc in the range of 0 to 3, inclusive.

- Referenced SPSs shall have bit_depth_minus8 in the range of 0 to 2, inclusive.

- general_level_idc and sublayer_level_idc[ i ] for all values of i in the VPS (when available) and in the referenced SPSs shall not be equal to 255 (which indicates level 8.5 15.15).

- The tier and level constraints specified for the Main 4:4:4 10 profile in clause A.4, as applicable, shall be fulfilled.

Conformance of a bitstream to the Main 4:4:4 10 profile is indicated by general_profile_idc being equal to 2.

Decoders conforming to the Main 4:4:4 10 profile at a specific level of a specific tier shall be capable of decoding all bitstreams for which all of the following conditions apply:

- The bitstream is indicated to conform to the Main 4:4:4 10 or Main 10 profile.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

- The bitstream is indicated to conform to a level S.s that is not level 8.5 15.15and is lower than or equal to the specified level D and S*2+s is lower than or equal to D*2+d.

A.4.1	General tier and level limits

Table A.1 specifies the limits for each level of each tier for levels other than level 8.5 15.15.

A tier and level to which a bitstream conforms are indicated by the syntax elements general_tier_flag and general_level_idc, and a level to which a sublayer representation conforms are indicated by the syntax element sublayer_level_idc[ i ], as follows:

–	If the specified level is not level 8.5 15.15, general_tier_flag equal to 0 indicates conformance to the Main tier, general_tier_flag equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table A.1 and general_tier_flag shall be equal to 0 for levels below level 4 (corresponding to the entries in Table A.1 marked with "-"). Otherwise (the specified level is level 8.5 15.15), it is a requirement of bitstream conformance that general_tier_flag shall be equal to 1 and the value 0 for general_tier_flag is reserved for future use by ITU-T | ISO/IEC and decoders shall ignore the value of general_tier_flag.

–	general_level_idc and sublayer_level_idc[ i ] shall be set equal to a value of 30 times the level number 16 times the major level number plus 1 times the minor level number specified in Table A.1.

All other references to level 8.5 also changed to level 15.15 (it would possibly be helpful to actually specify 15.15 as a term such as “limitless_level_idc”.

# References

- “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, B. Bross, J. Chen, S. Liu and Y-K. Wang