| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0248-v1 |
| --- | --- |

| Title: | AHG9: Supporting multiple slices within one tile for raster-scan slice mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Lulin Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, lulin.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes three modifications related to signalling slice partitioning of the coded picture, summarized as follows:

- Support the slice that consists of one or more consecutive complete CTU rows within one tile in raster-scan slice mode. Two syntax elements, num_rows_in_slice_idc and tile_row_id, are designed for this.

- When slice_address is equal to ( NumTilesInPic – 1 ), the syntax element num_tiles_in_slice_minus1 is not coded and is inferred to be equal to 0.

- A new syntax element multiple_slices_in_tile_enabled_flag is signalled in the picture parameter set (PPS) to specify whether it is allowed to further partition a tile into more than one slice for pictures referring to the PPS.

It is asserted that a finer granularity of partitioning units is desirable when partitioing of each coded slice is adaptively determined during encoding and signalled in raster-scan slice mode. The software of this proposal is ready and attached in the proposal package.

Introduction

In Versatile Video Coding (VVC) Draft 6 as specified in [1], a slice may contain one or more consecutive rows of complete CTUs in units of bricks within a tile for both raster-scan slice mode and rectangular slice mode. In VVC Draft 8 [2], each tile can no longer be further partitioned into more than one slice in the raster-scan slice mode ( rect_slice_flag = 0 ). It is desirable that a finer granularity of partitioning units can be provided when partitioing of each coded slice is adaptively determined and signalled in raster-scan slice mode. It is more consistent in design that the two slice modes can achieve the same granulairy for partitioning the picture into slices.

Proposed modifications

This contribution proposes three modifications to VVC Draft 8 related to signalling slice partitioning of the coded picture. The modified software based on VTM8.0 [3] for the proposals is ready and attached in the proposal package.

Modification 1: Supporting multiple slices within one tile in raster-scan slice mode

In Modification 1, it is proposed to support the slice that consists of one or more consecutive complete CTU rows within one tile in the raster-scan slice mode. When rect_slice_flag and num_tiles_in_slice_minus1 are both equal to 0, a new syntax element num_rows_in_slice_idc is further signalled for deriving the number of the CTU rows in the coded slice. num_rows_in_slice_idc equal to 0 indicates that the current slice conatins all CTU rows within the tile with the tile index equal to slice_address. When num_rows_in_slice_idc is greater than 0, it specifies the number of consecutive complete CTU rows within the associated tile in the current slice. When num_rows_in_slice_idc is greater than 0, a new syntax element tile_row_id is further signalled for specifying the first CTU row index of the current slice in the associated tile. When num_rows_in_slice_idc is equal to 0, tile_row_id is inferred to be equal to 0 and is not signalled.

Modification 2: Conditionally signalling num_tiles_in_slice_minus1

When rect_slice_flag is equal to 0, the value of the slice address in the raster-scan tile index plus the number of the consecutive tiles in tile raster-scan order in the coded slice shall be less than or equal to the number of the tiles in the picture. In Modification 2, num_tiles_in_slice_minus1 is not coded and is inferred to be equal to 0 when slice_address is equal to ( NumTilesInPic – 1 ).

Modification 3: Signalling multiple_slices_in_tile_enabled_flag

In Modification 3, a new syntax element multiple_slices_in_tile_enabled_flag is signalled in the PPS to specify whether it is allowed to further partitioned one tile into more than one slice for pictures referring to the PPS. When multiple_slices_in_tile_enabled_flag equal to 0 and a slice is contained within a tile, it is inferred that the slice contains the complete tile and information for signalling slice partioning within a tile is not coded.

Modifications to Specification

The proposed modifications to VVC Draft 8 are written in red and provided below:

- 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| …. |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| multiple_slices_in_tile_enabled_flag | u(1) |
| if( NumTilesInPic > 1 ¦ ¦ multiple_slices_in_tile_enabled_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0 ) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 && multiple_slices_in_tile_enabled_flag ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| for( j = 0; j < num_exp_slices_in_tile[ i ]; j++ ) |  |
| exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| …. |  |
| } |  |

- 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if( picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| if( subpic_info_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( ( rect_slice_flag && NumSlicesInSubpic[ CurrSubpicIdx ] > 1 ) ¦ ¦ ( !rect_slice_flag && NumTilesInPic > 1 ) ) |  |
| slice_address | u(v) |
| for( i = 0; i < NumExtraShBits; i++ ) |  |
| sh_extra_bit[ i ] | u(1) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) { |  |
| if( slice_address < NumTilesInPic – 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| if( num_tiles_in_slice_minus1 = = 0 && multiple_slices_in_tile_enabled_flag && RowHeight[ slice_address / NumTileColumns ] > 1 ) { |  |
| num_rows_in_slice_idc | u(v) |
| if( num_rows_in_slice_idc ) |  |
| tile_row_id | ue(v) |
| } |  |
| } |  |
| for( i = 0; i < NumExtraShBits; i++ ) |  |
| sh_extra_bit[ i ] | u(1) |
| …. |  |
| } |  |

- 7.4.2.4.5 Order of VCL NAL units and their association to coded pictures

The order of the VCL NAL units within a coded picture is constrained as follows:

–	For any two coded slice NAL units A and B of a coded picture, let SubpicIdVal[ subpicIdxA ] and SubpicIdVal[ subpicIdxB ] be their subpicture level index slice_subpic_id values, and sliceAddrA and sliceddrB be their slice_address values, and tileRowIdA and tileRowIdB be their tile_row_id values.

–	When any of the following conditions is true, coded slice NAL unit A shall precede coded slice NAL unit B:

–	subpicIdxA is less than subpicIdxB.

–	subpicIdxA is equal to subpicIdxB and sliceAddrA is less than sliceAddrB.

–	rect_slice_flag is equal to 0, subpicIdxA is equal to subpicIdxB, sliceAddrA is equal to sliceAddrB, and tileRowIdA is less than tileRowIdB.

- 7.4.3.4 Picture parameter set RBSP semantics

multiple_slices_in_tile_enabled_flag equal to 1 specifies that one or more slices may be contained in a tile for each coded picture referring to the PPS. multiple_slices_in_tile_enabled_flag equal to 0 specifies that no more than one slice shall be conatined in any tile for each coded picture referring to the PPS. When multiple_slices_in_tile_enabled_flag is not present, it is inferred to be equal to 0.

#### 7.4.8.1 General slice header semantics

slice_subpic_id specifies the subpicture ID of the subpicture that contains the slice. If slice_subpic_id is present, the value of the variable CurrSubpicIdx is derived to be such that SubpicIdVal[ CurrSubpicIdx ] is equal to slice_subpic_id. Otherwise (slice_subpic_id is not present), slice_subpic_id is inferred to be equal to SubpicIdVal[ 0 ] and CurrSubpicIdx is derived to be equal to 0. The length of slice_subpic_id is sps_subpic_id_len_minus1 + 1 bits.

slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0.

If rect_slice_flag is equal to 0, the following applies:

- The slice address is the raster scan tile index of the first tile in the slice.

- The length of slice_address is Ceil( Log2 ( NumTilesInPic ) ) bits.

- The value of slice_address shall be in the range of 0 to NumTilesInPic − 1, inclusive.

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the subpicture-level slice index of the slice.

- The length of slice_address is Ceil( Log2( NumSlicesInSubpic[ CurrSubpicIdx ] ) ) bits.

- The value of slice_address shall be in the range of 0 to NumSlicesInSubpic[ CurrSubpicIdx ] − 1, inclusive.

sh_extra_bit[ i ] may be equal to 1 or 0. Decoders conforming to this version of this Specification shall ignore the value of sh_extra_bit[ i ]. Its value does not affect decoder conformance to profiles specified in this version of specification.

num_tiles_in_slice_minus1 plus 1, when present, specifies the number of tiles in the slice when rect_slice_flag is equal to 0. The value of num_tiles_in_slice_minus1 shall be in the range of 0 to NumTilesInPic – 1 − slice_address, inclusive. When rect_slice_flag is equal to 0 and num_tiles_in_slice_minus1 is not present, the value of num_tiles_in_slice_minus1 is inferred to be equal to 0.

num_rows_in_slice_idc equal to 0 specifies that the number of consecutive complete CTU rows within the tile with the raster scan tile index slice_address in the slice is equal to RowHeight[ slice_address / NumTileColumns ] when num_tiles_in_slice_minus1 is equal to 0. When num_tiles_in_slice_minus1 is equal to 0 and num_rows_in_slice_idc is greater than 0, num_rows_in_slice_idc specifies the number of consecutive complete CTU rows within the tile with the raster scan tile index slice_address in the slice. The value of num_rows_in_slice_idc shall be in the range of 0 to RowHeight[ slice_address / NumTileColumns ] – 1. When num_tiles_in_slice_minus1 is equal to 0 and num_rows_in_slice_idc is not present, the value of num_rows_in_slice_idc is inferred to be equal to 0.

tile_row_id specifies the first CTU row index of the slice within the tile with the raster scan tile index slice_address. The length of tile_row_id is Ceil( Log2 ( RowHeight[ slice_address / NumTileColumns ] ) ) bits. The value of tile_row_id shall be in the range of 0 to RowHeight[ slice_address / NumTileColumns ] – 1. When rect_slice_flag is equal to 0 and tile_row_id is not present, the value of tile_row_id is inferred to be equal to 0.

It is a requirement of bitstream conformance that the following constraints apply:

- If rect_slice_flag is equal to 0 or subpic_info_present_flag is equal to 0, the value pair of slice_address and tile_row_id values shall not be equal to the value pair of slice_address and tile_row_id values of any other coded slice NAL unit of the same coded picture.

- Otherwise, the pair of slice_subpic_id and slice_address values shall not be equal to the pair of slice_subpic_id and slice_address values of any other coded slice NAL unit of the same coded picture.

- The shapes of the slices of a picture shall be such that each CTU, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded CTU(s).

- If rect_slice_flag is equal to 0 and num_tiles_in_slice_minus1 is equal to 0, the sum of num_rows_in_slice and tile_row_id shall be less than or equal to RowHeight[ slice_address / NumTileColumns ].

The variable NumCtusInCurrSlice, which specifies the number of CTUs in the current slice, and the list CtbAddrInCurrSlice[ i ], for i ranging from 0 to NumCtusInCurrSlice − 1, inclusive, specifying the picture raster scan address of the i-th CTB within the slice, are derived as follows:

if( rect_slice_flag ) {
	picLevelSliceIdx = slice_address
	for( j = 0; j < CurrSubpicIdx; j++ )
		picLevelSliceIdx  +=  NumSlicesInSubpic[ j ]
	NumCtusInCurrSlice = NumCtusInSlice[ picLevelSliceIdx ]
	for( i = 0; i < NumCtusInCurrSlice; i++ )
		CtbAddrInCurrSlice[ i ] = CtbAddrInSlice[ picLevelSliceIdx ][ i ]	
} else {
	NumCtusInCurrSlice = 0
   if( num_tiles_in_slice_minus1 = = 0  &&  num_rows_in_slice_idc ) {
     tileIdx = slice_address 
     tileX = tileIdx % NumTileColumns
		tileY = tileIdx / NumTileColumns
		for( i = 0, ctbY = tileRowBd[ tileY ] + tile_row_id; i < num_rows_in_slice_idc; ctbY++, i++ ) {
			for( ctbX = tileColBd[ tileX ]; ctbX < tileColBd[ tileX + 1 ]; ctbX++ ) {
				CtbAddrInCurrSlice[ NumCtusInCurrSlice ] = ctbY * PicWidthInCtb + ctbX
				NumCtusInCurrSlice++
			}
		}
   } else {
     for( tileIdx = slice_address; tileIdx  <=  slice_address + num_tiles_in_slice_minus1; tileIdx++ ) {
			tileX = tileIdx % NumTileColumns
			tileY = tileIdx / NumTileColumns
			for( ctbY = tileRowBd[ tileY ]; ctbY < tileRowBd[ tileY + 1 ]; ctbY++ ) {
				for( ctbX = tileColBd[ tileX ]; ctbX < tileColBd[ tileX + 1 ]; ctbX++ ) {
					CtbAddrInCurrSlice[ NumCtusInCurrSlice ] = ctbY * PicWidthInCtb + ctbX
					NumCtusInCurrSlice++
				}
			}
		}
	}
}

The variables SubpicLeftBoundaryPos, SubpicTopBoundaryPos, SubpicRightBoundaryPos, and SubpicBotBoundaryPos are derived as follows:

if( subpic_treated_as_pic_flag[ CurrSubpicIdx ] ) {
	SubpicLeftBoundaryPos = subpic_ctu_top_left_x[ CurrSubpicIdx ] * CtbSizeY
	SubpicRightBoundaryPos = Min( pic_width_max_in_luma_samples − 1,
			( subpic_ctu_top_left_x[ CurrSubpicIdx ] +
			subpic_width_minus1[ CurrSubpicIdx ] + 1 ) * CtbSizeY − 1 )
	SubpicTopBoundaryPos = subpic_ctu_top_left_y[ CurrSubpicIdx ] *CtbSizeY	(117)
	SubpicBotBoundaryPos = Min( pic_height_max_in_luma_samples − 1,
			( subpic_ctu_top_left_y[ CurrSubpicIdx ] +
			subpic_height_minus1[ CurrSubpicIdx ] + 1 ) * CtbSizeY − 1 )
}

sh_extra_bit[ i ] may be equal to 1 or 0. Decoders conforming to this version of this Specification shall ignore the value of sh_extra_bit[ i ]. Its value does not affect decoder conformance to profiles specified in this version of specification.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document of Joint Video Experts Team, JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document of Joint Video Experts Team, JVET-Q2001, 17th Meeting: Brussels, BE, 7–17 January 2020.

- VTM8.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM8.0

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).