| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0249 |
| --- | --- |

| Title: | AHG9: Proposed structural text changes to HLS in the VVC specification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian Zhi Zhang Jack Enhorn | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com zhi.a.zhang@ericsson.com jack.enhorn@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes the following structural text changes to the VVC specification:

- Replace ‘slice_’ prefix for slice header syntax elements by ‘sh_’, except for slice_address and slice_type which are proposed to be renamed to sh_slice_address and sh_slice_type, respectively. (editorial)

- Rename syntax elements in VPS, SPS, PPS, PH, SH to ensure that the names of all syntax elements in these places start with ‘vps_’, ‘sps_’, ‘pps_’, ‘ph_’ and ‘sh_’, respectively. (editorial)

- Change disable flags to enable flags throughout the VVC specification, i.e. change the following syntax elements to enable flags (normative, but only because the bits for these flags in the bitstream are inverted):

  - pps_deblocking_filter_disabled_flag

  - ph_disable_bdof_flag

  - ph_disable_dmvr_flag

  - ph_disable_prof_flag

  - ph_deblocking_filter_disabled_flag

  - scaling_matrix_for_lfnst_disabled_flag

  - slice_deblocking_filter_disabled_flag

  - slice_ts_residual_coding_disabled_flag

Proposed changes 1 and 2 are claimed to make it possible to see the scope and placement of syntax elements from their names alone. Proposed change 3 is claimed to align all enable/disable flags so that a ‘1’ always means enable and ‘0’ always means disable, thereby improving readability.

Specification text on top of JVET-Q2001-vE for replacing the disable flags with enable flags is provided with the contribution.

# Proposals

## Replace ‘slice_’ prefix in slice headers with ‘sh_’

At the 17th JVET meeting in Brussels it was decided to consistently prefix syntax elements in the picture header with ‘ph_’ instead of ‘pic_’. In line with that decision we propose to prefix slice header syntax elements with ‘sh_’ instead of ‘slice_’, except for slice_address and slice_type where it is believed that ‘slice_’ refers to the actual slice and not the slice header.

## Prefix all syntax elements in VPS, SPS, PPS, PH, SH with ‘vps_’, ‘sps_’, ‘pps_’, ‘ph_’ and ‘sh_’

The authors believe that the scope and placement of a syntax element may not always be obvious at a first glance when reading the VVC specification. To improve readability, we therefore propose to prefix each syntax element in the parameter sets and headers with a prefix that shows where the syntax element is signaled. It is proposed to rename syntax elements in VPS, SPS, PPS, PH, SH to ensure that the names of all syntax elements in these places start with ‘vps_’, ‘sps_’, ‘pps_’, ‘ph_’ and ‘sh_’, respectively, regardless whether a syntax element has a counterpart in another header or parameter set or not.

## Change disable flags to enable flags

VVC contains a number of flags for turning coding tools on or off from the parameter sets, picture or slice headers. Most of these flags are enabling flags, whereas just a few, eight flags to be exact, are disabling flags. We propose to replace the disabling flags in VVC with corresponding enable flags for consistency in the VVC specification. We believe this improves readability and comprehensibility and that it removes a number of double negations in the syntax and text.

The following eight disable flags are proposed to be changed to enable flags:

  - pps_deblocking_filter_disabled_flag

  - ph_disable_bdof_flag

  - ph_disable_dmvr_flag

  - ph_disable_prof_flag

  - ph_deblocking_filter_disabled_flag

  - scaling_matrix_for_lfnst_disabled_flag

  - slice_deblocking_filter_disabled_flag

  - slice_ts_residual_coding_disabled_flag

Proposed text on top of JVET-Q2001-vE is provided with the contribution. Below are three examples from the proposed text illustrating how double negations are avoided. Changes are marked in yellow with removed text in red strike-through.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| ph_deblocking_filter_disabledenabled_flag | u(1) |
| if( !ph_deblocking_filter_disabledenabled_flag ) { |  |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| … |  |
| } |  |

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( !transform_skip_flag[ x0 ][ y0 ][ 0 ] ¦¦&& slice_ts_residual_coding_disabledenabled_flag ) |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| … |  |
| } |  |

ph_enabledisable_bdof_flag equal to 01 specifies that bi-directional optical flow inter prediction based inter bi-prediction is disabled in the slices associated with the PH. ph_enabledisable_bdof_flag equal to 10 specifies that bi-directional optical flow inter prediction based inter bi-prediction may or may not be enabled in the slices associated with the PH.

When ph_enabledisable_bdof_flag is not present, the following applies:

- If sps_bdof_enabled_flag is equal to 1, the value of ph_enabledisable_bdof_flag is inferred to be equal to 0.

- Otherwise (sps_bdof_enabled_flag is equal to 0), the value of ph_disable_bdof_flag is inferred to be equal to 1.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).