| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0250 |
| --- | --- |

| Title: | AHG9: On B-slice signaling in the PH and derivation of slice_type |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Ruoyang Yu Rickard Sjöberg Mitra Damghanian Zhi Zhang Jack Enhorn | Email: | martin.m.pettersson@ericsson.com ruoyang.yu@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com zhi.zhang@ericsson.com jack.enhorn@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes the following two changes to the VVC specification:

- Signal a flag ph_inter_B_slice_allowed_flag in the PH when ph_inter_slice_allowed_flag is equal to 1 and condition the presence of ph_collocated_from_l0_flag, mvd_l1_zero_flag, ph_disable_bdof_flag, ph_disable_dmvr_flag, and pred_weight_table( ) on ph_inter_B_slice_allowed_flag.

- Use the proposed ph_inter_B_slice_allowed_flag to increase the number of cases when the value of slice_type is inferred without being signaled.

It is claimed that the proposed ph_inter_B_slice_allowed_flag will control the presence of the syntax elements in the PH that are used only for bi-prediction in a similar way to how it is done in the slice header, and allows preventing signalling of syntax elements that are used only for bi-prediction when there are no B slices in the picture associated with the PH. It is claimed that signalling of syntax elements that has no meaning should in general be avoided.

It is further claimed that the ph_inter_B_slice_allowed_flag provides an easy way to check whether a picture is a B-picture or not. It is asserted that this would otherwise require checking whether there is any slice in the picture with slice_type equal to B.

It is also claimed that the ph_inter_B_slice_allowed_flag provides some bit savings.

# Introduction

At the 17th JVET meeting in Brussels a two-flag approach was adopted for structuring of PH syntax elements based on if they are only used for intra slices or inter slices. The meeting notes states the following:

Decision (cleanup): Adopt two-flag approach for structuring of PH syntax elements. Text in JVET-Q0819.

Hypothetically, a third flag could distinguish B-only and P-only from a mixture when I slices are indicated not to be present. Hypothetically, slice_type could become a flag or be absent under relevant conditions.

This contribution addresses the hypothetical remarks from the meeting notes by proposing a ph_inter_B_slice_allowed_flag and condition several syntax elements in the PH on this flag. It is also proposed to derive the slice_type in certain cases based on the values of ph_inter_B_slice_allowed_flag, ph_inter_slice_allowed_flag, ph_intra_slice_allowed_flag and picture_header_in_slice_header_flag.

The ph_inter_B_slice_allowed_flag controls the presence of the syntax elements in the PH that are used only for bi-prediction, similar to how it is done in the slice header. This allows preventing signalling of syntax elements that are used only for bi-prediction when there are no B slices in the picture associated with the PH. We believe that signalling of syntax elements that has no meaning should in general be avoided.

The ph_inter_B_slice_allowed_flag also provides an easy way to check whether a picture is a B-picture or not. This would otherwise require checking that at least one slice in the picture has slice_type equal to B. The flag also This effectively prevents enabling bi-prediction tools when these may not be used for the picture. Another benefit is that the flag provides some bit savings.

# Proposal

The following changes are proposed to the VVC specification:

- Signal a flag ph_inter_B_slice_allowed_flag in the PH when ph_inter_slice_allowed_flag is equal to 1 and condition the presence of ph_collocated_from_l0_flag, mvd_l1_zero_flag, ph_disable_bdof_flag, ph_disable_dmvr_flag, and pred_weight_table( ) on ph_inter_B_slice_allowed_flag.

- Signal slice_type in the slice header only when ph_inter_slice_allowed_flag is equal to 1 and picture_header_in_slice_header_flag is equal to 0 and either one of ph_intra_slice_allowed_flag or ph_inter_B_slice_allowed_flag is equal to 1, otherwise infer slice_type as equal to:

  - 2 (I slice), if ph_inter_slice_allowed_flag is equal to 0.

  - 1 (P slice), if ph_inter_slice_allowed_flag is equal to 1 and ph_inter_B_slice_allowed_flag is equal to 0.

  - 0 (B slice), if ph_inter_slice_allowed_flag is equal to 1 and ph_inter_B_slice_allowed_flag is equal to 1.

Proposed syntax and semantics on top of JVET-Q2001-vC are shown below with added text in yellow and removed text in red strike-through.

## Signal a ph_inter_B_slice_allowed_flag in the PH

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) { |  |
| ph_intra_slice_allowed_flag | u(1) |
| ph_inter_B_slice_allowed_flag | u(1) |
| } |  |
| … |  |
| if( ph_inter_slice_allowed_flag ) { |  |
| … | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| ph_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| ph_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| if( sps_temporal_mvp_enabled_flag ) { |  |
| ph_temporal_mvp_enabled_flag | u(1) |
| if( ph_temporal_mvp_enabled_flag && rpl_info_in_ph_flag ) { |  |
| if( ph_inter_B_slice_allowed_flag ) |  |
| ph_collocated_from_l0_flag | u(1) |
| if( ( ph_collocated_from_l0_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( !ph_collocated_from_l0_flag && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) |  |
| ph_collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| mvd_l1_zero_flag | u(1) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| ph_fpel_mmvd_enabled_flag | u(1) |
| if( ph_inter_B_slice_allowed_flag ) { |  |
| mvd_l1_zero_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| ph_disable_bdof_flag |  |
| if( sps_dmvr_pic_present_flag ) |  |
| ph_disable_dmvr_flag |  |
| } |  |
| if( sps_prof_pic_present_flag ) |  |
| ph_disable_prof_flag | u(1) |
| if( ( pps_weighted_pred_flag ¦ ¦ ( pps_weighted_bipred_flag && ph_inter_B_slice_allowed_flag) ) && wp_info_in_ph_flag ) |  |
| pred_weight_table( ) |  |
| } |  |
| … |  |
| } |  |

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 0 or 1.

ph_intra_slice_allowed_flag equal to 0 specifies that allno coded slices of the picture have slice_type equal to 0 or 12. ph_intra_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 2.When not present, the value of ph_intra_slice_allowed_flag is inferred to be equal to 1.

ph_inter_B_slice_allowed_flag equal to 0 specifies that no coded slice of the picture have slice_type equal to 0. ph_inter_B_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 0. When not present, ph_inter_B_slice_allowed_flag is inferred to be equal to 0.

ph_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. ph_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1. When not present, ph_collocated_from_l0_flag is inferred to be equal to 1.

mvd_l1_zero_flag equal to 1 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is not parsed and MvdL1[ x0 ][ y0 ][ compIdx ] and MvdCpL1[ x0 ][ y0 ][ cpIdx ][ compIdx ] are set equal to 0 for compIdx = 0..1 and cpIdx = 0..2. mvd_l1_zero_flag equal to 0 when ph_inter_B_slice_allowed_flag is equal to 1 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is parsed. When not present, mvd_l1_zero_flag is inferred to be equal to 0.

## Derivation of slice_type

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if( picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| … |  |
| if( ph_inter_slice_allowed_flag && !picture_header_in_slice_header_flag && ( ph_intra_slice_allowed_flag ¦ ¦ ph_inter_B_slice_allowed_flag ) ) |  |
| slice_type | ue(v) |
| … |  |
| } |  |

slice_type specifies the coding type of the slice according to Table 9.

Table 9 – Name association to slice_type

| slice_type | Name of slice_type |
| --- | --- |
| 0 | B (B slice) |
| 1 | P (P slice) |
| 2 | I (I slice) |

When not present, the following applies:

- If ph_inter_slice_allowed_flag is equal to 0, the value of slice_type is inferred to be equal to 2.

- Otherwise, if ph_inter_slice_allowed_flag is equal to 1 and ph_inter_B_slice_allowed_flag is equal to 0, the value of slice_type is inferred to be equal to 1.

- Otherwise, (ph_inter_slice_allowed_flag is equal to 1 and ph_inter_B_slice_allowed_flag is equal to 1) the value of slice_type is inferred to be equal to 0.

When ph_intra_slice_allowed_flag is equal to 0, the value of slice_type shall be equal to 0 or 1. When ph_inter_B_slice_allowed_flag is equal to 0, the value of slice_type shall be equal to 1 or 2. When nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, inclusive, and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, slice_type shall be equal to 2.

…

# Table of inference

The following table shows how slice_type is inferred in this proposal according to the text in section 2.2 above.

| PH in SH | ph_inter_slice_allowed_flag | ph_intra_slice_allowed_flag | ph_inter_B_slice_allowed_flag | slice_type | slice_type |
| --- | --- | --- | --- | --- | --- |
| 0 or 1 | 0 | 1 (inferred) | 0 (inferred) | Inferred | 2 (I slice) |
| 1 | 1 | 0 or 1 | 0 | Inferred | 1 (P slice) |
| 1 | 1 | 0 or 1 | 1 | Inferred | 0 (B slice) |
| 0 | 1 | 0 | 0 | Inferred | 1 (P slice) |
| 0 | 1 | 0 | 1 | Signalled | P or B |
| 0 | 1 | 1 | 0 | Signalled | I or P |
| 0 | 1 | 1 | 1 | Signalled | I, P or B |

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).