| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0251 |
| --- | --- |

| Title: | AHG9: Fixes related to the picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian Zhi Zhang Jack Enhorn | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com zhi.a.zhang@ericsson.com jack.enhorn@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the picture header in the VVC specification:

- Rename the syntax elements pic_sign_data_hiding_enabled_flag, sps_bdof_pic_present_flag, sps_dmvr_pic_present_flag and sps_prof_pic_present_flag to ph_sign_data_hiding_enabled_flag, sps_bdof_control_present_in_ph_flag, sps_dmvr_control_present_in_ph_flag and sps_prof_control_present_in_ph_flag, respectively. (editorial)

- Change the semantics for ph_alf_enabled_flag such that when ph_alf_enabled_flag is present and equal to 0, adaptive loop filter is disabled for all color components in all slices associated with the PH. In the current version of VVC it is stated that when ph_alf_enabled_flag is equal to 0, the adaptive loop filter may be disabled for one, or more, or all slices associated with the PH. (expressed intent)

- Change the semantics for rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, alf_info_in_ph_flag, wp_info_in_ph_flag and qp_delta_info_in_ph_flag by removing the text “that do not contain a PH syntax structure” such that when these flags are equal to 0, the corresponding parameters may be present in slice headers that contain a PH syntax structure. It is further proposed to replace may with shall in the semantics for qp_delta_info_in_ph_flag, such that qp_delta_info_in_ph_flag equal to 0 specifies that QP delta information is not present in the PH syntax structure and shall be present in slice headers referring to the PPS. (expressed intent)

All proposed items above are independent from each other and may be considered for adoption separately.

# Proposals

## Rename ‘pic’ to ‘ph’

In line with the decision at the 17th JVET meeting in Brussels to change the prefix for picture header parameters from ‘pic’ to ‘ph’ it is proposed to do a similar renaming for a few syntax elements that were added at the same meeting.

To be precise, it is proposed to rename pic_sign_data_hiding_enabled_flag, sps_bdof_pic_present_flag, sps_dmvr_pic_present_flag and sps_prof_pic_present_flag to ph_sign_data_hiding_enabled_flag, sps_bdof_control_present_in_ph_flag, sps_dmvr_control_present_in_ph_flag and sps_prof_control_present_in_ph_flag, respectively.

## Disable ALF for all slices when ph_alf_enabled_flag is 0

In the current version of VVC, ALF may be enabled at SPS level and may be disabled at picture level or slice level by setting ph_alf_enabled_flag equal to 0 or slice_alf_enabled_flag equal to 0, respectively. For each picture, only one of ph_alf_enabled_flag and slice_alf_enabled_flag may be signaled, which one is conditioned on the value of the PPS syntax element alf_info_in_ph_flag. The semantics for ph_alf_enabled_flag states that when ph_alf_enabled_flag is equal to 0, the adaptive loop filter may be disabled for one, or more, or all slices associated with the PH.

We believe this is imprecise and that the intent is to disallow ALF for all slices in a picture when ph_alf_enabled_flag is set to 0. Also, similar to when slice_alf_enabled_flag is equal to 0, we believe ALF should be disallowed for all color components when ph_alf_enabled_flag is equal to 0.

We therefore propose that when ph_alf_enabled_flag is equal to 0, sps_alf_enabled_flag is equal to 1 and alf_info_in_ph_flag is equal to 1, the adaptive loop filter is disabled for all color components in all slices associated with the PH.

Proposed changes on top of the VVC draft (JVET-Q2001-vE) are shown below with added text in yellow and removed text in red strike-through.

### Proposed text changes

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && !alf_info_in_ph_flag ) |  |
| slice_alf_enabled_flag | u(1) |
| … |  |
| } |  |
| … |  |
| } |  |

slice_alf_enabled_flag equal to 1 specifies that adaptive loop filter is enabled and may be applied to Y, Cb, or Cr colour component in a slice. slice_alf_enabled_flag equal to 0 specifies that adaptive loop filter is disabled for all colour components in a slice. When not present, the value of slice_alf_enabled_flag is inferred to be equal to ph_alf_enabled_flag.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && alf_info_in_ph_flag ) { |  |
| ph_alf_enabled_flag | u(1) |
| … |  |
| } |  |
| … |  |
| } |  |

ph_alf_enabled_flag equal to 1 specifies that adaptive loop filter is enabled for all slices associated with the PH and may be applied to Y, Cb, or Cr colour component in the  slices. ph_alf_enabled_flag being present and equal to 0 specifies that adaptive loop filter may beis disabled for all colour components in one, or more, or all slices associated with the PH. When not present, ph_alf_enabled_flag is inferred to be equal to 0.

## Change semantics for PH or SH PPS flags

At the 17th JVET meeting in Brussels a set of flags were added to PPS for signaling whether certain parameters for RPL, deblocking, SAO, ALF, weighted prediction and QP delta are present in the picture header or the slice header. The PPS flags are named rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, alf_info_in_ph_flag, wp_info_in_ph_flag and qp_delta_info_in_ph_flag. We propose some minor clean-ups to the semantics for these flags.

Firstly, in the current version of VVC, the semantics for each of the PPS flags says that when the flag is equal to 0, the corresponding parameters are “not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure.” We believe the corresponding parameters may also be present in slice headers that contain a PH syntax structure and propose to remove “that do not contain a PH syntax structure” from the semantics of the PPS flags. What we believe is that any of the flags can be equal to 0 when picture_header_in_slice_header_flag is equal to 1.

Secondly, since the slice_qp_delta syntax element in the slice header is only conditioned on qp_delta_info_in_ph_flag being false, we believe slice_qp_delta shall be present in slice headers when qp_delta_info_in_ph_flag is equal to 0. We therefore propose replacing the may with a shall in the semantics for qp_delta_info_in_ph_flag. We don’t propose that for other PPS flags mentioned above, since the information corresponding to those flags are also conditioned on other syntax elements.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !qp_delta_info_in_ph_flag ) |  |
| slice_qp_delta | se(v) |
| … |  |
| } |  |

Proposed changes on top of the VVC draft (JVET-Q2001-vE) are shown below with added text in yellow and removed text in red strike-through.

### Proposed semantics

rpl_info_in_ph_flag equal to 1 specifies that reference picture list information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. rpl_info_in_ph_flag equal to 0 specifies that reference picture list information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure.

dbf_info_in_ph_flag equal to 1 specifies that deblocking filter information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. dbf_info_in_ph_flag equal to 0 specifies that deblocking filter information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure. When not present, the value of dbf_info_in_ph_flag is inferred to be equal to 0.

sao_info_in_ph_flag equal to 1 specifies that SAO filter information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. sao_info_in_ph_flag equal to 0 specifies that SAO filter information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure

alf_info_in_ph_flag equal to 1 specifies that ALF information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. alf_info_in_ph_flag equal to 0 specifies that ALF information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure.

wp_info_in_ph_flag equal to 1 specifies that weighted prediction information may be present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. wp_info_in_ph_flag equal to 0 specifies that weighted prediction information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure. When not present, the value of wp_info_in_ph_flag is inferred to be equal to 0.

qp_delta_info_in_ph_flag equal to 1 specifies that QP delta information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. qp_delta_info_in_ph_flag equal to 0 specifies that QP delta information is not present in the PH syntax structure and mayshall be present in slice headers referring to the PPS that do not contain a PH syntax structure.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).