| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0252 |
| --- | --- |

| Title: | AHG9: On high-level signaling of mvd_l1_zero_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Ruoyang Yu Rickard Sjöberg Mitra Damghanian Zhi Zhang Jack Enhorn Du Liu | Email: | martin.m.pettersson@ericsson.com ruoyang.yu@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com zhi.a.zhang@ericsson.com jack.enhorn@ericsson.com du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to move mvd_l1_zero_flag from the picture header back to the slice header. It is claimed that having mvd_l1_zero_flag in the slice header provides the same flexibility as in HEVC and may be useful when RPL information is different between slices, such as for subpictures with different random access points.

The contribution also proposes a claimed editorial fix to replace mvd_coding( x0, y0, 1 ) by mvd_coding( x0, y0, 1, cpIdx ) in the semantics for mvd_l1_zero_flag.

# Introduction

In the current version of VVC, mvd_l1_zero_flag is signalled in the picture header. In HEVC it is signalled in the slice header. The mvd_l1_zero_flag determines whether the mvd_coding( ) syntax structure is parsed for L1 or if the motion vector derivation information is set to 0 for L1.

In VVC, RPL information may be signaled in either the picture header or the slice header and the number of active reference pictures may be overridden in the slice header. Thus, the RPL and the number of active reference pictures can be very different between slices, and it may therefore be useful to have a different value of mvd_l1_zero_flag per slice. This is possible in HEVC, but not in the current version of VVC.

One example when it may be useful is when subpictures are used and there are different random access points for the subpictures in the picture. For instance, the figure below shows four sequential pictures in a bitstream with two subpictures per picture, each subpicture containing one slice, where the slice of each subpicture is either an IDR slice (intra slice with an IDR NUT) or a B-slice. In the picture with POC 3 the first slice may only have POC 2 as reference, while the second slice may have any of POC 0, POC 1 or POC 2 as reference. For the first slice it would make sense to have mvd_l1_zero_flag set to 1 while for the second slice it may make sense to have mvd_l1_zero_flag set to 0.

B-slice

B-slice

IDR slice

B-slice

B-slice

B-slice

IDR slice

IDR slice

POC 0

POC 1

POC 2

POC 3

# Proposal

It is proposed to move mvd_l1_zero_flag from the picture header back to the slice header in the VVC specification.

It is also proposed to editorially change mvd_coding( x0, y0, 1 ) to mvd_coding( x0, y0, 1, cpIdx ) in the semantics for mvd_l1_zero_flag. (editorial)

Proposed changes on top of the VVC draft (JVET-Q2001-vE) are shown below with added text in yellow and removed text in red strike-through.

## Proposed syntax and semantics

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| mvd_l1_zero_flag | u(1) |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !rpl_info_in_ph_flag && ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) |  |
| ref_pic_lists( ) |  |
| if( ( rpl_info_in_ph_flag ¦ ¦ ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) && ( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| if ( slice_type == B ) |  |
| mvd_l1_zero_flag | u(1) |
| … |  |
| } |  |

| mvd_coding( x0, y0, refList ,cpIdx ) { | Descriptor |
| --- | --- |
| … |  |
| } |  |

mvd_l1_zero_flag equal to 1 indicates that the mvd_coding( x0, y0, 1, cpIdx ) syntax structure is not parsed and MvdL1[ x0 ][ y0 ][ compIdx ] and MvdCpL1[ x0 ][ y0 ][ cpIdx ][ compIdx ] are set equal to 0 for compIdx = 0..1 and cpIdx = 0..2. mvd_l1_zero_flag equal to 0 indicates that the mvd_coding( x0, y0, 1, cpIdx ) syntax structure is parsed. When not present, mvd_l1_zero_flag is inferred to be equal to 0.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).