| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0255 |
| --- | --- |

| Title: | AHG9: Fixes related to RPL |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian Zhi Zhang Jack Enhorn | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com zhi.zhang@ericsson.com jack.enhorn@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to RPL in the VVC specification:

- Change the reference picture list structure semantics by replacing the non-defined parameters ph_rpl_idx[ listIdx ] and slice_rpl_idx [ listIdx ] with rpl_idx[ listIdx ]. (claimed cleanup)

- Remove conditioning the presence of num_ref_idx_active_override_flag in the slice header on the value of the rpl_info_in_ph_flag syntax element. (claimed expressed intent)

- Fix the inference of rpl_idx [ i ] such when not present, if rpl_sps_flag[ i ] is equal to 1 and rpl1_idx_present_flag is equal to 0, the value of rpl_idx[ 1 ] is inferred to be equal to rpl_idx[ 0 ], otherwise the value of rpl_idx[ i ] is inferred to be equal to 0. It is asserted that the inference of rpl_idx[ i ] is ambiguous in the current VVC specification. (claimed expressed intent)

All proposed items above are independent from each other and may be considered for adoption separately.

# Proposals

## Change semantics for reference picture list structure semantics

Since ph_rpl_idx[ listIdx ] and slice_rpl_idx [ listIdx ] was replaced by rpl_idx[ listIdx ] with the adoption of JVET-Q0273 at the Brussels meeting, ph_rpl_idx[ listIdx ] and slice_rpl_idx [ listIdx ] are no longer defined. It is therefore proposed to do the below modifications to the VVC specification. This issue was also pointed out in ticket #958 from Panasonic.

Proposed removed text on top of the VVC draft (JVET-Q2001-vE) are marked in red strike-through.

### Proposed semantics

### 7.4.10 	Reference picture list structure semantics

The ref_pic_list_struct( listIdx, rplsIdx ) syntax structure may be present in an SPS, in a PH syntax structure, or in a slice header. Depending on whether the syntax structure is included in an SPS, a PH syntax structure, or a slice header, the following applies:

–	If present in a PH syntax structure or a slice header, the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure specifies reference picture list listIdx of the current picture (the picture containing the slice).

–	Otherwise (present in an SPS), the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure specifies a candidate for reference picture list listIdx, and the term "the current picture" in the semantics specified in the remainder of this clause refers to each picture that 1) has a PH syntax structure containing ph_rpl_idx[ listIdx ] equal to an index into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures included in the SPS or one or more slices containing slice_rpl_idx[ listIdx ] equal to an index into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures included in the SPS, and 2) is in a CVS that refers to the SPS.

## Remove conditioning the presence of num_ref_idx_active_override_flag on rpl_info_in_ph_flag

In the current version of VVC, the number of active reference pictures is signaled in PPS and may be overridden in the slice header if the syntax element num_ref_idx_active_override_flag is equal to 1. The presence of num_ref_idx_active_override_flag is conditioned on a number of syntax elements including rpl_info_in_ph_flag.

We believe the override of the number of active reference pictures in slice header should be the same regardless of if RPL info is signaled in the picture header or in slice headers. We therefore propose to remove conditioning the presence of num_ref_idx_active_override_flag on the value of the rpl_info_in_ph_flag syntax element.

Proposed changes on top of the VVC draft (JVET-Q2001-vE) are shown below with removed text in red strike-through.

### Proposed syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !rpl_info_in_ph_flag && ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) |  |
| ref_pic_lists( ) |  |
| if( ( rpl_info_in_ph_flag ¦ ¦ ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) && ( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| … |  |
| } |  |

## Fix inference of rpl_idx[ i ] when not present

We believe that the inference of rpl_idx[ i ] is clear in the current VVC specification. The semantics of rpl_idx[ i ] states the following which seems contradictive:

- When not present, the value of rpl_idx[ i ] is inferred to be equal to 0.

- When not present, if rpl_sps_flag[ i ] is equal to 1 and rpl1_idx_present_flag is equal to 0, the value of rpl_idx[ 1 ] is inferred to be equal to rpl_idx[ 0 ]

We therefore propose to fix the inference of rpl_idx [ i ]  such when not present, if rpl_sps_flag[ i ] is equal to 1 and rpl1_idx_present_flag is equal to 0, the value of rpl_idx[ 1 ] is inferred to be equal to rpl_idx[ 0 ], otherwise the value of rpl_idx[ i ] is inferred to be equal to 0.

Proposed changes on top of the VVC draft (JVET-Q2001-vE) are shown below with added text in yellow and removed text in red strike-through.

### Proposed semantics

rpl_idx[ i ] specifies the index, into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i included in the SPS, of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is used for derivation of reference picture list i of the current picture. The syntax element rpl_idx[ i ] is represented by Ceil( Log2( num_ref_pic_lists_in_sps[ i ] ) ) bits. When not present, the value of rpl_idx[ i ] is inferred to be equal to 0. The value of rpl_idx[ i ] shall be in the range of 0 to num_ref_pic_lists_in_sps[ i ] − 1, inclusive. When rpl_sps_flag[ i ] is equal to 1 and num_ref_pic_lists_in_sps[ i ] is equal to 1, the value of rpl_idx[ i ] is inferred to be equal to 0. When not present, if rpl_sps_flag[ i ] is equal to 1 and rpl1_idx_present_flag is equal to 0, the value of rpl_idx[ 1 ] is inferred to be equal to rpl_idx[ 0 ], otherwise the value of rpl_idx[ i ] is inferred to be equal to 0.

The variable RplsIdx[ i ] is derived as follows:

RplsIdx[ i ] = rpl_sps_flag[ i ] ? rpl_idx[ i ] : num_ref_pic_lists_in_sps[ i ]	(149)

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).