| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0257 |
| --- | --- |

| Title: | AHG12: Raster scan order flag for subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mitra Damghanian Rickard Sjöberg Martin Pettersson Zhi Zhang Jack Enhorn Jacob Ström Ruoyang Yu | Email: | mitra.damghanian@ericsson.com rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com zhi.zhang@ericsson.com jack.enhorn@ericsson.com jacob.strom@ericsson.com ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to add an sps_raster_scan_order_subpics_flag in the SPS of VVC that specifies that subpictures are ordered in raster scan order in the bitstream. This contribution further proposes to skip the signalling of the top-left position of the subpictures in the SPS when sps_raster_scan_order_subpics_flag is equal to 1.

Proposal 1, SPS flag:

- Add an sps_raster_scan_order_subpics_flag to the SPS, where sps_raster_scan_order_subpics_flag equal to 1 specifies that subpictures are ordered in raster scan order in each picture in the CLVS such that for any two integer values i and j in the range of 0 to sps_num_subpics_minus1 inclusive, with j less than i, the position of the top left CTU of the i-th subpicture follows the position of the top left CTU of the j-th subpicture in raster scan order. sps_raster_scan_order_subpics_flag equal to 0 does not impose such a constraint.

Proposal 2, Subpicture signalling:

- Condition the signalling of the top-left position of the subpictures on sps_raster_scan_order_subpics_flag being equal to 0.

Proposal 1 is assessed by the proponents to be independent from proposal 2.

The current design of VVC specifies an availability rule for subpictures which requires “that the shapes of the subpictures shall be such that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures”.

The proponents claim that the current design generally does not provide a unique order of the subpictures in the bitstream, and that by introducing a raster scan order flag, a shortcut is provided for a mode to specify a raster scan order for the subpictures while conforming to the availability rule. Raster scan ordering of subpictures, which is also a unique ordering of subpictures in the bitstream, is claimed to be useful for extraction and merge purposes and to provide a hook for, e.g., external use.

For use cases with a regular grid of subpictures or when the heights of the subpictures do not change within a row, the proposed changes are claimed to reduce the signalling bit cost.

Specification text on top of JVET-Q2001-vE for proposal 1 and 2 are provided with the contribution.

# Introduction / Problem Statement

Subpictures in the current VVC specification are signalled in the SPS using the following syntax:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( subpic_id_mapping_explicitly_signalled_flag ) { |  |
| subpic_id_mapping_in_sps_flag | u(1) |
| if( subpic_id_mapping_in_sps_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

In the current VVC design, the availability rule, as quoted below from JVET-Q2001-vE, constrains the order of the subpictures:

“It is a requirement of bitstream conformance that the shapes of the subpictures shall be such that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures.”

The above rule constrains the order of the subpictures in the bitstream and so the decoding order of the subpictures. There is also a similar availability rule for slices in the current VVC design:

“It is a requirement of bitstream conformance that the following constraints apply:

- …

- The shapes of the slices of a picture shall be such that each CTU, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded CTU(s).”

A subpicture consists of one or more slices and so consists of one or more VCL NAL units. The availability rule limits the ordering of the subpictures in the bitstream but generally does not provide a unique order for the subpictures and their associated NAL units in the bitstream.

# Proposal

The flexibility from the availability rule is required to cover a broad range of applications while we claim that introducing the option of raster scan order subpictures, which also comply with the availability rule, is useful for extraction and merging purposes since it guarantees a unique and simple order of the subpictures in the bitstream. The raster scan order subpicture mode also provides a shortcut for specifying the subpicture order in use cases with a regular grid of subpictures or when the heights of the subpictures do not change within one row. In these use cases the signalling of the subpictures using raster scan order can also be simplified by skipping the signalling of the top left corner of the subpictures and deriving it instead.

## Proposal 1: SPS flag

It is proposed to add an sps_raster_scan_order_subpics_flag to the SPS, where sps_raster_scan_order_subpics_flag equal to 1 specifies that the subpictures in the CLVS are signalled in raster scan order and sps_raster_scan_order_subpics_flag equal to 0 does not impose such a constraint.

Proposed syntax and semantics on top of JVET-Q2001-vE are shown with added text in yellow.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| if( sps_num_subpics_minus1 > 0 ) |  |
| sps_raster_scan_order_subpics_flag | u(1) |
| … |  |
| } |  |

sps_num_subpics_minus1 plus 1 specifies the number of subpictures in each picture in the CLVS. The value of sps_num_subpics_minus1 shall be in the range of 0 to Ceil( pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( pic_height_max_in_luma_samples ÷ CtbSizeY ) − 1, inclusive. When not present, the value of sps_num_subpics_minus1 is inferred to be equal to 0.

sps_raster_scan_order_subpics_flag equal to 1 specifies that subpictures are ordered in raster scan order in each picture in the CLVS such that for any two integer values i and j in the range of 0 to sps_num_subpics_minus1 inclusive, with j less than i, the position of the top left CTU of the i-th subpicture follows the position of the top left CTU of the j-th subpicture in raster scan order. sps_raster_scan_order_subpics_flag equal to 0 does not impose such a constraint.

## Proposal 2: Signalling

It is proposed to add sps_raster_scan_order_subpics_flag to the SPS and condition the signalling of the top-left corner of the subpictures position on sps_raster_scan_order_subpics_flag equal to 0.

Proposed syntax and semantics on top of JVET-Q2001-vE are shown below with added text in yellow and removed text in red strike-through.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| if( sps_num_subpics_minus1 > 0 ) |  |
| sps_raster_scan_order_subpics_flag | u(1) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && !sps_raster_scan_order_subpics_flag && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && !sps_raster_scan_order_subpics_flag && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( subpic_id_mapping_explicitly_signalled_flag ) { |  |
| subpic_id_mapping_in_sps_flag | u(1) |
| if( subpic_id_mapping_in_sps_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

sps_num_subpics_minus1 plus 1 specifies the number of subpictures in each picture in the CLVS. The value of sps_num_subpics_minus1 shall be in the range of 0 to Ceil( pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( pic_height_max_in_luma_samples ÷ CtbSizeY ) − 1, inclusive. When not present, the value of sps_num_subpics_minus1 is inferred to be equal to 0.

sps_raster_scan_order_subpics_flag equal to 1 specifies that subpictures are ordered in raster scan order in each picture in the CLVS such that for any two integer values i and j in the range of 0 to sps_num_subpics_minus1 inclusive, with j less than i, the position of the top left CTU of the i-th subpicture follows the position of the top left CTU of the j-th subpicture in raster scan order. sps_raster_scan_order_subpics_flag equal to 0 does not impose such a constraint.

The lists SubpicCtuTopLeftX[ i ] and SubpicCtuTopLeftY[ i ], for i ranging from 0 to sps_num_subpics_minus1, inclusive, specifying the horizontal and vertical position of top left CTU of i-th subpicture in unit of CtbSizeY are derived as follows:

if( sps_raster_scan_order_subpics_flag ) {
	for( i = 0; i < PicSizeInCtbsY; i++ ) AssignedCtu[ i ] = 0
	for( i = 0; i <= sps_num_subpics_minus1; i++ ) {
		s = 0
		while( AssignedCtu[ s ] ) s++
		SubpicCtuTopLeftX[ i ] = s % PicWidthInCtbsY
		SubpicCtuTopLeftY[ i ] = s / PicWidthInCtbsY
		for( s = SubpicCtuTopLeftX[ i ]; s <= SubpicCtuTopLeftX[ i ] + subpic_width_minus1[ i ]; s++ ) 
			for( t = SubpicCtuTopLeftY[ i ]; t <= SubpicCtuTopLeftY[ i ] + subpic_height_minus1[ i ]; t++ ) 
				AssignedCtu[ s + t * PicWidthInCtbsY ] = 1
	}
} else {
	for( i = 0; i <= sps_num_subpics_minus1; i++ ) {
		SubpicCtuTopLeftX[ i ] = subpic_ctu_top_left_x[ i ]
		SubpicCtuTopLeftY[ i ] = subpic_ctu_top_left_y[ i ] 
	}
}

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).