| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0258 |
| --- | --- |

| Title: | AHG9: Reduce redundant signaling in picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jack Enhorn Martin Pettersson Rickard Sjöberg Mitra Damghanian Zhi Zhang | Email: | jack.enhorn@ericsson.com martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com zhi.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document contains two proposals claimed to reduce the picture header bit count. The proponents claim that the total number of PH+SH bits for RA has increased by approximately 22% from VTM-7.0 to VTM-8.0. Note that picture headers were implemented first in VTM-8.0. The following two modifications are proposed:

- Add three SPS flags sps_temporal_mvp_ph_present_flag, sps_fpel_mmvd_ph_present_flag  and sps_dep_quant_ph_present_flag to control the presence of ph_temporal_mvp_enabled_flag, ph_fpel_mmvd_enabled_flag and ph_dep_quant_enabled_flag in the picture header. (normative)

- Set sps_bdof_pic_present_flag, sps_dmvr_pic_present_flag and sps_prof_pic_present_flag to 0 in the CTC RA. The proponents claim that the values for the corresponding flags controlled by these flags do not change in the CTC. (encoder only)

The proponents point out that the two proposed modifications may be considered for adoption independently from each other.

The proponents claim that the two changes together would reduce the total number of picture header bits for the CTC RA bitstreams by 12%.

# Introduction

Using VTM with tracing and calculating the total number of bits used for slice and picture headers for the CTC bitstreams an increase of bits can be seen when comparing VTM-8.0 with VTM-7.0 and HM-16.21 as picture headers were introduced.

Table 1 - Bits for picture and slice headers

| CTC Random Access | Total picture header bits | Total slice header bits | Total header bits |
| --- | --- | --- | --- |
| HM-16.21 | N/A | 2 934 110 | 2 934 110 |
| VTM-7.0 | N/A | 2 871 550 | 2 871 550 |
| VTM-8.0 | 1 965 596 | 1 606 023 | 3 571 619 |
| Proposal 1 | 1 843 956 | 1 606 023 | 3 449 979 |
| Proposal 2 | 1 844 900 | 1 606 023 | 3 450 923 |
| Proposal 1+2 | 1 723 260 | 1 606 023 | 3 329 283 |

As seen in Table 1, before the introduction of picture headers the total number of bits for slice header in VTM was comparable to HM under CTC RA configurations. However, in VTM-8.0 there is a bit-count increase of approximately 22% for CTC RA compared to HM-16.21 counting both slice and picture headers.

In this document, changes to the picture header signaling of the VVC draft and the VTM software are proposed to reduce this increase of header bit data.

# Proposals

It is proposed to reduce the increase of header bit signaling by avoiding redundant signaling of flags in the picture header. Firstly, new control flags to SPS to signal whether corresponding picture header flags are present are proposed. Secondly, we propose to utilize existing SPS control flags in VTM to avoid unnecessary picture header flag signaling for the CTC test cases.

The proposed changes together would reduce the average picture header bits by 12% resulting in increased picture plus slice header bits for VTM compared to HM-16.21 of 13% for RA.

## Introduction of additional SPS present in PH flags

Studying the trace of VTM-8.0 picture headers it was noticed that the values of the three flags ph_temporal_mvp_enabled_flag, ph_fpel_mmvd_enabled_flag and ph_dep_quant_enabled_flag never changes in the CTC and thus always correspond to the respective flags in SPS. It is proposed to signal whether these flags are present in picture header by new proposed SPS flags named sps_temporal_mvp_enabled_ph_present_flag, sps_fpel_mmvd_enabled_ph_present_flag and sps_dep_quant_enabled_ph_present_flag, respectively.

The proposed VVC draft text changes are shown below. Added text is shown in yellow and removed text is shown in red strikethrough text.

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) { |  |
| sps_temporal_mvp_ph_present_flag | u(1) |
| sps_sbtmvp_enabled_flag | u(1) |
| } |  |
| … |  |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_ph_present _flag | u(1) |
| … |  |
| sps_dep_quant_enabled_flag | u(1) |
| if( sps_dep_quant_enabled_flag ) |  |
| sps_dep_quant_ph_present_flag | u(1) |
| … |  |
| } |  |

7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ph_inter_slice_allowed_flag ) { |  |
| … |  |
| if( sps_temporal_mvp_enabled_flag ) { |  |
| if( sps_temporal_mvp_ph_present_flag ) |  |
| ph_temporal_mvp_enabled_flag | u(1) |
| if( ph_temporal_mvp_enabled_flag && rpl_info_in_ph_flag ) { |  |
| … |  |
| } |  |
| … |  |
| if( sps_fpel_mmvd_enabled_flag sps_fpel_ mmvd_ph_present_flag ) |  |
| ph_fpel_mmvd_enabled_flag | u(1) |
| … |  |
| } |  |
| … |  |
| if( sps_dep_quant_enabled_flag sps_dep_quant_ph_present_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| … |  |
| } |  |

7.4.3.3	Sequence parameter set RBSP semantics

…

sps_temporal_mvp_ph_present_flag equal to 1 specifies that ph_temporal_mvp_enabled_flag is present in PHs referring to the SPS. sps_temporal_mvp_ph_present_flag equal to 0 specifies that ph_temporal_mvp_enabled_flag is not present in PHs referring to the SPS. When not present, the value of sps_temporal_mvp_ph_present_flag is inferred to be equal to 0.

…

sps_fpel_mmvd_ph_present_flag equal to 1 specifies that ph_fpel_mmvd_enabled_flag is present in PHs referring to the SPS. sps_fpel_mmvd_ph_present_flag equal to 0 specifies that ph_fpel_mmvd_enabled_flag is not present in PHs referring to the SPS. When not present, the value of sps_fpel_mmvd_ph_present_flag is inferred to be equal to 0.

…

sps_dep_quant_ph_present_flag equal to 1 specifies that ph_dep_quant_enabled_flag is present in PHs referring to the SPS. sps_dep_quant_ph_present_flag equal to 0 specifies that ph_dep_quant_enabled_flag is not present in PHs referring to the SPS. When not present, the value of sps_dep_quant_ph_present_flag is inferred to be equal to 0.

…

7.4.3.7	Picture header structure semantics

…

ph_temporal_mvp_enabled_flag specifies whether temporal motion vector predictors can be used for inter prediction for slices associated with the PH. If ph_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the slices associated with the PH shall be constrained such that no temporal motion vector predictor is used in decoding of the slices. Otherwise (ph_temporal_mvp_enabled_flag is equal to 1), temporal motion vector predictors may be used in decoding of the slices associated with the PH. When not present, the value of ph_temporal_mvp_enabled_flag is inferred to be equal to 0.

When ph_temporal_mvp_enabled_flag is not present, the following applies:

- If ph_inter_slice_allowed_flag is equal to 1 and sps_temporal_mvp_enabled_flag is equal to 1, the value of ph_temporal_mvp_enabled_flag is inferred to be equal to 1.

- Otherwise (ph_inter_slice_allowed_flag is equal to 0 or sps_temporal_mvp_enabled_flag is equal to 0), the value of ph_temporal_mvp_enabled_flag is inferred to be equal to 0.

When no reference picture in the DPB has the same spatial resolution as the current picture, the value of ph_temporal_mvp_enabled_flag shall be equal to 0.

The maximum number of subblock-based merging MVP candidates, MaxNumSubblockMergeCand, is derived as follows:

if( sps_affine_enabled_flag )
	MaxNumSubblockMergeCand = 5 − five_minus_max_num_subblock_merge_cand	(87)
else
	MaxNumSubblockMergeCand = sps_sbtmvp_enabled_flag  &&  ph_temporal_mvp_enabled_flag

The value of MaxNumSubblockMergeCand shall be in the range of 0 to 5, inclusive.

…

ph_fpel_mmvd_enabled_flag equal to 1 specifies that merge mode with motion vector difference uses integer sample precision in the slices associated with the PH. ph_fpel_mmvd_enabled_flag equal to 0 specifies that merge mode with motion vector difference can use fractional sample precision in the slices associated with the PH. When not present, the value of ph_fpel_mmvd_enabled_flag is inferred to be 0.

When ph_fpel_mmvd_enabled_flag is not present, the following applies:

- If ph_inter_slice_allowed_flag is equal to 1 and sps_fpel_mmvd_enabled_flag is equal to 1, the value of ph_fpel_mmvd_enabled_flag is inferred to be equal to 1.

- Otherwise (ph_inter_slice_allowed_flag is equal to 0 or sps_fpel_mmvd_enabled_flag is equal to 0), the value of ph_fpel_mmvd_enabled_flag is inferred to be equal to 0.

…

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to sps_dep_quant_enabled_flag.

…

## VTM software changes

In the study conducted on picture header traces it was noted that the three flags ph_disable_bdof_flag, ph_disable_dmvr_flag and ph_disable_prof_flag are signaled in the CTC RA but the values never change between pictures, the values are 0 for all picture headers. It is proposed to set the existing corresponding SPS control flags sps_bdof_pic_present_flag, sps_dmvr_pic_present_flag and sps_prof_pic_present_flag to 0 in the VTM source code to avoid this redundant signaling.

The resulting software change is to replace the following lines of code in EncLib.cpp:

sps.setBdofControlPresentFlag(m_BIO);

sps.setDmvrControlPresentFlag(m_DMVR);

sps.setProfControlPresentFlag(m_PROF);

by:

sps.setBdofControlPresentFlag(false);

sps.setDmvrControlPresentFlag(false);

sps.setProfControlPresentFlag(false);

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).