| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0260 |
| --- | --- |

| Title: | AHG9: On decoding capability information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He Vadim Seregin Muhammed Coban Marta Karczewicz | Email: | YongHe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to move Decoding Capability Information (DCI) from parameter set to SEI message as the information is not necessary for decoding process. The proposed DCI SEI message carries the preferred decoding capability information applied to the entire bitstream.

# Proposal

In VVC draft 8, DCI parameter set is specified to convey the decoding capability information to the decoder through either being present in the bitstream or provided through external means.

Table 1 DCI RBSP syntax

| decoding_capability_information_rbsp( ) { | Descriptor |
| --- | --- |
| dci_max_sublayers_minus1 | u(3) |
| dci_reserved_zero_bit | u(1) |
| dci_num_ptls_minus1 | u(4) |
| for( i = 0; i <= dci_num_ptls_minus1; i++ ) |  |
| profile_tier_level( 1, 0 ) |  |
| dci_extension_flag | u(1) |
| if( dci_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dci_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The information is not necessary for the operation of the decoding process. The PTL information contained in the DCI RBSP is redundant as the PTL information is also conveyed to the decoder by either VPS or SPS.

It is proposed to carry DCI in SEI message instead of parameter set, and DCI SEI message may only indicate the preferred profile, tier and level for a decoder that is capable of decoding the entire bitstream.

The proposed DCI SEI message syntax in shown in Table 1.

Table 2 Proposed DCI SEI message syntax

| decoding_capability_info(payLoadSize ) { | Descriptor |
| --- | --- |
| dci_profile_idc | u(7) |
| dci_tier_flag | u(1) |
| dci_level_idc | u(8) |
| dci_num_sub_profiles | u(8) |
| for( i = 0; i < num_sub_profiles; i++ ) |  |
| dci_sub_profile_idc[ i ] | u(32) |
| } |  |

A DCI SEI message informs a decoder with the preferred decoding capability applied to the bitstream.

dci_profile_idc indicates a profile as specified in Annex A that provides the preferred decoded result or the preferred bitstream identification, as determined by the encoder (in a manner not specified in this Specification).

dci_tier_flag specifies the maximu tier context as specified in Annex A that applies to the bitstream.

dci_level_idc indicates the maximum level as specified in Annex A that applies to the bitstream. It may be higher than but cannot be lower than the level signalled in the SPS or VPS.

dci_num_sub_profiles specifies the number of the general_sub_profile_idc[ i ] syntax elements.

dci_sub_profile_idc[ i ] indicates the i-th interoperability metadata registered as specified by Rec. ITU-T T.35, the contents of which are not specified in this Specification.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).