| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0264-v2 |
| --- | --- |

| Title: | AHG9: On sub-bitstream extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He Vadim Seregin Muhammed Coban Marta Karczewicz | Email: | YongHe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to modify sub-bitstream extraction process to keep the VCL NAL units of the output layer.

V2 proposes to modify sub-bitstream extraction process for mixed NAL unit type cases.

# Introduction

Sub-bitstream extraction process is specified in VVC draft 8 as follows:

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, and a target highest TemporalId value tIdTarget. Output of this process is a sub-bitstream outBitstream. The output sub-bitstream OutBitstream is derived as follows: – The bitstream outBitstream is set to be identical to the bitstream inBitstream. – Remove from outBitstream all NAL units with TemporalId greater than tIdTarget. – Remove from outBitstream all NAL units with nal_unit_type not equal to any of VPS_NUT, DCI_NUT, and EOB_NUT and with nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ]. – Remove from outBitstream all NAL units for which all of the following conditions are true: – nal_unit_type is not equal to IDR_W_RADL, IDR_N_LP, or CRA_NUT. – nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive. – TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ j ]. – Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has nesting_ols_flag equal to 1 and there is no value of i in the range of 0 to nesting_num_olss_minus1, inclusive, such that NestingOlsIdx[ i ] is equal to targetOlsIdx. …

For a picture with mixed IRAP and non-IRAP NAL units whose  nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] and TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ j ], the IRAP NAL units are kept and non-IRAP NAL units are removed which ends up with a partial picture.

Figure 1 shows two examples of aforementioned sub-stream extraction issues, all IRAP NAL units will be left in the extracted sub-bitstream.

Figure 1 Sub-bitstream extraction examples

# Proposal

It is proposed to modify sub-bitstream extraction process as follows to to address the above issues.

The output sub-bitstream OutBitstream is derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	Remove from outBitstream all NAL units with TemporalId greater than tIdTarget.

–	Remove from outBitstream all NAL units with nal_unit_type not equal to any of VPS_NUT, DCI_NUT, and EOB_NUT and with nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ].

–	Remove from outBitstream all NAL units for which all of the following conditions are true:

–	mixed_nalu_nal_unit_type is equal to 0 and nal_unit_type is not equal to IDR_W_RADL, IDR_N_LP, or CRA_NUT, or mixed_nalu_nal_unit_type is equal to 1.

–	nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive.

–	TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ j ].

Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).