| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0265 |
| --- | --- |

| Title: | AHG9/AHG12: On subpicture ID mapping signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | 1-858-658-5611 YongHe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The contribution proposes subpicture ID mapping signaling override design to avoid signaling IDs for all subpictures in PPS.

# Introduction

The subpicture ID mapping may be explicitly signaled either in SPS or PPS or inferred from the subpicture index. When explicitly signaled, the ID of all subpictures are signaled either in SPS or PPS but not both. The maximum number of slices is 200 for level 5 and 600 for level 6, which may imply the same number of subpictures available in the bitstream, the overhead for signaling all subpicture in PPS is significant.

Signaling subpicture ID mapping in PPS is mainly for viewport dependent streaming where a subset of subpictures may be updated to match the viewing orientation changes. In general, such change is subtle and the number of subpicture to switch is limited.

Figure 1 Viewport dependent streaming example

Figure 1 shows an example that a user changes the viewing orientation from time t0 to time t1, the number of subpictures to be replaced is limited to maintain the coding efficiency since each replaced subpicture shall be coded as IRAP subpicture.

Figure 2 OMAF viewport switching example

Figure 2 is an example from OMAF D.10.3 where subpictures with intra-coded CUs only may be used as switching points for viewport switching. There is only half of the subpictures are switched in such example.

The mixed NAL unit types feature enables the region-based random access using sub-picture, and it also implies only subset of subpictures may be switched intermittently.

Therefore, we propose subpicture ID mapping override scheme to reduce the explicitly signaled subpictures IDs in PPS.

# Proposal

The proposed SPS syntax is shown in Table 1. The ID mapping of all subpictures is present in SPS when subpic_id_mapping_explicitly_signalled_flag is equal to 1, otherwise, the subpicture IDs are inferred to be equal to the subpicture index.

Table 1 Proposed SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( subpic_id_mapping_explicitly_signalled_flag ) { |  |
| subpic_id_mapping_in_sps_flag | u(1) |
| if( subpic_id_mapping_in_sps_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

The proposed subpicture ID mapping override syntax in PPS is shown in Table 2.

Table 2 Proposed PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_id_mapping_override_in_pps_flag | u(1) |
| if( subpic_id_mapping_override_in_pps_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |

subpic_id_mapping_override_in_pps_flag equal to 1 specifies the presence of subpicture ID mapping syntax elements in PPS. subpic_id_mapping_override_in_pps_flag equal to 0 specifies the absence of subpicture ID mapping syntax elements in PPS.

pps_num_subpics_minus1 shall be equal to sps_num_subpics_minus1 plus 1 specifies the number of subpictures IDs present in the PPS.

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

The variable SubpicIdVal[ i ], for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	if( subpic_id_mapping_explicitly_signalled_flag )
		SubpicIdVal[ i ] = ( subpic_id_mapping_override_in_pps_flag && (i <= pps_num_subpics_minus1 )) ? 						  pps_subpic_id[ i ] : sps_subpic_id[ i ]	(80)
	else
		SubpicIdVal[ i ] = i

Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).