| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0265-v2 |
| --- | --- |

| Title: | AHG9/AHG12: On subpicture ID mapping signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | 1-858-658-5611 YongHe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The contribution proposes subpicture ID mapping signaling override design.

# Introduction

The subpicture ID mapping may be explicitly signaled either in SPS or PPS or inferred from the subpicture index. When explicitly signaled, the ID of all subpictures are signaled either in SPS or PPS but not both. The maximum number of slices is 200 for level 5 and 600 for level 6, which may imply the significant number of subpictures may be available in the bitstream, the overhead for signaling all subpicture in PPS is significant. With 96 subpictures, the subpicture ID mapping overhead is around 690bits per PPS.

Signaling subpicture ID mapping in PPS is mainly for viewport dependent streaming where a subset of subpictures may be updated within a CLVS to match the viewing orientation changes. In general, such change is subtle and the number of subpicture to switch is limited.

Figure 1 Viewport dependent streaming example

Figure 1 shows an example that a user changes the viewing orientation from time t0 to time t1, the number of subpictures to be replaced is limited to maintain the coding efficiency since each replaced subpicture shall be coded as IRAP subpicture.

Figure 2 OMAF viewport switching example

Figure 2 is an example from OMAF D.10.3 where subpictures with intra-coded CUs only may be used as switching points for viewport switching. There is only half of the subpictures switched in such example.

The mixed NAL unit types feature enables the region-based random access using sub-picture, and it also implies only subset of subpictures may be switched intermittently.

In case the ID of every subpicture is changed, the entire picture shall be an IRAP picture based on current constraint.

It is a requirement of bitstream conformance that both of the following constraints apply:

–	For any two differenty values of i and j in the range of 0 to sps_num_subpics_minus1, inclusive, SubpicIdVal[ i ] shall not be equal to SubpicIdVal[ j ].

–	When the current picture is not the first picture of the CLVS, for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, if the value of SubpicIdVal[ i ] is not equal to the value of SubpicIdVal[ i ] of the previous picture in decoding order in the same layer, the nal_unit_type for all coded slice NAL units of the subpicture in the current picture with subpicture index i shall be equal to a particular value in the range of IDR_W_RADL to CRA_NUT, inclusive.

It is asserted that only a subset of all subpictures would be switched within a CLVS. Therefore, we propose subpicture ID mapping override scheme to reduce the explicitly signaled subpictures IDs in PPS.

Table 1 is the current SPS subpicture ID mapping syntax.

Table 1 SPS RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| … |  |
| sps_subpic_id_len_minus1 | ue(v) |
| subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( subpic_id_mapping_explicitly_signalled_flag ) { |  |
| subpic_id_mapping_in_sps_flag | u(1) |
| if( subpic_id_mapping_in_sps_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

subpic_id_mapping_explicitly_signalled_flag equal to 1 specifies that the subpicture ID mapping is explicitly signalled, either in the SPS or in the PPSs referred to by coded pictures of the CLVS. subpic_id_mapping_explicitly_signalled_flag equal to 0 specifies that the subpicture ID mapping is not explicitly signalled for the CLVS. When not present, the value of subpic_id_mapping_explicitly_signalled_flag is inferred to be equal to 0.

subpic_id_mapping_in_sps_flag equal to 1 specifies that the subpicture ID mapping is signalled in the SPS when subpic_id_mapping_explicitly_signalled_flag is equal to 1. subpic_id_mapping_in_sps_flag equal to 0 specifies that subpicture ID mapping is signalled in the PPSs referred to by coded pictures of the CLVS when subpic_id_mapping_explicitly_signalled_flag is equal to 1.

sps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the sps_subpic_id[ i ] syntax element is sps_subpic_id_len_minus1 + 1 bits.

Table 2 is the current PPS subpicture ID mapping syntax.

Table 2 PPS RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_id_mapping_in_pps_flag | u(1) |
| if( subpic_id_mapping_in_pps_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| … |  |
| } |  |

subpic_id_mapping_in_pps_flag equal to 1 specifies that the subpicture ID mapping is signalled in the PPS. subpic_id_mapping_in_pps_flag equal to 0 specifies that the subpicture ID mapping is not signalled in the PPS. If subpic_id_mapping_explicitly_signalled_flag is 0 or subpic_id_mapping_in_sps_flag is equal to 1, the value of subpic_id_mapping_in_pps_flag shall be equal to 0. Otherwise (subpic_id_mapping_explicitly_signalled_flag is equal to 1 and subpic_id_mapping_in_sps_flag is equal to 0), the value of subpic_id_mapping_in_pps_flag shall be equal to 1.

pps_num_subpics_minus1 shall be equal to sps_num_subpics_minus1.

pps_subpic_id_len_minus1 shall be equal to sps_subpic_id_len_minus1.

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

# Proposal

The proposed SPS syntax is shown in Table 3. The ID mapping of all subpictures is present in SPS when subpic_id_mapping_explicitly_signalled_flag is equal to 1, otherwise, the subpicture IDs are inferred to be equal to the subpicture index.

Table 3 Proposed SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| … |  |
| sps_subpic_id_len_minus1 | ue(v) |
| subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( subpic_id_mapping_explicitly_signalled_flag ) { |  |
| subpic_id_mapping_in_sps_flag | u(1) |
| if( subpic_id_mapping_in_sps_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

subpic_id_mapping_in_sps_flag equal to 1 specifies that the subpicture ID mapping is signalled in the SPS when subpic_id_mapping_explicitly_signalled_flag is equal to 1. subpic_id_mapping_in_sps_flag equal to 0 specifies that subpicture ID mapping is not signalled in the SPS signalled in the PPSs referred to by coded pictures of the CLVS when subpic_id_mapping_explicitly_signalled_flag is equal to 1.

sps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the sps_subpic_id[ i ] syntax element is sps_subpic_id_len_minus1 + 1 bits. When not present, the value of sps_subpic_id[ i ] is inferred to be equal to i.

The proposed subpicture ID mapping override syntax in PPS is shown in Table 2.

Table 4 Proposed PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_id_mapping_override_in_pps_flag | u(1) |
| if( subpic_id_mapping_override_in_pps_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |

subpic_id_mapping_override_in_pps flag equal to 1 specifies the presence of subpicture ID mapping syntax elements in PPS. subpic_id_mapping_override_in_pps_flag equal to 0 specifies the absence of subpicture ID mapping syntax elements in PPS.

pps_num_subpics_minus1 shall be equal to sps_num_subpics_minus1 plus 1 specifies the number of subpictures IDs present in the PPS.

pps_subpic_id_len_minus1 shall be equal to sps_subpic_id_len_minus1

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

The current SubPicIdVal[ i ] is derived as follows:

The variable SubpicIdVal[ i ], for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, is derived as follows: for( i = 0; i <= sps_num_subpics_minus1; i++ ) if( subpic_id_mapping_explicitly_signalled_flag ) SubpicIdVal[ i ] = ( subpic_id_mapping_in_pps_flag) ? pps_subpic_id[ i ] : sps_subpic_id[ i ] (80) else SubpicIdVal[ i ] = i

The proposed derivation is as follows:

The variable SubpicIdVal[ i ], for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, is derived as follows: for( i = 0; i <= sps_num_subpics_minus1; i++ ) SubpicIdVal[ i ] = ( subpic_id_mapping_override_in_pps_flag && ( i <= pps_num_subpics_minus1 )) ? pps_subpic_id[ i ] : (( subpic_id_mapping_in_sps_flag ) ? sps_subpic_id[ i ] : i ) (80)

Figure 3 illustrates an example of the proposed subpicture ID derivation.

Figure 3 subpicture ID override example

The updated VTM8.0 software source code is enclosed for the proposed approach.

Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).