| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0266 |
| --- | --- |

| Title: | AHG9: Miscellaneous HLS topics |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He Yao-Jen Chang Muhammed Coban Vadim Seregin Marta Karczewicz | Email: | YongHe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The contribution proposes the following changes on SPS syntax elements.

- Change sps_reserved_zero_4bits to sps_reserved_one_4bits to prevent the SPS start code emulation without bit cost.

- Consolidate two syntax elements, sps_poc_msb_flag and poc_msb_len_minus1 into a single syntax element poc_msb_len.

- Condition the presence of subpic_info_present_flag by res_change_in_clvs_allowed_flag

- Code PPS ID, pps_pic_parameter_set_id, with u(6)

- gdr_enabled_flag value constrained by no_gdr_constraint_flag

- Code the locations of virtual boundaries, i.e., ps_virtual_boundaries_pos_x[ i ], sps_virtual_boundaries_pos_y[ i ], ph_virtual_boundaries_pos_x[ i ], and ph_virtual_boundaries_pos_y[ I ], with ue(v).

# Proposal

## Start code emulation

The first few syntax elements of SPS are shown in Table 1.

Table 1 SPS RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| chroma_format_idc | u(2) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| res_change_in_clvs_allowed_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| … | u(1) |
| } |  |

The value of first three bytes in SPS may be equal to 0x000000, 0x000001 or 0x000002 under the circumstance as shown in Figure 1.

Figure 1 Value of SPS syntax elements example

Even though emulation_prevention_three_byte is designed to deal with such situation, a simple solution without overhead cost may be desired to address such obvious emulation.

It is proposed to change sps_reserved_zero_4bits to sps_reserved_one_4bits to avoid such emulation in SPS as shown in Table 2.

Table 2 Proposed SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_one_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| … |  |
| } |  |

sps_reserved_one_4bits shall be equal to 0xF in bitstreams conforming to this version of this Specification. Other values for sps_reserved_one_4bits are reserved for future use by ITU-T | ISO/IEC.

## POC MSB signaling

Two syntax elements, sps_poc_msb_flag and poc_msb_len_minus1, are signaled in SPS as shown below.

Table 3 SPS RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_poc_msb_flag | u(1) |
| if( sps_poc_msb_flag ) |  |
| poc_msb_len_minus1 | ue(v) |
| … |  |
| } |  |

sps_poc_msb_flag equal to 1 specifies that the ph_poc_msb_present_flag syntax element is present in PHs referring to the SPS. sps_poc_msb_flag equal to 0 specifies that the ph_poc_msb_present_flag syntax element is not present in PHs referring to the SPS.

poc_msb_len_minus1 plus 1 specifies the length, in bits, of the poc_msb_val syntax elements, when present in the PHs referring to the SPS. The value of poc_msb_len_minus1 shall be in the range of 0 to 32 − log2_max_pic_order_cnt_lsb_minus4 − 5, inclusive.

It is proposed to consolidate these two syntax elements into one syntax element for the simplicifcation. The proposed syntax change in SPS and PH are shown below.

Table 4 Proposed SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_poc_msb_flag | u(1) |
| if( sps_poc_msb_flag ) |  |
| poc_msb_len_minus1 | ue(v) |
| poc_msb_len | ue(v) |
| … |  |
| } |  |

poc_msb_len specifies the length, in bits, of the poc_msb_val syntax elements, when present in the PHs referring to the SPS. The value of poc_msb_len shall be in the range of 0 to 32 − log2_max_pic_order_cnt_lsb_minus4 − 4, inclusive. When the value of poc_msb_len is equal to 0, the ph_poc_msb_present_flag syntax element is not present in PHs referring to the SPS.

Table 5 Proposed PH syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_poc_msb_flag poc_msb_len ) { |  |
| ph_poc_msb_present_flag | u(1) |
| if( ph_poc_msb_present_flag ) |  |
| poc_msb_val | u(v) |
| } |  |
| … |  |
| } |  |

poc_msb_val specifies the POC MSB value of the current picture. The length of the syntax element poc_msb_val is poc_msb_len_minus1 + 1 bits.

## subpic_info_present_flag

Current VVC always signals subpicture present flag in SPS and subpicture layout is static over the CLVS. When res_change_in_clvs_allowed_flag is equal to 1, the value of subpic_info_present_flag shall be equal to 0. Instead of constraining it in the semantic, a condition in the syntax structure may be more straightforward and avoid signaling subpic_info_present_flag at all.

The proposed condition is shown in Table 6.

Table 6 Proposed SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| res_change_in_clvs_allowed_flag | u(1) |
| … |  |
| if( !res_change_in_clvs_allowed_flag ) { |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| … |  |
| } |  |
| } |  |
| … |  |
| } |  |

subpic_info_present_flag equal to 1 specifies that subpicture information is present for the CLVS and there may be one or more than one subpicture in each picture of the CLVS. subpic_info_present_flag equal to 0 specifies that subpicture information is not present for the CLVS and there is only one subpicture in each picture of the CLVS. When not present, the value of subpic_info_present_flag is inferred to be equal to 0.

When res_change_in_clvs_allowed_flag is equal to 1, the value of subpic_info_present_flag shall be equal to 0.

## PPS ID

The value of syntax element, pps_pic_parameter_set_id, is in the range of 0 to 63, inclusive, and is currently coded with ue(v). It is proposed to code this syntax element with u(6) to be consistent with other similar syntax elements, e.g. the IDs for the VPS, SPS and APS.

## gdr_enabled_flag

gdr_pic_flag equal to 1 specifies the picture associated with the PH is a GDR picture. gdr_pic_flag equal to 0 specifies that the picture associated with the PH is not a GDR picture. When not present, the value of gdr_pic_flag is inferred to be equal to 0. When gdr_enabled_flag is equal to 0, the value of gdr_pic_flag shall be equal to 0.

When no_gdr_constraint_flag is equal to 1, the value of gdr_pic_flag shall be equal to 0.

## The locations of virtual boundaries

The locations of virtual boundaries, i.e., sps_virtual_boundaries_pos_x[ i ], sps_virtual_boundaries_pos_y[ i ], ph_virtual_boundaries_pos_x[ i ], and ph_virtual_boundaries_pos_y[ i ], uses u(13) for coding, which disallows the values larger than 8191. The related semantics in the current VVC specification is shown as follows:

sps_virtual_boundaries_pos_x[ i ] specifies the location of the i-th vertical virtual boundary in units of luma samples divided by 8. The value of sps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_max_in_luma_samples ÷ 8 ) − 1, inclusive.

sps_virtual_boundaries_pos_y[ i ] specifies the location of the i-th horizontal virtual boundary in units of luma samples divided by 8. The value of sps_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_max_in_luma_samples ÷ 8 ) − 1, inclusive.

ph_virtual_boundaries_pos_x[ i ] specifies the location of the i-th vertical virtual boundary in units of luma samples divided by 8. The value of ph_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 8 ) − 1, inclusive.

ph_virtual_boundaries_pos_y[ i ] specifies the location of the i-th horizontal virtual boundary in units of luma samples divided by 8. The value of ph_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_in_luma_samples ÷ 8 ) − 1, inclusive.

The maximum values of these syntax elements are constrained by the picture width and picture height, whose values are unlimited in the current specification for certain applications. If the picture width or height is larger than 8192*8, the maximum values of these syntax elements shall be larger than 8191. In such case, there is a bug to use u(13) for coding the values larger than 8191. Therefore, it is proposed to replace u(13) with ue(v). The proposed coding is more flexible for the future applications and consistent with the picture resolution syntax design in SPS and PPS.

Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).